# TMB 2005 + 2013 Design

# TAMU & UCLA High Energy Physics

Version 5.03

April 24, 2014

Table of Contents

Figures	5

Tables	5

TMB Overview	6

CLCT Processing Algorithm	7

DDR Signal Synchronization	9

CFEB DDR Receiver Sync Stages	9

ALCT DDR Receiver Sync Stages	10

ALCT DDR Transmitter Sync Stages	11

TTC Sequences	15

Trigger State	15

Bunch Crossing Counter	15

Resync	15

ALCT+CLCT Matching Algorithm	16

Matching Logic	16

LCT Duplication	16

LCT Quality	16

module lct_quality (ACC,A,C,A4,C4,P,CPAT,Q);	17

// Ports	17

input  ACC;     // ALCT accelerator muon bit	17

input  A;       // bit: ALCT was found	17

input  C;       // bit: CLCT was found	17

input  A4;      // bit (N_A>=4), where N_A=number of ALCT layers	17

input  C4;      // bit (N_C>=4), where N_C=number of CLCT layers	17

input  [3:0] P; // 4-bit CLCT pattern number that is presently 1 for n-layer triggers, 2-10 for current patterns, 11-15 "for future expansion".	17

input  CPAT;    // bit for cathode .pattern trigger., i.e. (P>=2 && P<=10) at present	17

output [3:0] Q; // 4-bit TMB quality output	17

// Quality-by-quality definition	17

reg [3:0] Q;	17

always @* begin	17

if      ( !ACC && A4 && C4 && P==10 )          Q=15; // HQ muon, straight	17

else if ( !ACC && A4 && C4 && (P==9 || P==8) ) Q=14; // HQ muon, slight bend	17

else if ( !ACC && A4 && C4 && (P==7 || P==6) ) Q=13; // HQ muon, more	17

else if ( !ACC && A4 && C4 && (P==5 || P==4) ) Q=12; // HQ muon, more	17

else if ( !ACC && A4 && C4 && (P==3 || P==2) ) Q=11; // HQ muon, more	17

//                                             Q=10; // reserved for HQ muons with future patterns	17

//                                             Q=9;  // reserved for HQ muons with future patterns	17

else if (  ACC       &&  A4       &&  C4 && CPAT ) Q=8; // HQ muon, but accel ALCT	17

else if (          A && !A4       &&  C4 && CPAT ) Q=7; // HQ cathode, but marginal anode	17

else if (            &&  A4 &&  C && !C4 && CPAT ) Q=6; // HQ anode, but marginal cathode	17

else if (          A && !A4 &&  C && !C4 && CPAT ) Q=5; // marginal anode and cathode	17

//                                                 Q=4; // reserved for LQ muons with 2D information in the future	17

else if (          A        &&  C        && P==1 ) Q=3; // any match but layer CLCT	17

else if (         !A        &&  C                ) Q=2; // some CLCT, no ALCT (unmatched)	17

else if (          A        && !C                ) Q=1; // some ALCT, no CLCT (unmatched)	17

else                                               Q=0; // should never be assigned	17

end	17

MPC Format	18

VME Registers	19

Addressing Modes	19

Base Address	19

Boot Register	19

Register Addresses	20

Register Definitions	26

TTC Commands:	89

TMB Board Status Operations	90

ID Registers	90

Digital Serial Numbers	90

Power Supply ADC	90

Clock Delays	90

JTAG Chains	91

RAT Module Status Register USER1	91

RAT Module Control Register USER2	92

User PROM Programming	93

Register Initialization	93

VME PROM-0	94

JTAG PROM-1	95

DMB Readout	98

Full-Readout and Local-Readout Format (Long Header):	98

Long Header-only Format:	99

Short Header-only Format:	99

Miniscope:	100

Blocked CFEB DiStrips List Format:	102

Header Word Descriptions:	103

Sample TMB Raw Hits Dump	117

Configuration	118

Shunt Settings	118

Switch Settings	119

Fuses	120

Signal Summary	121

CCB	121

ALCT	122

DMB	123

CFEB	124

MPC	125

RPC	126

VME	126

JTAG	126

LEDs & Testpoints	127

TMB Total I/O Count	128

Connectors	129

TMB Connector Summary	129

J0-J4 CFEB0-CFEB4 Connectors	130

J5 ALCT Cable1 Connector (Receiver)	131

J6 ALCT Cable2 Connector (Transmitter)	132

J1-J6 SCSI-II 50-Pin Connector Pin Convention	133

J7 Xilinx LVDS Xilinx X-Blaster Connector	134

P1 Backplane VME64x J1/P1 Connector	135

P2A Backplane CCB+DMB Connector	136

P2A Backplane CCB+DMB Connector Continued	137

P2B Backplane DMB Connector	138

P3A Backplane MPC Connector	139

P3B Backplane RPC+ALCT Connector	140

Backplane Pin Diagram	141

Front Panel Connector Locations	142

CCB Front Panel	143

Documentation Revision History	145

Firmware Change Log	148

## Figures

Figure 1 TMB Overview	8

Figure 2 CFEB Muonic Receiver	14

Figure 3 ALCT Muonic Receiver	14

Figure 4 ALCT Muonic Transmitter	15

Figure 5: 50-Pin PCB Connector (Female)	135

Figure 6: 50 Pin Cable Connector (Male)	135

Figure 7: 50 Pin PCB Connector Pin Convention	135

## Tables

Table 1: Shunts	120

Table 2: Switch Settings	121

Table 3: Fuses for TMB2005	122

Table 3A: Fuses for TMB2013	122

Table 4: CCB Signal Summary	123

Table 5: ALCT Signal Summary	124

Table 6: DMB Signal Summary	125

Table 7: CFEB Signal Summary	126

Table 8: MPC Signal Summary	127

Table 9: RPC Signal Summary	128

Table 10: VME Signal Summary	128

Table 11: JTAG Signal Summary	128

Table 12: TMB Front Panel LEDs	129

Table 13: TMB Total I/O Count	130

Table 14: TMB2005 Connector Summary	131

Table 15: J0-J4 CFEB0/4-to-TMB Connectors	132

Table 16: J5 ALCT Cable1 Connector [J10  on ALCT board]	133

Table 17: J6 ALCT Cable2 Connector [J11 on ALCT board]	134

Table 18: J8 Xilinx LVDS X-Blaster Connector	136

Table 19: P1 VME64x Connector	137

Table 20: P2A Backplane CCB+DMB Connector	138

Table 21: P2B Backplane DMB Connector	140

Table 22: P3A Backplane MPC Connector	141

Table 23: P3B Backplane RPC+ALCT Connector	142

Table 24: J5 ALCT Cable1 Connector [J10  on ALCT board]	164

# TMB Overview

# Figure 1 TMB Overview

24 x80MHz

A24+GA5

3

n

8

A2

1

CLCT + TMB

Logic

PROM

JTAG

MPC

DMB

CCB

VME

RPC

ALCT

CFEB4

CFEB3

CFEB2

CFEB1

CFEB0

40MHz clock

D16

24 x80MHz

40MHz clock

46 x40MHz

24 x80MHz

46 x40MHz

40MHz clock

40MHz clock

44 x40MHz

24 x80MHz

40MHz clock

4 x40MHz

40MHz clock

32 x80MHz

24 x80MHz

1 x80MHz

40MHz clock

28x80MHz+1x10MHz

40MHz clock

15x80MHz+5x10MHz

40 x80MHz

7 x3.2gbps
(expandable to 12 links)

DCFEB [6:0]

Fiber Links

(TMB2013 only)

# CLCT Processing Algorithm

bx -½	Latch 1st-in-time CFEB transmission

bx 0 	Latch 2nd in-time CFEB transmission

Demultiplex 1st-in-time and 2nd-in-time CFEB Triads

Map cable-signal ordering into 5 CFEBs x 8 DiStrip Triads x 6 Layers (240 signals)

Store CFEB Triads in Raw-hits RAM

Multiplex CFEB Triads with internal pattern-injector RAM

AND Triads with Hot Channel Mask to disable errant DiStrips

Decode Triad start bits (240 processed in parallel)

Triad decoder state machines run continuously to preclude missing any start bits

Each ½-strip one-shot can fire again on the same clock cycle that the previous ½-strip pulse ends

If a 2nd triad arrives for the same DiStrip while busy, the triad is decoded but the one-shot does not fire

In that case, the triad-skipped counter is incremented

bx 1	Decode Triad strip bits

bx 2	Decode Triad ½-strip bits

bx 3	Fire ½-strip one-shots for 6bx (triad_persist is programmable, 6bx is the default)

OR 160 ½-strips on each layer for layer-trigger mode

Stagger correction (if stagger_hs_csc =1) shifts alternate layers by -1hs

ly0	-0hs		i.e.   hs5 hs5

ly1	-1hs		i.e.   hs6 hs5

ly2	-0hs		i.e.   hs5  hs5 	key layer ly2

ly3	-1hs		i.e.   hs6  hs5

ly4	-0hs		i.e.   hs5  hs5

ly5	-1hs		i.e.   hs6  hs5

Pattern Finding:

For each of 160 key ½-strips consider the 42 neighboring ½-strips (i.e. on key 5 use the following ½-strips)

hs		0123456789A

ly0[10:0]	xxxxxkxxxxx    5+1+5 =11

ly1[ 7:3]	   xxkxx       2+1+2 = 5

ly2[ 5:5]	     k         0+1+0 = 1

ly3[ 7:3]	   xxkxx       2+1+2 = 5

ly4[ 9:1]	 xxxxkxxxx     4+1+4 = 9

ly5[10:0]	xxxxxkxxxxx    5+1+5 =11

For each of 160 key ½-strips, count layers with hits matching the 9 pattern templates

Pattern ID=1 is a layer-OR trigger, Pattern ID=0 is no-pattern-found

Hit pattern LUTs for 1 layer: - = don’t care, xx= one hit or the other or both

Pattern       id=2        id=3        id=4        id=5        id=6        id=7        id=8        id=9        idA

Bend dir      bd=0        bd=1        bd=0        bd=1        bd=0        bd=1        bd=0        bd=1        bd=0

|           |           |           |           |           |           |           |           |

ly0      --------xxx xxx-------- -------xxx- -xxx------- ------xxx-- --xxx------ -----xxx--- ---xxx----- ----xxx----

ly1      ------xx--- ---xx------ ------xx--- ---xx------ -----xx---- ----xx----- -----xx---- ----xx----- -----x-----

ly2 key  -----x----- -----x----- -----x----- -----x----- -----x----- -----x----- -----x----- -----x----- -----x-----

ly3      ---xxx----- -----xxx--- ---xx------ ------xx--- ----xx----- -----xx---- ----xx----- -----xx---- -----x-----

ly4      -xxx------- -------xxx- -xxx------- -------xxx- ---xx------ ------xx--- ---xxx----- -----xxx--- ----xxx----

ly5      xxx-------- --------xxx -xxx------- -------xxx- --xxx------ ------xxx-- ---xxx----- -----xxx--- ----xxx----

|           |           |           |           |           |           |           |           |

// Extent   0123456789A 0123456789A 0123456789A 0123456789A 0123456789A 0123456789A 0123456789A 0123456789A 0123456789A

// Avg.bend - 8.0 hs    + 8.0 hs    -6.0 hs     +6.0 hs     -4.0 hs     +4.0 hs     -2.0 hs     +2.0 hs      0.0 hs

// Min.bend -10.0 hs    + 6.0 hs    -8.0 hs     +4.0 hs     -6.0 hs     +2.0 hs     -4.0 hs      0.0 hs     -1.0 hs

// Max.bend - 6.0 hs    +10.0 hs    -4.0 hs     +8.0 hs     -2.0 hs     +6.0 hs      0.0 hs     +4.0 hs     +1.0 hs

bx 4	Result for each of 160 keys is a list of 9 pattern-ID numbers (pid) [2 to A] and corresponding number of layers [0 to 6] with matching hits (nhits)

Find the best 1-of-9 pattern ID numbers for each key by comparing nhits

Ignore bend direction: left and right bends have equal priority (bit 0 of pid implies bend direction)

If two pattern IDs have the same nhits, take the higher pattern ID

A key with no matching hits, would always return pid=A and nhits=0

bx 5	Pre-trigger if any 1-of-160 keys have nhits ≥ hit_thresh_pretrig and pid ≥ pid_thresh_pretrig

Construct 5-bit active-cfeb list for DMB:

cfebs with a key that has nhits ≥ hit_thresh_pretrig and pid ≥ pid_thresh_pretrig

cfebs with a key that has nhits ≥ dmb_thresh_pretrig

cfebs adjacent to a cfeb that has nhits ≥ hit_thresh_pretrig and pid ≥ pid_thresh_pretrrig within adjfeb_dist

bx 6	Finding 1st CLCT:

Construct 7-bit pattern quality for sorting:  pat[7:0]

pat[7:5]=nhits[2:0]

pat[4:0]=pid[3:0]

Ignore the bend direction bit (pid[0]), left and right bends have equal priority

Store pat[7:0] for 160 keys for use later to find 2nd CLCT

Start finding best 1-of-160 keys by sorting on the 6-bit number pat[7:1]

If two keys have the same pat[7:1] take the lower key

bx 7	Find 1st CLCT:

Finish finding best 1-of-160 keys by sorting on the 6-bit number pat[7:1]

Store 1st CLCT info: key, pattern ID, and number of hits

For empty events, key=0, pid=A and nhits=0. If clct_blanking=1, then key=pid=hits=0

bx 8	Finding 2nd CLCT:

Construct list of busy keys

Mark keys near 1st CLCT as busy from 1st key-nspan to 1st key+pspan

If clct_sep_src=1, pspan and nspan are set equal to clct_sep_vme, typically 10hs

If clct_sep_src=0, pspan and nspan are read from RAM and depend on the pattern ID number

This allows two non-bending tracks |  | to be closer than bending tracks /   \

Start finding best 1-of-160 keys by sorting on the 6-bit number pat[7:1]

Skip busy keys

If two keys have the same pat[7:1] take the lower key

bx 9	Find 2nd CLCT:

Finish finding best 1-of-160 keys by sorting on the 6-bit number pat[7:1]

Store 2nd  CLCT info: key, pattern ID, and number of hits

For empty events, key=11, pid=A and nhits=0. If clct_blanking=1, then key=pid=hits=0

bx 10	Drift Delay 1bx (waits for CSC drifting)

bx 11	Drift Delay 1bx

If clct0 nhits < hit_thresh_postdrift OR pid < pid_thresh_post_drift, discard event

bx 12	Match to ALCT window 0

If alct matches, jump to bx15 logic, latency is shortened 2bx

bx 13	Match to ALCT window 1

If alct matches, jump to bx15 logic, latency is shortened 1bx

bx 14	Match to ALCT window 2

If ALCT does not arrive, and clct_only mode is enabled, accept CLCT at window 2

If ALCT does not arrive, and not in clct_only mode, discard event

bx 15	 Construct two LCTs from CLCT and ALCT data

If event has 2 CLCTs and 1 ALCT, copy 1st ALCT into 2nd ALCT position

If event has 1 CLCT   and 2 ALCTs, copy 1st CLCT into 2nd CLCT position

Calculate LCT quality

Multiplex mpc injector ram data

bx 16	 Transmit 1st-in-time LCT frame to MPC

bx 16½ Transmit 2nd-in-time LCT frame to MPC

# DDR Signal Synchronization

### CFEB DDR Receiver Sync Stages

CFEB receiver logic is designed to synchronize incoming comparator data to TMBs main clock, while minimizing CLCT trigger-path latency.  It has a programmable 0-16bx delay stage to compensate for differing CFEB cable lengths.

The logic is shown schematically (Figure 2) but the actual TMB logic is written in behavioral Verilog.

U1A	Latches incoming CFEB data din on the falling edge of clock_iob.

The user has already tuned the associated cfeb[n]_rxd digital phase shifter so that clock_iob always latches stable data.

U1B	Latches incoming din on the rising edge of clock_iob.

U1A and U1B comprise a single Double Data Rate (DDR) I/O-Block flip-flop, and are only shown here separately for clarity.

U2	Latches data transferred from U1A on the rising edge of clock_iob.

U2 now holds din_1st-in-time data, aligned with the rising edge of clock_iob, while

U1B holds din_2nd-in-time data, also aligned with the rising edge of clock_iob.

In non-muonic firmware, this would be the end stage, because clock_iob would be the same as TMBs main clock.

In muonic firmware versions, din_1st and din_2nd still need to be synchronized to TMBs main clock.  This is done by latching data on both the posedge and negedge of the main clock, then selecting one latch or the other with the posneg bit.

If the posedge flip-flops  U11,U12 are out-of-time (creating a dead-spot in the din receive window) then the negedge flip-flops U3,U4 must be in-time. One set of latches, either U11,U12 or U3,U4 will always be in-time for a given phase of clock_iob.

U3,U4	Latch din_1st and din_2nd on the falling edge TMBs main clock_1x.

U5,U6	Select either posedge data from U2,U1B or negedge data from U3,U4.

U7,U8	Delay data by 1-to-16 bx, according to delay[3:0] from a VME register.

Setting address=0 for U7,U8 gives a 1bx delay, so delay-1 is used to form the address.

U14	Subtracts 1 from delay[3:0] to form the shift register address for U7,U8.

U9,U10	Bypass delays U7,U8 if the delay is 0, thus giving a 0-16bx delay span.

U11,U12	Latch data on the rising edge of TMBs main clock_1x, and are the final synchronization stage.

### ALCT DDR Receiver Sync Stages

ALCT receiver logic is designed to synchronize incoming data to TMBs main clock. It uses a small number of flip-flops, and is able to fit in a single FPGA column.  It uses a different technique than the CFEB sync section, and does not have a delay stage.

The logic is shown schematically (Figure 3), but the actual TMB logic is written in behavioral Verilog.

U1A	Latches incoming ALCT data din on the falling edge of clock_iob.

The user has already tuned the associated alct_rxd digital phase shifter so that clock_iob always latches stable data.

U1B	Latches incoming din on the rising edge of clock_iob.

U1A and U1B comprise a single Double Data Rate (DDR) I/O-Block flip-flop, and are only shown here separately for clarity.

U2	Latches ALCT data transferred from U1A on the rising edge of clock_iob.

U2 now holds din_1st-in-time data, aligned with the rising edge of clock_iob, while

U1B holds din_2nd-in-time data, also aligned with the rising edge of clock_iob.

In non-muonic firmware, this would be the end stage, because clock_iob would be the same as TMBs main clock.

U3,U4	Latch din_1st and din_2nd on either the rising or falling edge of TMBs main clock_1x.  They are clocked at 2x the main clock frequency, but are enabled for only one edge direction, according to the value of the posneg bit. The edges of clock_2x are closely aligned to the edges of clock_1x.

U7	If posneg =1, U7 inverts a logic accessible copy of clock_1x, called clock_lac.

If posneg =0, U7 passes clock_lac un-inverted.

Because clock_lac is delayed by about 2ns after clock_1x, posneg =1 enables U3,U4 to latch data on the edge of clock_2x that corresponds to the rising edge of clock_1x, and posneg=0 enables latching on the falling edge.

U5,U6	Latch data on the rising edge of TMBs main clock_1x, and are the final synchronization stage.

### ALCT DDR Transmitter Sync Stages

The ALCT transmitter logic is designed to synchronize TMBs outgoing data to the ALCT board’s clock. It has a programmable 0-16bx delay stage, and a data-path multiplexer for sending test patterns to ALCT.

The logic is shown schematically (Figure 4), but the actual TMB logic is written in behavioral Verilog.

U1,U2	Select the transmitter data source, either ALCT data or the test pattern generator, according to the value of sync_mode.

U3,U4	Delay data by 1-to-16 bx, according to the VME-set delay-1  (the subtractor is not shown).

U3,U4	Delay data by 1-to-16 bx, according to delay[3:0] from a VME register.

Setting address=0 for U3,U4 gives a 1bx delay, so delay-1 is used to form the address.

U15	Subtracts 1 from delay[3:0] to form the shift register address for U3,U4.

U5,U6	Bypass delays U3,U4 if the delay is 0, thus giving a 0-16bx delay span.

U7,U8	Latch data on the rising edge of TMBs main clock_1x, and are the final main-clock stage.

In non-muonic firmware, this would be the end stage, because clock_iob would be the same as TMBs main clock.

U9,U10	Latch data on either the rising or falling edge of TMBs main clock_1x.  They are clocked at 2x the main clock frequency, but are enabled for only one edge direction, according to the value of the posneg bit. The edges of clock_2x are closely aligned to the edges of clock_1x.

U14	If posneg =1, U14 inverts a logic accessible copy of clock_1x, called clock_lac.

If posneg =0, U14 passes clock_lac un-inverted.

Because clock_lac is delayed by about 2ns after clock_1x, posneg =1 enables U9,U10 to latch data on the edge of clock_2x that corresponds to the rising edge of clock_1x, and posneg=0 enables latching on the falling edge.

U12	Latches 1st-in-time data on the rising edge of clock_iob.  It is a Double Date Rate I/O Block flip-flop that transmits 1st-in-time data on the rising edge of clock_iob, and 2nd-in-time on the falling edge.

U11	Holds 2nd-in-time data for ½ cycle while waiting for U12 to transmit 1st-in-time data.

Figure 2 CFEB Muonic Receiver

Figure 3 ALCT Muonic Receiver

Figure 4 ALCT Muonic Transmitter

# TTC Sequences

### Trigger State

Start/Stop Triggering Sequence:

----------------------------------------------

On hard_reset go to the StopTrigger state:	TMBs CLCT processing machine goes to “idle”.

On ttc_start_trigger, wait for the next BX0 (if ttc starts are allowed)

On the next ttc_bx0, resume triggering.

On ttc_stop_trigger (if ttc stops are allowed) or ttc_resync go the StopTrigger state.

### Bunch Crossing Counter

Bunch Counter Reset

--------------------------------

On ttc_resync or ttc_bxreset, preset the BXN, and hold the count.

On the next BX0 resume counting bunch crossings.

On subsequent BX0s, check that BXN is again at the preset value,

if not, set sync_error.

TMB further checks that when BXN is at the preset value, that a BX0

arrived, if not, set sync_error.

### Resync

L1A Counter Reset

----------------------------

On ttc_resync or ccb_evcntreset, clear L1A event counters.

Buffer Reset

--------------------

ttc_resync clears TMBs buffer-pointers, aborts any readout in progress,

and returns various state machines to their idle states.

(a similar condition to after power-up or hard-reset).

# ALCT+CLCT Matching Algorithm

### Matching Logic

### LCT Duplication

// Fill in missing ALCT if CLCT has 2 muons, missing CLCT if ALCT has 2 muons

wire	no_alct  = !alct0_vpf;

wire	no_clct  = !clct0_vpf;

wire	one_alct = alct0_vpf && !alct1_vpf;

wire	one_clct = clct0_vpf && !clct1_vpf;

wire	two_alct = alct0_vpf && alct1_vpf;

wire	two_clct = clct0_vpf && clct1_vpf;

wire	dupe_alct = one_alct && two_clct;

wire	dupe_clct = one_clct && two_alct;

wire	[MXALCT-1:0]	alct_dummy = clct0_real[18:17] << 11;			// Inserts clct bxn into

wire	[MXCLCT-1:0]	clct_dummy = 0;					// frame for clct_only events

wire	[MXALCT-1:0]	alct0 = (no_alct  ) ? alct_dummy : alct0_real;	// Substitute dummy alct

wire 	[MXALCT-1:0]	alct1 = (dupe_alct) ? alct0_real : alct1_real;

wire	[MXCLCT-1:0]	clct0 = (no_clct  ) ? clct0_real : clct0_real;	// Do not

wire	[MXCLCT-1:0]	clct1 = (dupe_clct) ? clct0_real : clct1_real;

wire first_vpf		= alct0_vpf || clct0_vpf;				// First muon exists

wire second_vpf	= alct1_vpf || clct1_vpf;				// Second muon exists

### LCT Quality

### module lct_quality (ACC,A,C,A4,C4,P,CPAT,Q);

### // Ports

### input  ACC;     // ALCT accelerator muon bit

### input  A;       // bit: ALCT was found

### input  C;       // bit: CLCT was found

### input  A4;      // bit (N_A>=4), where N_A=number of ALCT layers

### input  C4;      // bit (N_C>=4), where N_C=number of CLCT layers

### input  [3:0] P; // 4-bit CLCT pattern number that is presently 1 for n-layer triggers, 2-10 for current patterns, 11-15 "for future expansion".

### input  CPAT;    // bit for cathode .pattern trigger., i.e. (P>=2 && P<=10) at present

### output [3:0] Q; // 4-bit TMB quality output

### // Quality-by-quality definition

### reg [3:0] Q;

### always @* begin

### if      ( !ACC && A4 && C4 && P==10 )          Q=15; // HQ muon, straight

### else if ( !ACC && A4 && C4 && (P==9 || P==8) ) Q=14; // HQ muon, slight bend

### else if ( !ACC && A4 && C4 && (P==7 || P==6) ) Q=13; // HQ muon, more

### else if ( !ACC && A4 && C4 && (P==5 || P==4) ) Q=12; // HQ muon, more

### else if ( !ACC && A4 && C4 && (P==3 || P==2) ) Q=11; // HQ muon, more

### //                                             Q=10; // reserved for HQ muons with future patterns

### //                                             Q=9;  // reserved for HQ muons with future patterns

### else if (  ACC       &&  A4       &&  C4 && CPAT ) Q=8; // HQ muon, but accel ALCT

### else if (          A && !A4       &&  C4 && CPAT ) Q=7; // HQ cathode, but marginal anode

### else if (            &&  A4 &&  C && !C4 && CPAT ) Q=6; // HQ anode, but marginal cathode

### else if (          A && !A4 &&  C && !C4 && CPAT ) Q=5; // marginal anode and cathode

### //                                                 Q=4; // reserved for LQ muons with 2D information in the future

### else if (          A        &&  C        && P==1 ) Q=3; // any match but layer CLCT

### else if (         !A        &&  C                ) Q=2; // some CLCT, no ALCT (unmatched)

### else if (          A        && !C                ) Q=1; // some ALCT, no CLCT (unmatched)

### else                                               Q=0; // should never be assigned

### end

### MPC Format

assign	mpc0_frame0[6:0]	=	alct0_key[6:0];

assign	mpc0_frame0[10:7]	=	clct0_pat[3:0];

assign	mpc0_frame0[14:11]	=	lct0_quality[3:0] * lct0_vpf;

assign	mpc0_frame0[15]	=	lct0_vpf;

assign	mpc0_frame1[7:0]	=	{clct0_cfeb[2:0],clct0_key[4:0]};

assign	mpc0_frame1[8]		=	clct0_bend;

assign	mpc0_frame1[9]		=	clct_sync_err & tmb_sync_err_en[0];

assign	mpc0_frame1[10]	=	alct0_bxn[0];

assign	mpc0_frame1[11]	=	clct_bx0 * lct0_vpf;

assign	mpc0_frame1[15:12]	=	csc_id[3:0]* lct0_vpf;

assign	mpc1_frame0[6:0]	=	alct1_key[6:0];

assign	mpc1_frame0[10:7]	=	clct1_pat[3:0];

assign	mpc1_frame0[14:11]	=	lct1_quality[3:0] * lct1_vpf;

assign	mpc1_frame0[15]	=	lct1_vpf;

assign	mpc1_frame1[7:0]	=	{clct1_cfeb[2:0],clct1_key[4:0]};

assign	mpc1_frame1[8]		=	clct1_bend;

assign	mpc1_frame1[9]		=	clct_sync_err & tmb_sync_err_en[1] & lct1_vpf;

assign	mpc1_frame1[10]	=	alct1_bxn[0];

assign	mpc1_frame1[11]	=	clct_bx0 * lct1_vpf;

assign	mpc1_frame1[15:12]	=	csc_id[3:0] * lct1_vpf;

Transmission to MPC is		1st Frame[31:0] = {mpc1_frame0[15:0],mpc0_frame0[15:0]}

2nd Frame[31:0] = {mpc1_frame1[15:0],mpc0_frame1[15:0]}

| Signal | 1st in Time | 2nd in Time | P3Apin | Test Point |
| --- | --- | --- | --- | --- |
| mpc_tx[00] | alct_first_key[0] | clct_first_key[0] | A1 | TP331-1 |
| mpc_tx[01] | alct_first_key[1] | clct_first_key[1] | B1 | TP331-2 |
| mpc_tx[02] | alct_first_key[2] | clct_first_key[2] | D1 | TP331-3 |
| mpc_tx[03] | alct_first_key[3] | clct_first_key[3] | E1 | TP331-4 |
| mpc_tx[04] | alct_first_key[4] | clct_first_key[4] | A2 | TP331-5 |
| mpc_tx[05] | alct_first_key[5] | clct_first_key[5] | B2 | TP331-6 |
| mpc_tx[06] | alct_first_key[6] | clct_first_key[6] | D2 | TP331-7 |
| mpc_tx[07] | clct_first_pat[0] | clct_first_key[7] | E2 | TP331-8 |
| mpc_tx[08] | clct_first_pat[1] | clct_first_bend | A3 | TP332-1 |
| mpc_tx[09] | clct_first_pat[2] | clct_first_sync_err | B3 | TP332-2 |
| mpc_tx[10] | clct_first_pat[3] | alct_first_bxn[0] | D3 | TP332-3 |
| mpc_tx[11] | lct_first_quality[0] | clct_first_bx0_local | E3 | TP332-4 |
| mpc_tx[12] | lct_first_quality[1] | csc_id[0] | A4 | TP332-5 |
| mpc_tx[13] | lct_first_quality[2] | csc_id[1] | B4 | TP332-6 |
| mpc_tx[14] | lct_first_quality[3] | csc_id[2] | D4 | TP332-7 |
| mpc_tx[15] | first_vpf | csc_id[3] | E4 | TP332-8 |
| mpc_tx[16] | alct_second_key[0] | clct_second_key[0] | A5 | TP341-1 |
| mpc_tx[17] | alct_second_key[1] | clct_second_key[1] | B5 | TP341-2 |
| mpc_tx[18] | alct_second_key[2] | clct_second_key[2] | D5 | TP341-3 |
| mpc_tx[19] | alct_second_key[3] | clct_second_key[3] | E5 | TP341-4 |
| mpc_tx[20] | alct_second_key[4] | clct_second_key[4] | A6 | TP341-5 |
| mpc_tx[21] | alct_second_key[5] | clct_second_key[5] | B6 | TP341-6 |
| mpc_tx[22] | alct_second_key[6] | clct_second_key[6] | D6 | TP341-7 |
| mpc_tx[23] | clct_second_pat[0] | clct_second_key[7] | E6 | TP341-8 |
| mpc_tx[24] | clct_second_pat[1] | lct_second_bend | A7 | TP342-1 |
| mpc_tx[25] | clct_second_pat[2] | clct_second_sync_err | B7 | TP342-2 |
| mpc_tx[26] | clct_second_pat[3] | alct_second_bxn[0] | D7 | TP342-3 |
| mpc_tx[27] | lct_second_quality[0] | clct_second_bx0_local | E7 | TP342-4 |
| mpc_tx[28] | lct_second_quality[1] | csc_id[0] | A8 | TP342-5 |
| mpc_tx[29] | lct_second_quality[2] | csc_id[1] | B8 | TP342-6 |
| mpc_tx[30] | lct_second_quality[3] | csc_id[2] | D8 | TP342-7 |
| mpc_tx[31] | second_vpf | csc_id[3] | E8 | TP342-8 |

# VME Registers

### Addressing Modes

TMB2005 responds to A24D16 VME addressing modes:

Address Modifier 3916,   A24 non-privileged mode

Address Modifier 3D16,  A24 supervisor mode

# It does not respond to byte-addressing modes, so all valid addresses must be even numbers.

### Base Address

TMB2005s “base address” bits A[23:19] select which TMB is being addressed by the VME crate controller.  The base address is determined either by the 5 VME-backplane-slot Geographic Address bits or by the Local Address set by two on-board hexadecimal rotary switches. Shunt SH62 selects between Geographic [1-2] and Local [2-3] modes.

A[23:19] = VME Crate Slot Geographic Address, (Slot= 2 to 21)10		SH62 [1-2]

A[23:19] = Hexadecimal Switch Address SW2x16+SW1			SH62 [2-3]

Multiple TMBs can be addressed simultaneously using a Global Address:

A[23:19] = 2610  Addresses all TMBs in parallel

A[23:19] = 2710  Address all peripheral crate modules

### Boot Register

When geographic addressing is used, the S2/S1 hexadecimal switches should be set to 1Ah,

which allows the hardware Boot Register to respond to the slot 2610 global address.

The Boot Register responds to all even VME addresses + base between 70000h and 7FFFEh to allow block-mode VME writes.

### Register Addresses

| Address Hexadecimal Add to Base | Register Name | Description |
| --- | --- | --- |
| 70000 | ADR_BOOT | Hardware Bootstrap Register |
| 00 | ADR_IDREG0 | ID Register 0 |
| 02 | ADR_IDREG1 | ID Register 1 |
| 04 | ADR_IDREG2 | ID Register 2 |
| 06 | ADR_IDREG3 | ID Register 3 |
| 08 | ADR_VME_STATUS | VME Status Register |
| 0A | ADR_VME_ADR0 | VME Address read-back |
| 0C | ADR_VME_ADR1 | VME Address read-back |
| 0E | ADR_LOOPBK | Loop-back Register |
| 10 | ADR_USR_JTAG | User JTAG |
| 12 | ADR_PROM | PROM |
| 14 | ADR_DDDSM | 3D3444 State Machine Register + Clock DCMs |
| 16 | ADR_DDD0 | 3D3444 Delay Chip 0 |
| 18 | ADR_DDD1 | 3D3444 Delay Chip 1 |
| 1A | ADR_DDD2 | 3D3444 Delay Chip 2 |
| 1C | ADR_DDDOE | 3D3444 Delay Chip Output Enables |
| 1E | ADR_RATCTRL | RAT Module Control |
| 20 | ADR_STEP | Step Register |
| 22 | ADR_LED | Front Panel +On-Board LEDs |
| 24 | ADR_ADC | ADCs |
| 26 | ADR_DSN | Digital Serials |
| 28 | ADR_MOD_CFG | TMB Configuration |
| 2A | ADR_CCB_CFG | CCB Configuration |
| 2C | ADR_CCB_TRIG | CCB Trigger Control |
| 2E | ADR_CCB_STAT0 | CCB Status |
| 30 | ADR_ALCT_CFG | ALCT Configuration |
| 32 | ADR_ALCT_INJ | ALCT Injector Control |
| 34 | ADR_ALCT0_INJ | ALCT Injected ALCT0 |
| 36 | ADR_ALCT1_INJ | ALCT Injected ALCT1 |
| 38 | ADR_ALCT_STAT | ALCT Sequencer Control/Status |
| 3A | ADR_ALCT0_RCD | ALCT LCT0 Received by TMB |
| 3C | ADR_ALCT1_RCD | ALCT LCT1 Received by TMB |
| 3E | ADR_ALCT_FIFO | ALCT FIFO RAM Status |
| 40 | ADR_DMB_MON | DMB Monitored signals |
| 42 | ADR_CFEB_INJ | CFEB Injector Control |
| 44 | ADR_CFEB_INJ_ADR | CFEB Injector RAM address |
| 46 | ADR_CFEB_INJ_WDATA | CFEB Injector Write Data |
| 48 | ADR_CFEB_INJ_RDATA | CFEB Injector Read Data |
| 4A | ADR_HCM001 | CFEB0 Ly0,Ly1 Hot Channel Mask |
| 4C | ADR_HCM023 | CFEB0 Ly2,Ly3 Hot Channel Mask |
| 4E | ADR_HCM045 | CFEB0 Ly4,Ly5 Hot Channel Mask |
| 50 | ADR_HCM101 | CFEB1 Ly0,Ly1 Hot Channel Mask |
| 52 | ADR_HCM123 | CFEB1 Ly2,Ly3 Hot Channel Mask |
| 54 | ADR_HCM145 | CFEB1 Ly4,Ly5 Hot Channel Mask |
| 56 | ADR_HCM201 | CFEB2 Ly0,Ly1 Hot Channel Mask |
| 58 | ADR_HCM223 | CFEB2 Ly2,Ly3 Hot Channel Mask |
| 5A | ADR_HCM245 | CFEB2 Ly4,Ly5 Hot Channel Mask |
| 5C | ADR_HCM301 | CFEB3 Ly0,Ly1 Hot Channel Mask |
| 5E | ADR_HCM323 | CFEB3 Ly2,Ly3 Hot Channel Mask |
| 60 | ADR_HCM345 | CFEB3 Ly4,Ly5 Hot Channel Mask |
| 62 | ADR_HCM401 | CFEB4 Ly0,Ly1 Hot Channel Mask |
| 64 | ADR_HCM423 | CFEB4 Ly2,Ly3 Hot Channel Mask |
| 66 | ADR_HCM445 | CFEB4 Ly4,Ly5 Hot Channel Mask |
| 68 | ADR_SEQ_TRIG_EN | Sequencer Trigger Source Enables |
| 6A | ADR_SEQ_TRIG_DLY0 | Sequencer Trigger Source Delays |
| 6C | ADR_SEQ_TRIG_DLY1 | Sequencer Trigger Source Delays |
| 6E | ADR_SEQ_ID | Sequencer Board + CSC ID |
| 70 | ADR_SEQ_CLCT | Sequencer CLCT Configuration |
| 72 | ADR_SEQ_FIFO | Sequencer FIFO Configuration |
| 74 | ADR_SEQ_L1A | Sequencer L1A Configuration |
| 76 | ADR_SEQ_OFFSET0 | Sequencer Counter Offsets |
| 78 | ADR_SEQ_CLCT0 | Sequencer Latched CLCT0 |
| 7A | ADR_SEQ_CLCT1 | Sequencer Latched CLCT1 |
| 7C | ADR_SEQ_TRIG_SRC | Sequencer Trigger Source Read-back |
| 7E | ADR_DMB_RAM_ADR | Sequencer RAM Address |
| 80 | ADR_DMB_RAM_WDATA | Sequencer RAM Write Data |
| 82 | ADR_DMB_RAM_WDCNT | Sequencer RAM Word Count |
| 84 | ADR_DMB_RAM_RDATA | Sequencer RAM Read Data |
| 86 | ADR_TMB_TRIG | TMB Trigger Configuration / MPC Accept |
| 88 | ADR_MPC0_FRAME0 | MPC0 Frame 0 Data sent to MPC |
| 8A | ADR_MPC0_FRAME1 | MPC0 Frame 1 Data sent to MPC |
| 8C | ADR_MPC1_FRAME0 | MPC1 Frame 0 Data sent to MPC |
| 8E | ADR_MPC1_FRAME1 | MPC1 Frame 1 Data sent to MPC |
| 90 | ADR_MPC_INJ | MPC Injector Control |
| 92 | ADR_MPC_RAM_ADR | MPC Injector RAM address |
| 94 | ADR_MPC_RAM_WDATA | MPC Injector RAM Write Data |
| 96 | ADR_MPC_RAM_RDATA | MPC Injector RAM Read Data |
| 98 | ADR_SCP_CTRL | Scope control |
| 9A | ADR_SCP_RDATA | Scope read data |
| 9C | ADR_CCB_CMD | CCB TTC Command Generator |
| 9E | ADR_BUF_STAT0 | Buffer Status |
| A0 | ADR_BUF_STAT1 | Buffer Status |
| A2 | ADR_BUF_STAT2 | Buffer Status |
| A4 | ADR_BUF_STAT3 | Buffer Status |
| A6 | ADR_BUF_STAT4 | Buffer Status |
| A8 | ADR_ALCT_FIFO1 | ALCT Raw hits RAM Control |
| AA | ADR_ALCT_FIFO2 | ALCT Raw hits RAM data |
| AC | ADR_SEQMOD | Sequencer Trigger Modifiers |
| AE | ADR_SEQSM | Sequencer Machine State |
| B0 | ADR_SEQCLCTM | Sequencer CLCT msbs |
| B2 | ADR_TMBTIM | TMB Timing for ALCT*CLCT coincidence |
| B4 | ADR_LHC_CYCLE | LHC Cycle period, Maximum BXN+1 |
| B6 | ADR_RPC_CFG | RPC Configuration |
| B8 | ADR_RPC_RDATA | RPC Sync Mode Read Data |
| BA | ADR_RPC_RAW_DELAY | RPC Raw Hits Delay + RPC BXN Differences |
| BC | ADR_RPC_INJ | RPC Injector Control |
| BE | ADR_RPC_INJ_ADR | RPC Injector RAM Addresses |
| C0 | ADR_RPC_INJ_WDATA | RPC Injector Write Data |
| C2 | ADR_RPC_INJ_RDATA | RPC Injector Read Data |
| C4 | ADR_RPC_TBINS | RPC FIFO Time Bins |
| C6 | ADR_RPC0_HCM | RPC0 Hot Channel Mask |
| C8 | ADR_RPC1_HCM | RPC1 Hot Channel Mask |
| CA | ADR_BX0_DELAY | BX0 to MPC Delays |
| CC | ADR_NON_TRIG_RO | Non-triggering Event Enables |
| CE | ADR_SCP_TRIG | Scope Trigger Source Channel |
| D0 | ADR_CNT_CTRL | Status Counter Control |
| D2 | ADR_CNT_RDATA | Status Counter Data |
| D4 | ADR_JTAGSM0 | JTAG State Machine Control (reads JTAG PROM) |
| D6 | ADR_JTAGSM1 | JTAG State Machine Word Count |
| D8 | ADR_JTAGSM2 | JTAG State Machine Checksum |
| DA | ADR_VMESM0 | VME State Machine Control (reads VME PROM) |
| DC | ADR_VMESM1 | VME State Machine Word Count |
| DE | ADR_VMESM2 | VME State Machine Checksum |
| E0 | ADR_VMESM3 | Number of VME Addresses Written by VMESM |
| E2 | ADR_VMESM4 | VME State Machine Write-Data Check |
| E4 | ADR_DDDRSM | RAT 3D3444 State Machine Control |
| E6 | ADR_DDDR0 | RAT 3D3444 RPC Delays |
| E8 | ADR_UPTIME | Uptime Counter |
| EA | ADR_BDSTATUS | Board Status Summary |
| EC | ADR_BXN_CLCT | CLCT BXN At CLCT-Pretrigger |
| EE | ADR_BXN_ALCT | ALCT BXN At ALCT-Valid-Pattern-Flag |
| F0 | ADR_LAYER_TRIG | Layer-Trigger Mode |
| F2 | ADR_ISE_VERSION | ISE Version + Service Pack |
| F4 | ADR_TEMP0 | Pattern Finder Pre-Trigger |
| F6 | ADR_TEMP1 | CLCT Separation |
| F8 | ADR_TEMP2 | CLCT Separation RAM Data |
| FA | ADR_PARITY |  |
| FC | ADR_CCB_STAT1 |  |
| FE | ADR_BXN_L1A | CLCT BXN at last L1A arrival |
| 100 | ADR_L1A_LOOKBACK | L1A Lookback distance |
| 102 | ADR_SEQ_DEBUG | Sequencer debug signals |
| 104 | ADR_ALCT_SYNC_CTRL | ALCT sync mode control |
| 106 | ADR_ALCT_SYNC_TXDATA_1ST | ALCT sync mode transmit data 1st |
| 108 | ADR_ALCT_SYNC_TXDATA_2ND | ALCT sync mode transmit data 2nd |
| 10A | ADR_SEQ_OFFSET1 | Sequencer Counter Offsets Continued |
| 10C | ADR_MINISCOPE | Internal 16 Channel Digital Miniscope |
| 10E | ADR_PHASER0 | ALCT rxd delay digital phase shifter |
| 110 | ADR_PHASER1 | ALCT txd delay digital phase shifter |
| 112 | ADR_PHASER2 | CFEB0 rxd delay digital phase shifter |
| 114 | ADR_PHASER3 | CFEB1 rxd delay digital phase shifter |
| 116 | ADR_PHASER4 | CFEB2 rxd delay digital phase shifter |
| 118 | ADR_PHASER5 | CFEB3 rxd delay digital phase shifter |
| 11A | ADR_PHASER6 | CFEB4 rxd delay digital phase shifter |
| 11C | ADR_DELAY0_INT | CFEB0-3 DDR RxD Interstage delays |
| 11E | ADR_DELAY1_INT | CFEB4 DDR RxD Interstage delays Continued |
| 120 | ADR_SYNC_ERR_CTRL | Synchronization Error Control |
| 122 | ADR_CFEB_BADBITS_CTRL | CFEB Bad Bit Control/Status |
| 124 | ADR_CFEB_BADBITS_TIMER | CFEB Bad Bit Check Interval |
| 126 | ADR_CFEB0_BADBITS_LY01 | CFEB0 Bad Bits Array |
| 128 | ADR_CFEB0_BADBITS_LY23 | CFEB0 Bad Bits Array |
| 12A | ADR_CFEB0_BADBITS_LY45 | CFEB0 Bad Bits Array |
| 12C | ADR_CFEB1_BADBITS_LY01 | CFEB1 Bad Bits Array |
| 12E | ADR_CFEB1_BADBITS_LY23 | CFEB1 Bad Bits Array |
| 130 | ADR_CFEB1_BADBITS_LY45 | CFEB1 Bad Bits Array |
| 132 | ADR_CFEB2_BADBITS_LY01 | CFEB2 Bad Bits Array |
| 134 | ADR_CFEB2_BADBITS_LY23 | CFEB2 Bad Bits Array |
| 136 | ADR_CFEB2_BADBITS_LY45 | CFEB2 Bad Bits Array |
| 138 | ADR_CFEB3_BADBITS_LY01 | CFEB3 Bad Bits Array |
| 13A | ADR_CFEB3_BADBITS_LY23 | CFEB3 Bad Bits Array |
| 13C | ADR_CFEB3_BADBITS_LY45 | CFEB3 Bad Bits Array |
| 13E | ADR_CFEB4_BADBITS_LY01 | CFEB4 Bad Bits Array |
| 140 | ADR_CFEB4_BADBITS_LY23 | CFEB4 Bad Bits Array |
| 142 | ADR_CFEB4_BADBITS_LY45 | CFEB4 Bad Bits Array |
| 144 | ADR_ALCT_STARTUP_DELAY | ALCT startup delay milliseconds for Spartan-6 |
| 146 | ADR_ALCT_STARTUP_STATUS | ALCT startup delay machine status |
|  | Virtex-6 VME Registers |  |
| 148 | ADR_V6_SNAP12_QPLL | Virtex-6 SNAP12 Serial interface + QPLL status |
| 14A | ADR_V6_GTX_RX_ALL | Virtex-6 GTX common control and status |
| 14C | ADR_V6_GTX_RX0 | Virtex-6 GTX0 control and status |
| 14E | ADR_V6_GTX_RX1 | Virtex-6 GTX1 control and status |
| 150 | ADR_V6_GTX_RX2 | Virtex-6 GTX2 control and status |
| 152 | ADR_V6_GTX_RX3 | Virtex-6 GTX3 control and status |
| 154 | ADR_V6_GTX_RX4 | Virtex-6 GTX4 control and status |
| 156 | ADR_V6_GTX_RX5 | Virtex-6 GTX5 control and status |
| 158 | ADR_V6_GTX_RX6 | Virtex-6 GTX6 control and status |
| 15A | ADR_V6_SYSMON | Virtex-6 Sysmon ADC |
| 15C | ADR_V6_CFEB_BADBITS_CTRL | CFEB Bad Bit Control/Status extends Adr 122 |
| 15E | ADR_V6_CFEB5_BADBITS_LY01 | CFEB5 Bad Bit Array |
| 160 | ADR_V6_CFEB5_BADBITS_LY23 | CFEB5 Bad Bit Array |
| 162 | ADR_V6_CFEB5_BADBITS_LY45 | CFEB5 Bad Bit Array |
| 164 | ADR_V6_CFEB6_BADBITS_LY01 | CFEB6 Bad Bit Array |
| 166 | ADR_V6_CFEB6_BADBITS_LY23 | CFEB6 Bad Bit Array |
| 168 | ADR_V6_CFEB6_BADBITS_LY45 | CFEB6 Bad Bit Array |
| 16A | ADR_V6_PHASER7 | Phaser 7 cfeb5_rxd phase |
| 16C | ADR_V6_PHASER8 | Phaser 8 cfeb6_rxd phase |
| 16E | ADR_V6_HCM501 | CFEB5 Ly0,Ly1 Hot Channel Mask |
| 170 | ADR_V6_HCM523 | CFEB5 Ly2,Ly3 Hot Channel Mask |
| 172 | ADR_V6_HCM545 | CFEB5 Ly4,Ly5 Hot Channel Mask |
| 174 | ADR_V6_HCM601 | CFEB6 Ly0,Ly1 Hot Channel Mask |
| 176 | ADR_V6_HCM623 | CFEB6 Ly2,Ly3 Hot Channel Mask |
| 178 | ADR_V6_HCM645 | CFEB6 Ly4,Ly5 Hot Channel Mask |
| 17A | ADR_V6_EXTEND | DCFEB 7-bit extensions to 5 bit fields in 0x42,68 |

### Register Definitions

#### Adr 7000016	ADR_BOOT	Hardware Bootstrap Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R=tdo | R=ready | hard reset RPC/RAT | /mez clock enable | /fpga vme_en | /en_fpga reset_alct | hard reset TMB | hard reset ALCT | JTAG source vme/fpga | sel3 | sel2 | sel1 | sel0 | tck | tms | Tdi |

| Bit | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [ 0] | RW | jtag_vme1 (tdi) | 0 | vme tdi |
| [ 1] | RW | jtag_vme2 (tms) | 0 | vme tms |
| [ 2] | RW | jtag_vme3 (tck) | 0 | vme tck |
| [ 3] | RW | sel_vme0 | 0 | 00XX ALCT JTAG Chain |
| [ 4] | RW | sel_vme1 | 0 | 01XX TMB Mezzanine FPGA + FPGA PROMs Chain |
| [ 5] | RW | sel_vme2 | 0 | 10XX TMB User PROMs JTAG chain |
| [ 6] | RW | sel_vme3 | 0 | 11XX TMB FPGA User JTAG chain |
| [ 7] | RW | vme/usr_en | 0 | 1=JTAG sourced by Bootstrap Register, 0= from FPGA |
| [ 8] | RW | hard_reset_alct_vme | 0 | 1=Hard reset to ALCT FPGA |
| [ 9] | RW | hard_reset_tmb_vme | 0 | 1=Hard reset to TMB FPGA |
| [10] | RW | /en_fpga_reset_alct | 0 | 0=Allow TMB FPGA to hard reset ALCT |
| [11] | RW | /fpga_vme_en | 0 | 0=Allow TMB FPGA to issue VME commands |
| [12] | RW | /mez_clock_en | 0 | 0=Enable TMB FPGA mezzanine clock |
| [13] | RW | hard_reset_rpc | 0 | 1=Hard reset to RPC (RAT) FPGA |
| [14] | R | vme_ready | x | 1=FPGA vme logic indicates ready |
| [15] | R | jtag_vme0 (tdo) | 0 | vme tdo |
| [14] | W | unassigned | - | No connection on PCB |
| [15] | W | unassigned | - | No connection on PCB |

#### Adr 00	ADR_IDREG0	ID Register 0

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | ga4 | ga3 | ga2 | ga1 | ga0 | fvers3 | fvers2 | fvers1 | fvers0 | ftype3 | ftype2 | ftype1 | ftype0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [03:00] | R | C | Firmware type, C=Normal CLCT/TMB, D=Debug loopback |
| [07:04] | R | D | Firmware version code |
| [12:08] | R | 15 | Geographic address for this board |
| [15:13] | R | 0 | Unassigned |

#### Adr 02	ADR_IDREG1	ID Register 1

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| month msd3 | month msd2 | month msd1 | month msd0 | month lsd3 | month lsd2 | month lsd1 | month lsd0 | day msd3 | day msd2 | day msd1 | day msd0 | day lsd3 | day lsd2 | day lsd1 | day lsd0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [07:00] | R | 09 | DD Firmware Version Day (BCD) |
| [15:08] | R | 04 | MM Firmware Version Month (BCD) |

#### Adr 04	ADR_IDREG2	ID Register 2

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| year digit3 3 | year digit3 2 | year digit3 1 | year digit3 0 | year digit2 3 | year digit2 2 | year digit2 1 | year digit2 0 | year digit1 3 | year digit1 2 | year digit1 1 | year digit1 0 | year digit0 3 | year digit0 2 | year digit0 1 | year digit0 0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [15:00] | R | 2007 | YYYY Firmware Version Year (BCD) |

#### Adr 06	ADR_IDREG3	ID Register 3

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rev code 15 | rev code 14 | rev code 13 | rev code 12 | rev code 11 | rev code 10 | rev code 9 | rev code 8 | rev code 7 | rev code 6 | rev code 5 | rev code 4 | rev code 3 | rev code 2 | rev code 1 | rev code 0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [15:00] | R |  | Firmware Revcode (as stored in raw hits header) |

#### Adr 08	ADR_VME_STATUS	VME Status Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMB ready | local/geo | iack | acfail | sysreset | sysfail | sysclk | ds1 | as | lword | gap | ga4 | ga3 | ga2 | ga1 | ga0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [04:00] | R |  | Crate slot Geographic Address |
| [05] | R |  | Crate slot Geographic Address Parity |
| [06] | R |  | VME signal lword |
| [07] | R |  | VME signal as |
| [08] | R |  | VME signal ds1 |
| [09] | R |  | VME signal sysclk |
| [10] | R |  | VME signal sysfail |
| [11] | R |  | VME signal sysreset |
| [12] | R |  | VME signal acfail |
| [13] | R |  | VME signal iack |
| [14] | R |  | 1=Address mode set to local, 0=Geographic |
| [15] | R |  | 1=TMB reports ready to boot register |

#### Adr 0A	ADR_VME_ADR0	VME Address Read-Back

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a15 | a14 | a13 | a12 | a11 | a10 | a9 | a8 | a7 | a6 | a5 | a4 | a3 | a2 | a1 | lword |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [15:00] | R | a[15:0] | VME Address captured at last write cycle {a[15:1},lword} |

#### Adr 0C	ADR_VME_ADR1	VME Address Read-Back

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMB ready | local/geo | iack | acfail | sysreset | sysfail | sysclk | ds1 | as | lword | gap | ga4 | ga3 | ga2 | ga1 | ga0 |

| Bits | Dir | Typical | Description |
| --- | --- | --- | --- |
| [07:00] | R | a[23:16] | VME Address captured at last write cycle |
| [13:08] | R | am[5:0] | VME Address modifier |
| [15:14] | R | 0 | Unassigned |

#### Adr 0E	ADR_LOOPBK	Loop-Back Control Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | dmb tx res 2 | dmb tx res 1 | dmb tx res 0 | gtl_ oe | gtl_ loop | dmb_ oe | dmb_ loop | rpc loop bdtest | rpc_ loop_ tmb | rpc_ loop_ rat | alct_ txoe | alct_ rxoe | alct_ loop | cfeb_oe |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | R | cfeb_oe | 1 | 1=CFEB output enable |
| [01] | R | alct_loop | 0 | 0=No ALCT loop-back |
| [02] | RW | alct_rxoe | 1 | 1=Enable RAT ALCT LVDS receiver, 0=power down |
| [03] | RW | alct_txoe | 1 | 1=Enable RAT ALCT LVDS transmitter, 0=power down |
| [04] | R | rpc_loop_rat | 0 | 1=RAT FPGA enters loop-back mode |
| [05] | R | rpc_loop_bdtest | 0 | 1=En RPC Loop-back (no RAT ), used only in bdtest firmware |
| [06] | R | rpc_loop_tmb | 0 | 1=TMBs RAT backplane ICs loop-back mode |
| [07] | R | dmb_loop | 0 | 0=No DMB loop-back |
| [08] | R | dmb_oe | 0 | 0=DMB driver enable |
| [09] | R | gtl_loop | 0 | 0=No GTL loop-back |
| [10] | R | gtl_oe | 0 | 0=Enable GTL outputs |
| [13:11] | RW | dmb_tx_reserved[2:0] | 0 | dmb_tx[48:46] unused, set to 0 |
| [15:14] | RW | -- |  | Unassigned |

#### Adr 10	ADR_USR_JTAG	User JTAG Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tdo usr | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | sel3 usr | sel2 usr | sel1 usr | sel0 usr | tck usr | tms usr | tdi usr |

| Bits | Dir | Signal | Description |
| --- | --- | --- | --- |
| [00] | RW | tdi_usr | User JTAG Chain TDI (output from FPGA) |
| [01] | RW | tms_usr | User JTAG Chain TMS |
| [02] | RW | tck_usr | User JTAG Chain TCK |
| [06:03] | RW | sel_usr[3:0] | User JTAG Chain Select, 0=ALCT,1=Mez,2=UserPROMs,3=UserChain |
| [13:07] | RW | -- | Unassigned |
| [14] | RW | wr_usr_jtag_dis | 1=disable write access to ADR_USR_JTAG, set in Adr D4[11] |
| [15] | R | tdo_usr | User JTAG Chain TDO (input to FPGA) |

#### Adr 12	ADR_PROM	User PROMs Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | prom_ src | prom1 ce | prom1 oe | prom1 clk | prom0 ce | prom0 oe | prom0 clk | prom_ led7 | prom_ led6 | prom_ led5 | prom_ led4 | prom_ led3 | prom_ led2 | prom_ led1 | prom_ led0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | prom_led[7:0] | CD | PROM data bus shared with On-Board LEDs |
| [08] | RW | prom0_clk | 0 | PROM 0 clock |
| [09] | RW | prom0_oe | 0 | PROM 0 output enable |
| [10] | RW | prom0_ce | 1 | PROM 0 /chip_enable |
| [11] | RW | prom1_clk | 0 | PROM 1 clock |
| [12] | RW | prom1_oe | 0 | PROM 1 output enable |
| [13] | RW | prom1_ce | 1 | PROM 1 /chip_enable |
| [14] | RW | prom_src | 0 | Data bus 0=on-board LEDs, 1=enabled PROM |
| [15] | RW | -- | 0 | Unassigned |

#### Adr 14	ADR_DDDSM	3D3444 State Machine Control + DCM Lock Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rpc lock | dcc lock | mpc lock | alctd lock | alct rxclk lock | tmb clock1 lock | tmb0d lock | tmb clock0 lock | ddd verify | dddsm busy | auto start | serial from | serial to ddd | adr latch | ddd clock | ddd start |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | ddd_start_vme | 0 | Start DDD State Machine |
| [01] | RW | ddd_clock | 0 | DDD manual-mode clock |
| [02] | RW | ddd_adr_latch | 1 | DDD manual-mode address latch, active low |
| [03] | RW | ddd_serial_in | 0 | Serial data to DDD chain |
| [04] | RW | ddd_serial_out | 0 | Serial data from DDD chain |
| [05] | RW | ddd_auto_start | 1 | DDD State Machine autostart state |
| [06] | R | ddd_busy | 0 | DDD State Machine busy |
| [07] | R | ddd_verify_ok | 1 | DDD data read back verified OK |
| [08] | R | lock_tmb_clock0 | 1 | TMB clock 0 DCM locked |
| [09] | R | lock_tmb_clock0d | 1 | TMB clock 0d DCM locked |
| [10] | R | lock_tmb_clock1 | 1 | TMB clock 1 DCM locked |
| [11] | R | lock_alct_rxclock | 1 | ALCT rxclock DCM locked |
| [12] | R | lock_alct_clockd | 1 | ALCT rxclockd DCM locked |
| [13] | R | lock_mpc_clock | 1 | CFEB rxd clock DCM locked (was mpc) |
| [14] | R | lock_dcc_clock | 1 | DCC clock DCM locked |
| [15] | R | lock_rpc_clock | 1 | RPC clock DCM locked |

#### Adr 16	ADR_DDD0	3D3444 Chip 0 Delays, 1 step = 2ns

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | delay_ch0[3:0] | 0 | alct_tof_delay, shift entire ALCT in clockspace |
| [07:04] | RW | delay_ch1[3:0] | 1 | alct_rxclock delay, not used in muonic firmware |
| [11:08] | RW | delay_ch2[3:0] | 6 | DMB tx clock |
| [15:12] | RW | delay_ch3[3:0] | 9 | RPC tx clock |

#### Adr 18	ADR_DDD1	3D3444 Chip 1 Delays, 1 step = 2ns

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | delay_ch4[3:0] | 0 | tmb_clock1, not used |
| [07:04] | RW | delay_ch5[3:0] | 0 | mpc_clock not used |
| [11:08] | RW | delay_ch6[3:0] | 0 | cfeb_tof_delay, shift all cfebs in clockspace |
| [15:12] | RW | delay_ch7[3:0] | 7 | CFEB 0 clock |

#### Adr 1A	ADR_DDD2	3D3444 Chip 2 Delays, 1 step = 2ns

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | delay_ch8[3:0] | 7 | CFEB 1 clock |
| [07:04] | RW | delay_ch9[3:0] | 7 | CFEB 2 clock |
| [11:08] | RW | delay_ch10[3:0] | 7 | CFEB 3 clock |
| [15:12] | RW | delay_ch11[3:0] | 7 | CFEB 4 clock |

#### Adr 1C	ADR_DDDOE	3D3444 Chip Output Enables

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | cfeb 4 | cfeb 3 | cfeb 2 | cfeb 1 | cfeb 0 | dcc | mpc | tmb1 | rpc tx | dmb tx | alct rx | alct tx |

| [11:00] | RW | ddd_oe[11:0] | FFF | Bit(n)=1=Enable DDD output channel n |
| --- | --- | --- | --- | --- |
| [15:12] | RW | Unassigned | 0 | Unassigned |

#### Adr 1E	ADR_RATCTRL	RAT Module Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | rpc dsn en | rpc free | rpc lptm | rpc posneg | rpc sync |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [0] | RW | rpc_sync | 0 | 1=RPC 80MHz sync pattern mode |
| [1] | RW | rpc_posneg | 0 | 1=shift RPC data ½ cycle in RAT FPGA + dsn |
| [2] | RW | rpc_lptmb | 0 | Not used (for matching rpc_tx array) |
| [3] | RW | rpc_free_tx[0] | 0 | Unassigned |
| [4] | RW | rat_dsn_en | 0 | 1=Enable RAT dsn readout |
| [15:5] | RW | -- | 0 | Unassigned |

#### Adr 20	ADR_STEP	Clock Single-Step + Hard Resets

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | /tmb hard | /alct hard | alct clken | cfeb4 clken | cfeb3 clken | cfeb2 clken | cfeb1 clken | cfeb0 clken | step run | step cfeb | step rpc | step dmb | step alct |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | step_alct | 0 | Step ALCT clock |
| [01] | RW | step_dmb | 0 | Step DMB clock |
| [02] | RW | step_rpc | 0 | Step RPC clock |
| [03] | RW | step_cfeb | 0 | Step CFEB clock |
| [04] | RW | step_run | 0 | 0=run mode, 1=step clocks |
| [05] | RW | cfeb_clock_en0 | 1 | 1=enable CFEB0 clock |
| [06] | RW | cfeb_clock_en1 | 1 | 1=enable CFEB1 clock |
| [07] | RW | cfeb_clock_en2 | 1 | 1=enable CFEB2 clock |
| [08] | RW | cfeb_clock_en3 | 1 | 1=enable CFEB3 clock |
| [09] | RW | cfeb_clock_en4 | 1 | 1=enable CFEB4 clock |
| [10] | RW | alct_clock_en | 1 | 1=enable ALCT clock |
| [11] | RW | /alct_hard_reset_en | 1 | 1=disable ALCT hard reset |
| [12] | RW | /tmb_hard_reset_en | 1 | 1=disable TMB hard reset |
| [15:13] | RW | -- | 0 | Unassigned |

#### Adr 22	ADR_LED	Front Panel + On-Board LED Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| led bd7 | led bd6 | led bd5 | led bd4 | led bd3 | led bd2 | led bd1 | led bd0 | VME | NL1A | NMAT | INVP | L1A | CLCT | ALCT | LCT |

| Bits | Dir | Signal | Color | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | led_fp_lct | Blue | LCT TMB matched ALCT+CLCT |
| [01] | RW | led_fp_lct | Green | ALCT found a muon |
| [02] | RW | led_fp_clct | Green | CLCT found a muon |
| [03] | RW | led_fp_l1a | Green | L1A level 1 accept |
| [04] | RW | led_fp_invp | Amber | INVP invalid pattern after CSC drift |
| [05] | RW | led_fp_nmat | Amber | NMAT no match after ALCT or CLCT triggered |
| [06] | RW | led_fp_nl1a | Red | NL1A no L1A after trigger |
| [07] | RW | led_fp_vme | Green | VME power-up = on, off=vme access flash |
| [08] | RW | led_bd0 | Blue | Buffer busy[0] |
| [09] | RW | led_bd1 | Green | Buffer busy[1] |
| [10] | RW | led_bd2 | Green | Buffer busy[2] |
| [11] | RW | led_bd3 | Green | Buffer busy[3] |
| [12] | RW | led_bd4 | Green | Buffer busy[4] |
| [13] | RW | led_bd5 | Green | Buffer busy[5] |
| [14] | RW | led_bd6 | Green | Buffer busy[6] |
| [15] | RW | led_bd7 | Red | Buffer busy[7] |

#### Adr 24	ADR_ADC	ADC + Power Comparator Register

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | smb data | smb clk | ADC /cs | ADC din | ADC sclock | ADC dout | /tcrit | V1.5 | V1.8 | V3.3 | V5.0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | vstat_5p0v | 1 | 1 = 5.0V power supply OK |
| [01] | R | vstat_3p3v | 1 | 1 = 3.3V power supply OK |
| [02] | R | vstat_1p8v | 1 | 1 = 1.8V power supply OK |
| [03] | R | vstat_1p5v | 1 | 1 = 1.5V power supply OK |
| [04] | R | /t_crit | 1 | 1 = FPGA and Board Temperature OK |
| [05] | R | adc_dout | 0 | Voltage monitor ADC serial data receive |
| [06] | RW | adc_sclock | 0 | Voltage monitor ADC serial clock |
| [07] | RW | adc_din | 0 | Voltage monitor ADC serial data transmit |
| [08] | RW | /adc_cs | 1 | Voltage monitor ADC chip select |
| [09] | RW | smb_clk | 0 | Temperature monitor ADC serial clock |
| [10] | RW | smb_data | 1 | Temperature monitor ADC serial data, open drain |
| [15:11] | RW | -- | 0 | Unassigned |

#### Adr 26	ADR_DSN	Digital Serial Numbers

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RAT DSN Data | RAT DSN Busy | RAT DSN Init | RAT DSN Write | RAT DSN Start | Mez DSN Data | Mez DSN Busy | Mez DSN Init | Mez DSN Write | Mez DSN Start | TMB DSN Data | TMB DSN Busy | TMB DSN Init | TMB DSN Write | TMB DSN Start |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | tmb_sn_start | 0 | TMB Digital serial SM start |
| [01] | RW | tmb_sn_write | 0 | TMB Digital serial write pulse |
| [02] | RW | tmb_sn_init | 0 | TMB Digital serial Init pulse |
| [03] | R | tmb_sn_busy | - | TMB State DSN State Machine busy |
| [04] | R | tmb_sn_data | - | TMB State DSN read data |
| [05] | RW | mez_sn_start | 0 | Mez Digital Serial State Machine start |
| [06] | RW | mez_sn_write | 0 | Mez Digital Serial Write pulse |
| [07] | RW | mez_sn_init | 0 | Mez Digital Serial Init pulse |
| [08] | R | mez_sn_busy | - | Mez State DSN State Machine busy |
| [09] | R | mez_sn_data | - | Mez State DSN read data |
| [10] | RW | rat_sn_start | 0 | RAT Digital Serial State Machine start |
| [11] | RW | rat_sn_write | 0 | RAT Digital Serial Write pulse |
| [12] | RW | rat_sn_init | 0 | RAT Digital Serial Init pulse |
| [13] | R | rat_sn_busy | - | RAT State DSN State Machine busy |
| [14] | R | rat_sn_data | - | RAT State DSN read data |
| [15] | RW | - | 0 | Unassigned |

#### Adr 28	ADR_MOD_CFG	TMB Module Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mez done | ddd auto | power up | global reset enable | cfeb6 exists | cfeb5 exists | cfeb4 exists | cfeb3 exists | cfeb2 exists | cfeb1 exists | cfeb0 exists | bdled cylon | bdled vme | fpled flash | fpled cylon | fpled vme |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | led_fp_src_vme | 0 | 1=Front Panel LEDs sourced from VME register |
| [01] | RW | led_fp_cylon | 0 | 1=FP LED Cylon mode, cool |
| [02] | RW | led_flash_on_stop | 1 | 1=Flash Front Panel LEDs on TTT stop_trigger |
| [03] | RW | led_bd_src_vme | 0 | 1=On-Board LEDs sourced from VME register |
| [04] | RW | led_bd_cylon | 0 | 1=BD LED Cylon mode, cool |
| [11:5] | R | cfeb_exists[6:0] | 7F | CFEB(n) instantiated in this firmware version |
| [12] | RW | global_reset_en | 1 | 1=fire global reset if main DLL loses lock |
| [13] | R | power_up |  | Power-up FF |
| [14] | R | ddd_autostart |  | 1=3D3444 auto-start enabled, copy of Adr14[05] |
| [15] | R | mez_done |  | 1=Mezzanine FPGA loaded from PROM |

#### Adr 2A	ADR_CCB_CFG	CCB Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| adb pulse async | adb pulse sync | alct hard reset | tmb hard reset | tmb resout2 | tmb resout1 | tmb resout0 | tmb res1 | tmb res0 | l1a vme | clct status en | alct status en | ccb status oe | int l1aen | disabl tx | ignore rx |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | ccb_ignore_rx | 0 | 1=Ignore Received CCB backplane inputs |
| [01] | RW | ccb_disable_tx | 0 | 1=Disble transmitted CCB backplane outputs |
| [02] | RW | ccb_int_l1a_en | 0 | 1=Enable internal L1A emulator |
| [03] | RW | ccb_status_oe_vme | 0 | 1=Enable ALCT+CLCT status to CCB front panel |
| [04] | RW | alct_status_en | 0 | 1=Enable ALCT status GTL outputs (req [03]=1) |
| [05] | RW | clct_status_en | 0 | 1=Enable CLCT status GTL outputs (req [03]=1) |
| [06] | RW | l1accept_vme | 0 | 1=fire ccb_l1accept oneshot |
| [08:07] | R | tmb_reserved[1:0] |  | Future use |
| [11:09] | R | tmb_reserved_out[2:0] |  | Future use |
| [12] | R | tmb_hard_reset |  | Reload TMB FPGA |
| [13] | R | alct_hard_reset |  | Reload ALCT FPGA |
| [14] | R | alct_adb_pulse_sync |  | ALCT synchronous test pulse from CCB |
| [15] | R | alct_adb_pulse_async |  | ALCT asynchronous test pulse from CCB |
| [12] | W | vme_evcntres | 0 | Event counter reset ¦¦ ccb_evcntres |
| [13] | W | vme_bcntres | 0 | Bunch crossing reset ¦¦ ccb_bcntres |
| [14] | W | vme_bx0 | 0 | Bx0 signal ¦¦ ccb_bx0 |
| [15] | W | vme_bx0_emu_en | 0 | Bx0 Emulator enable |

#### Adr 2C	ADR_CCB_TRIG	CCB Trigger Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l1a delay vme7 | l1a delay vme6 | l1a delay vme5 | l1a delay vme4 | l1a delay vme3 | l1a delay vme2 | l1a delay vme1 | l1a delay vme0 | Ignore start/ stop | ccb exttrig bypas | ext trig both | clct ext trg vme | alct ext trg vme | seq trig l1aen | clct ext trg l1aen | alct ext trg l1aen |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | alct_ext_trig_l1aen | 0 | 1=Request ccb l1a on alct ext_trig |
| [01] | RW | clct_ext_trig_l1aen | 0 | 1=Request ccb l1a on clct ext_trig |
| [02] | RW | seq_trig_l1aen | 1 | 1=Request ccb l1a on sequencer trigger |
| [03] | RW | alct_ext_trig_vme | 0 | 1=Fire alct_ext_trig oneshot |
| [04] | RW | clct_ext_trig_vme | 0 | 1=Fire clct_ext_trig oneshot |
| [05] | RW | ext_trig_both | 0 | 1=clct_ext_trig fires alct + alct fires clct_trig, DC |
| [06] | RW | ccb_allow_extbypass | 0 | 1=Allow clct_exttrig_ccb when ccb_ignore_rx=1 |
| [07] | RW | ccb_ignore_startstop | 0 | 1=Ignore ttc_trig_start, ttc_trig_stop |
| [15:08] | RW | l1a_delay_vme | 7216 | Internal L1A delay (not same as sequencer L1A) |

#### Adr 2E	ADR_CCB_STAT0	CCB Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ccb bx0 | ccb bcntrs | ccb res4 | ccb res3 | ccb res2 | ccb qpll locked | ccb ttcrx ready | ccb clock en | ccb cmd7 | ccb cmd6 | ccb cmd5 | ccb cmd4 | ccb cmd3 | ccb cmd2 | ccb cmd1 | ccb cmd0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | ccb_cmd[7:0] |  | CCB Command word from TTC |
| [08] | R | ccb_clock40_enable | 1 | 1=TMB 40MHz clock from CCB enabled |
| [09] | R | ccb_reserved[0] | 1 | ccb_ttcrx_ready TTC ready signal from CCB |
| [10] | R | ccb_reserved[1] | 1 | ccb_qpll_locked PLL locked signal from CCB |
| [13:09] | R | ccb_reserved[4:2] |  | Future use |
| [14] | R | ccb_bcntres |  | Bunch counter reset from CCB (backplane) |
| [15] | R | ccb_bx0 |  | Bunch crossing 0 from CCB (backplane) |

#### Adr 30	ADR_ALCT_CFG	ALCT Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | cfeb muon ic | alct muon ic | alct clk vme | alct clk ccb | alct seq cmd3 | alct seq cmd2 | alct seq cmd1 | alct seq cmd0 | assert alct ext inj | assert alct ext trg | alct ext inj en | alct ext trg en |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | cfg_alct_ext_trig_en | 1 | 1=Enable alct_ext_trig from CCB |
| [01] | RW | cfg_alct_ext_inject_en | 0 | 1=Enable alct_ext_inject from CCB |
| [02] | RW | cfg_alct_ext_trig | 0 | 1=Assert alct_ext_trig |
| [03] | RW | cfg_alct_ext_inject | 0 | 1=Assert alct_ext_inject |
| [07:04] | RW | alct_seq_cmd[3:0] | 0 | ALCT Sequencer command |
| [08] | RW | alct_clock_en_use_ccb |  | 1=alct_clock_en_vme = ccb_clock40_enable |
| [09] | RW | alct_clock_en_use_vme |  | sets alct_clock_en cable signal if [8]=0 |
| [10] | R | alct_muonic | 1 | ALCT board has independent time-of-flight delay |
| [11] | R | cfeb_muonic | 0 | CFEBs have independent time-of-flight delay |
| [15:12] | RW | -- | 0 | Unassigned |

#### Adr 32	ADR_ALCT_INJ	ALCT Injector Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | inject delay 4 | inject delay 3 | inject delay 2 | inject delay 1 | inject delay 0 | l1a inj ram | alct inj ram | link inject w clct | start inject | clear alct |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | alct_clear | 0 | 1=Blank ALCT received data |
| [01] | RW | alct_inject_mux | 0 | 1=Start ALCT injector State Machine |
| [02] | RW | alct_sync_clct | 0 | 1=Link ALCT injector with CLCT inject command |
| [03] | RW | alct_inj_ram_en | 0 | 1=Link ALCT injector to CFEB injector RAM |
| [04] | RW | l1a_inj_ram_en | 0 | 1=Link L1A injector to CFEB injector RAM |
| [09:05] | RW | alct_inj_delay[4:0] | 13 | Injector delay |
| [15:10] | RW | -- | 0 | Unassigned |

#### Adr 34	ADR_ALCT0_INJ	ALCT0 1st Muon To Inject

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1st bxn 1 | 1st bxn 0 | 1st key 6 | 1st key 5 | 1st key 4 | 1st key 3 | 1st key 2 | 1st key 1 | 1st key 0 | 1st amu | 1st qualty 1 | 1st qualty 0 | 1st vpf |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | alct_first_valid | 1 | Valid pattern flag |
| [02:01] | RW | alct_first_quality[1:0] | 3 | Pattern quality |
| [03] | RW | alct_first_amu | 0 | Accelerator muon flag |
| [10:04] | RW | alct_first_key[6:0] | 7 | Injected ALCT0 key wire-group |
| [12:11] | RW | alct_first_bxn[1:0] | 1 | Injected ALCT0 bunch crossing number |
| [15:13] | RW | -- | 0 | Unassigned |

#### Adr 36	ADR_ALCT1_INJ	ALCT1 2nd Muon To Inject

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 2nd bxn 1 | 2nd bxn 0 | 2nd key 6 | 2nd key 5 | 2nd key 4 | 2nd key 3 | 2nd key 2 | 2nd key 1 | 2nd key 0 | 2nd amu | 2nd qualty 1 | 2nd qualty 0 | 2nd vpf |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | alct_second_valid | 1 | Valid pattern flag |
| [02:01] | RW | alct_second_quality[1:0] | 2 | Pattern quality |
| [03] | RW | alct_second_amu | 0 | Accelerator muon flag |
| [10:04] | RW | alct_second_key[6:0] | 6110 | Injected ALCT1 key wire-group |
| [12:11] | RW | alct_second_bxn[1:0] | 1 | Injected ALCT1 bunch crossing number |
| [15:13] | RW | -- | 0 | Unassigned |

#### Adr 38	ADR_ALCT_STAT	ALCT Sequencer Control/Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alct txdint delay 3 | alct txdint delay 2 | alct txdint delay 1 | alct txdint delay 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | alct sync eccerr 1 | alct sync eccerr 0 | alct ecc blank | alct ecc en | alct cfg done |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | R | alct_cfg_done | 1 | ALCT FPGA loaded from PROM |
| [01] | RW | alct_ecc_en | 1 | ALCT ECC trigger data correction enable |
| [02] | RW | alct_ecc_err_blank | 1 | Blank alcts with uncorrected ecc errors |
| [04:03] | R | alct_sync_ecc_err[1:0] | 0 | ALCT sync-mode ECC error code |
| [11:05] | RW | -- | 0 | Unassigned |
| [15:12] | RW | alct_txd_int_delay[3:0] | 0 | Delay data transmitted to ALCT by integer bx |

#### Adr 3A	ADR_ALCT0_RCD	ALCT 1st Muon Received by TMB

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1st bxn 1 | 1st bxn 0 | 1st key 6 | 1st key 5 | 1st key 4 | 1st key 3 | 1st key 2 | 1st key 1 | 1st key 0 | 1st amu | 1st qualty 1 | 1st qualty 0 | 1st vpf |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | alct_first_valid | 1 | Valid pattern flag |
| [02:01] | R | alct_first_quality[1:0] | 0-3 | Pattern quality |
| [03] | R | alct_first_amu | 0 | Accelerator muon flag |
| [10:04] | R | alct_first_key[6:0] | 0-111 | ALCT0 key wire-group |
| [12:11] | R | alct_first_bxn[1:0] | 0-3 | ALCT0 bunch crossing number |
| [15:13] | R | -- | 0 | Unassigned |

#### Adr 3C	ADR_ALCT1_RCD	ALCT 2nd Muon Received by TMB

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 2nd bxn 1 | 2nd bxn 0 | 2nd key 6 | 2nd key 5 | 2nd key 4 | 2nd key 3 | 2nd key 2 | 2nd key 1 | 2nd key 0 | 2nd amu | 2nd qualty 1 | 2nd qualty 0 | 2nd vpf |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | alct_second_valid | 1 | Valid pattern flag |
| [02:01] | R | alct_second_quality[1:0] | 0-3 | Pattern quality |
| [03] | R | alct_second_amu | 0 | Accelerator muon flag |
| [10:04] | R | alct_second_key[6:0] | 0-111 | ALCT1 key wire-group |
| [12:11] | R | alct_second_bxn[1:0] | 0-3 | ALCT1 bunch crossing number |
| [15:13] | R | -- | 0 | Unassigned |

#### Adr 3E	ADR_ALCT_FIFO	ALCT FIFO RAM Status

(Split with Adr A2 ADR_ALCT_FIFO1 and A4 ADR_ALCT_FIFO2)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | alct data17 | alct data16 | alct wdcnt 10 | alct wdcnt 9 | alct wdcnt 8 | alct wdcnt 7 | alct wdcnt 6 | alct wdcnt 5 | alct wdcnt 4 | alct wdcnt 3 | alct wdcnt 2 | alct wdcnt 1 | alct wdcnt 0 | alct RAM done | alct RAM busy |

| Bits | Dir | Signal |  | Description |
| --- | --- | --- | --- | --- |
| [00] | R | alct_raw_busy |  | ALCT raw hits FIFO busy writing ALCT data |
| [01] | R | alct_raw_done |  | ALCT raw hits ready for VME readout |
| [12:02] | R | alct_raw_wdcnt[10:0] |  | ALCT raw hits word count stored in RAM |
| [14:13] | R | alct_raw_rdata[17:16] |  | ALCT raw hits data MSBs |
| [15] | R | -- | 0 | Unassigned |

#### Adr 40	ADR_DMB_MON	DMB Monitored Signals

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 2nd bxn 1 | 2nd bxn 0 | 2nd key 6 | 2nd key 5 | 2nd key 4 | 2nd key 3 | 2nd key 2 | 2nd key 1 | 2nd key 0 | 2nd amu | 2nd qualty 1 | 2nd qualty 0 | 2nd vpf |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [02:00] | R | dmb_cfeb_calibrate[2:0] | 0 | DMB calibration |
| [03] | R | dmb_l1a_release | 0 | DMB test |
| [08:04] | R | dmb_reserved_out[4:0] | 0 | DMB future use |
| [11:09] | R | dmb_reserved_in[2:0] | 0 | DMB future use |
| [15:12] | R | dmb_rx_ff[3:0] | 0 | DMB received |

#### Adr 42	ADR_CFEB_INJ	CFEB Injector Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj start | inj mask 4 | inj mask 3 | inj mask 2 | inj mask 1 | inj mask 0 | inj febsel 4 | inj febsel 3 | inj febsel 2 | inj febsel 1 | inj febsel 0 | mask all cfeb4 | mask all cfeb3 | mask all cfeb2 | mask all cfeb1 | mask all cfeb0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [04:00] | RW | mask_all[4:0] | 111112 | 1=Enable, 0=Turn off CFEBn inputs See Adr68 p44 |
| [09:05] | RW | inj_febsel[4:0] | 0 | 1=Select CFEBn for RAM read/write |
| [14:10] | RW | injector_mask[4:0] | 111112 | Enable CFEBn for injector trigger |
| [15] | RW | inj_trig_vme | 0 | Start pattern injector |

#### Adr 44	ADR_CFEB_INJ_ADR	CFEB Injector RAM Address

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj adr 9 | inj adr 8 | inj adr 7 | inj adr 6 | inj adr 5 | inj adr 4 | inj adr 3 | inj adr 2 | inj adr 1 | inj adr 0 | inj ren 2 | inj ren 1 | inj ren 0 | inj wen 2 | inj wen 1 | inj wen 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [02:00] | RW | inj_wen[2:0] | 0 | 1=Write enable injector RAMn (Ly01,23,45) |
| [05:03] | RW | inj_ren[2:0] | 0 | 1=Read enable Injector RAMn |
| [15:06] | RW | inj_rwadr[9:0] | 0 | Injector RAM read/write address |

#### Adr 46	ADR_CFEB_INJ_WDATA	CFEB Injector Write Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj wdata 15 | inj wdata 14 | inj wdata 13 | inj wdata 12 | inj wdata 11 | inj wdata 10 | inj wdata 9 | inj wdata 8 | inj wdata 7 | inj wdata 6 | inj wdata 5 | inj wdata 4 | inj wdata 3 | inj wdata 2 | inj wdata 1 | inj wdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | inj_wdata[7:0] | 0 | Triad bit for addressed Tbin Ly0 (or 2,4) |
| [15:08] | RW | inj_wdata[15:8] | 0 | Triad bit for addressed Tbin Ly1 (or 3,5) |

#### Adr 48	ADR_CFEB_INJ_RDATA	CFEB Injector Read Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj rdata 15 | inj rdata 14 | inj rdata 13 | inj rdata 12 | inj rdata 11 | inj rdata 10 | inj rdata 9 | inj rdata 8 | inj rdata 7 | inj rdata 6 | inj rdata 5 | inj rdata 4 | inj rdata 3 | inj rdata 2 | inj rdata 1 | inj rdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | inj_rdata[7:0] | 0 | Triad bit for addressed Tbin Ly0 (or 2,4) |
| [15:08] | R | inj_rdata[15:8] | 0 | Triad bit for addressed Tbin Ly1 (or 3,5) |

#### Adr 4A	ADR_HCM001	CFEB0 Ly0,Ly1 Hot Channel Mask

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | cfeb0_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| [15:08] | RW | cfeb0_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 4C	ADR_HCM023	CFEB0 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb0_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb0_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 4E	ADR_HCM045	CFEB0 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb0_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb0_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 50	ADR_HCM101	CFEB1 Ly0,Ly1 Hot Channel Mask

| [07:00] | RW | cfeb1_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb1_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 52	ADR_HCM123	CFEB1 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb1_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb1_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 54	ADR_HCM145	CFEB1 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb1_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb1_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 56	ADR_HCM201	CFEB2 Ly0,Ly1 Hot Channel Mask

| [07:00] | RW | cfeb2_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb2_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 58	ADR_HCM223	CFEB2 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb2_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb2_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 5A	ADR_HCM245	CFEB2 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb2_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb2_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 5C	ADR_HCM301	CFEB3 Ly0,Ly1 Hot Channel Mask

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | cfeb3_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| [15:08] | RW | cfeb3_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 5E	ADR_HCM323	CFEB3 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb3_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb3_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 60	ADR_HCM345	CFEB3 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb3_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb3_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 62	ADR_HCM401	CFEB4 Ly0,Ly1 Hot Channel Mask

| [07:00] | RW | cfeb4_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb4_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 64	ADR_HCM423	CFEB4 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb4_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb4_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 66	ADR_HCM445	CFEB4 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb4_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb4_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 68	ADR_SEQ_TRIG_EN	Sequencer Trigger Source Enables **

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeben source | cfeben 4 | cfeben 3 | cfeben 2 | cfeben 1 | cfeben 0 | all cfebs active | ext trig inject | vme trig | alct ext trig en | clct ext trig en | adb ext trig en | adb ext trig en | alct*clct match trig en | alct pat trig en | clct pat trig en |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | clct_pat_trig_en | 1 | 1=Allow CLCT pattern triggers (CLCT Active FEB) |
| [01] | RW | alct_pat_trig_en | 0 | 1=Allow ALCT pattern triggers (ALCT Active FEB) |
| [02] | RW | alct_match_trig_en | 0 | 1=ALCT*CLCT pattern triggers |
| [03] | RW | adb_ext_trig_en | 0 | 1=Allow ADB external triggers from CCB |
| [04] | RW | dmb_ext_trig_en | 0 | 1=Allow DMB external triggers |
| [05] | RW | clct_ext_trig_en | 0 | 1=Allow CLCT external triggers (scintillator) from CCB |
| [06] | RW | alct_ext_trig_en | 0 | 1=Allow ALCT external triggers from CCB |
| [07] | RW | vme_ext_trig | 0 | 1=Initiate Sequencer trigger (write 0 to recover) |
| [08] | RW | ext_trig_inject | 0 | 1=Change clct_ext_trig to fire pattern injector |
| [09] | RW | all_cfebs_active | 0 | 1=Make all CFEBs active when triggered |
| [14:10] | RW* | cfeb_en | 111112 | 1=Enable CFEB[n] to trigger and send active_feb_flag |
| [15] | RW | cfeb_en_source | 1 | 1=cfeb_en set by mask_all[4:0] in Adr 42, 0=set by 68 |

* normally, cfeb_en is copied from mask_all in Adr42 so that masked-off cfebs do not trigger TMB or send active feb to DMB.  That prevents the CFEB pattern injector from triggering, so setting_cfeb_en_source=0, allows cfeb_en to be written independently via Adr68[14:10].

** See adr F0 p70 for layer-trigger mode

#### Adr 6A	ADR_SEQ_TRIG_DLY0	Sequencer Trigger Source Delays

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| adb delay 3 | adb delay 2 | adb delay 1 | adb delay 0 | alct aff delay 3 | alct aff delay 2 | alct aff delay 1 | alct aff delay 0 | alct pretrig delay | alct pretrig delay 2 | alct pretrig delay 1 | alct pretrig delay 0 | alct width 3 | alct width 2 | alct width 1 | alct width 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | alct_trig_width[3:0] | 3 | ALCTCLCT Pre-trigger window width |
| [07:04] | RW | alct_pre_trig_dly | 0 (2) | ALCT Pre-trigger delay for ALCT*CLCT |
| [11:08] | RW | alct_pat_trig_dly[3:0] | 0 | Delay alct_pat_trig (active feb flag from ALCT) |
| [15:12] | RW | adb_ext_trig_dly[3:0] | 1 | Delay adb_ext_trig from CCB |

#### Adr 6C	ADR_SEQ_TRIG_DLY1	Sequencer Trigger Source Delays

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | alct ext delay 3 | alct ext delay 2 | alct ext delay 1 | alct ext delay 0 | clct ext delay 3 | clct ext delay 2 | clct ext delay 1 | clct ext delay 0 | dmb ext delay 3 | dmb ext delay 2 | dmb ext delay 1 | dmb ext delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | dmb_ext_trig_dly[3:0] | 1 | Delay dmb_ext_trig from DMB |
| [07:04] | RW | clct_ext_trig_dly[3:0] | 7 | Delay clct_ext_trig (scintillator) from CCB |
| [11:08] | RW | alct_ext_trig_dly[3:0] | 7 | Delay alct_ext_trig from CCB |
| [15:12] | RW | - | 0 | Unused |

#### Adr 6E	ADR_SEQ_ID	Sequencer Board + CSC Ids

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | run id 3 | run id 2 | run id 1 | run id 0 | csd id 3 | csd id 2 | csd id 1 | csd id 0 | board id 4 | board id 3 | board id 2 | board id 1 | board id 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [04:00] | RW | board_id[4:0] | 21 | Board ID = VME Slot Geographic Adr |
| [08:05] | RW | csc_id[3:0] | 5 | CSC Chamber ID number |
| [12:09] | RW | run_id[3:0] | 0 | Run ID number |
| [15:13] | RW | -- | 0 | Unassigned |

#### Adr 70	ADR_SEQ_CLCT	Sequencer CLCT Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pretrig halt | drft delay 0 | drft delay 0 | hit thresh post 2 | hit thresh post 1 | hit thresh post 0 | dmb thresh 2 | dmb thresh 1 | dmb thresh 0 | hs thresh 2 | hs thresh 1 | hs thresh 0 | triad persist 3 | triad persist 2 | triad persist 1 | triad persist 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | triad_persist | 6 | Triad One-Shot Persistence (6=150ns) |
| [06:04] | RW | hit_thresh_pretrig[2:0] | 4 | Pattern hits pre-trigger threshold |
| [09:07] | RW | dmb_thresh_pretrig[2:0] | 4 | Minimum pattern hits 0-6 for DMB active-febs |
| [12:10] | RW | hit_thresh_postdrift[2:0] | 4 | Minimum pattern hits allowed after drift |
| [14:13] | RW | drift_delay[1:0] | 2 | CSC Drift delay, number 25ns clock periods |
| [15] | RW | pretrig_halt | 0 | Pretrigger and halt until unhalt arrives |

#### Adr 72	ADR_SEQ_FIFO	Sequencer FIFO Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bcb read enable | 0 | fifo no raw hits | fifo pretrig 4 | fifo pretrig 3 | fifo pretrig 2 | fifo pretrig 1 | fifo pretrig 0 | fifo tbins 4 | fifo tbins 3 | fifo tbins 2 | fifo tbins 1 | fifo tbins 0 | fifo mode 2 | fifo mode 1 | fifo mode 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [02:00] | RW | fifo_mode[2:0] | 1 | FIFO Mode: 0=no CFEB raw hits full header 1=all CFEB raw hits full header 2=local CFEB raw hits full header 3=no CFEB raw hits short header 4=no CFEB raw hits no header |
| [07:03] | RW | fifo_tbins[4:0] | 7 | Number FIFO time bins to read out |
| [12:08] | RW | fifo_pretrig[4:0] | 2 | Number FIFO time bins before pretrigger |
| [13] | RW | fifo_no_raw_hits | 0 | 1=do not wait to store raw hits [a no_daq mode] |
| [14] | RW | -- | 0 | Unassigned |
| [15] | RW | bcb_read_enable | 0 | 1=enable cfeb blocked distrip bits in dmb readout |

#### Adr 74	ADR_SEQ_L1A	Sequencer L1A Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l1a intern dly 2 | l1a intern dly 1 | l1a intern dly 0 | l1a intern | l1a windo 3 | l1a windo 2 | l1a windo 1 | l1a windo 0 | l1a delay 7 | l1a delay 6 | l1a delay 5 | l1a delay 4 | l1a delay 3 | l1a delay 2 | l1a delay 1 | l1a delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | l1a_delay[7:0] | 12810 | Level1 Accept delay from pretrig status output |
| [11:08] | RW | l1a_window[3:0] | 3 | Level1 Accept window width after delay |
| [12] | RW | l1a_internal | 0 | Generate internal Level 1, overrides external |
| [15:13] | RW | l1a_internal_dly[2:0] | 0 | Window position for internal L1A |

#### Adr 76	ADR_SEQ_OFFSET0	Sequencer Counter Offsets [see Adr10A p75 for L1A bxn offset]

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bxn offset 11 | bxn offset 10 | bxn offset 9 | bxn offset 8 | bxn offset 7 | bxn offset 6 | bxn offset 5 | bxn offset 4 | bxn offset 3 | bxn offset 2 | bxn offset 1 | bxn offset 0 | l1a offset 3 | l1a offset 2 | l1a offset 1 | l1a offset 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | l1a_offset[3:0] | 0 | L1A counter preset value |
| [15:04] | RW | bxn_offset_pretrig[11:0] | 0 | BXN offset at reset for pretrigger bxn |

#### Adr 78	ADR_SEQ_CLCT0	Sequencer Latched CLCT0 (LSBs)

(Split with Adr B0 ADR_SEQCLCTM)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hs key 7 | hs key 6 | hs key 5 | hs key 4 | hst key 3 | hs key 2 | hs key 1 | hs key 0 | hs pid 3 | hs pid 2 | hs pid 1 | hs pid 0 | hs hits 2 | hs hits 1 | hs hits 0 | vpf |

| Bits | Dir | Signal | Typical |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| [00] | R | clct0[0] clct_1st_valid |  | 1 |  | Valid pattern flag |
| [03:01] | R | clct0[3:1] hs_hit_1st[2:0] |  | 4-6 |  | Hits on pattern: 0 to 6 |
| [07:04] | R | clct0[7:4] hs_pid_1st[3:0] |  | 0-10 |  | Pattern shape 0 to 10 |
| [15:08] | R | clct0[15:8] hs_key_1st[7:0] |  | 0-15910 |  | Key ½-strip |

#### Adr 7A	ADR_SEQ_CLCT1	Sequencer Latched CLCT1 (LSBs)

(Split with Adr B0 ADR_SEQCLCTM)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hs key 7 | hs key 6 | hs key 5 | hs key 4 | hst key 3 | hs key 2 | hs key 1 | hs key 0 | hs pid 3 | hs pid 2 | hs pid 1 | hs pid 0 | hs hits 2 | hs hits 1 | hs hits 0 | vpf |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | clct1[0] clct_2nd_valid | 1 | Valid pattern flag |
| [03:01] | R | clct1[3:1] hs_hit_2nd[2:0] | 4-6 | Hits on pattern: 0 to 6 |
| [07:04] | R | clct1[7:4] hs_pid_2nd[3:0] | 0-10 | Pattern shape 0 to 10 |
| [15:08] | R | clct1[15:8] hs_key_2nd[7:0] | 0-15910 | Key ½-strip |

#### Adr 7C	ADR_SEQ_TRIG_SRC	Sequencer Trigger Source Read-back

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | me1b pretrig | me1a pre trig | layer trig | vme trig | alct ext trig en | clct ext trig en | adb ext trig en | adb ext trig en | alct*clct pat trig en | alct pat trig en | clct pat trig en |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | clct_pat_trig_en | 1 | CLCT pattern triggered sequencer |
| [01] | R | alct_pat_trig_en | 0 | ALCT pattern triggered sequencer |
| [02] | R | alct_match_trig_en | 0 | ALCT*CLCT pattern triggered sequencer |
| [03] | R | adb_ext_trig_en | 0 | ADB external triggered sequencer |
| [04] | R | dmb_ext_trig_en | 0 | DMB external triggered sequencer |
| [05] | R | clct_ext_trig_en | 0 | CLCT (CCB scintillator) external triggered sequencer |
| [06] | R | alct_ext_trig_en | 0 | ALCT (CCB) external triggered sequencer |
| [07] | R | vme_ext_trig | 0 | VME triggered sequencer |
| [08] | R | layer_trig | 0 | Layer trigger |
| [09] | R | me1a_only_pretrig | 0 | CLCT pattern trigger was on ME1A only |
| [10] | R | me1b_only_pretrig | 0 | CLCT pattern trigger was on ME1B only |
| [13:11] | R | -- | 0 | Unassigned |

#### Adr 7E	ADR_DMB_RAM_ADR	Sequencer RAM Address

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dmb wdata 17 | dmb wdata 16 | dmb reset | dmb wr | dmb adr 11 | dmb adr 10 | dmb adr 9 | dmb adr 8 | dmb adr 7 | dmb adr 6 | dmb adr 5 | dmb adr 4 | dmb adr 3 | dmb adr 2 | dmb adr 1 | dmb adr 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [11:00] | RW | dmb_adr[11:0] | 0 | Raw hits RAM VME read/write address |
| [12] | RW | dmb_wr | 0 | Raw hits RAM VME write enable |
| [13] | RW | dmb_reset | 0 | Raw hits RAM VME address reset |
| [15:14] | RW | dmb_wdata[17:16] | 0 | Raw hits RAM VME write data MSBs |

#### Adr 80	ADR_DMB_RAM_WDATA	Sequencer RAM Write Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dmb wdata 15 | dmb wdata 14 | dmb wdata 13 | dmb wdata 12 | dmb wdata 11 | dmb wdata 10 | dmb wdata 9 | dmb wdata 8 | dmb wdata 7 | dmb wdata 6 | dmb wdata 5 | dmb wdata 4 | dmb wdata 3 | dmb wdata 2 | dmb wdata 1 | dmb wdata 16 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | dmb_wdata[15:0] | 0 | Raw hits RAM VME write data (msb in adr 76) |

#### Adr 82	ADR_DMB_RAM_WDCNT	Sequencer RAM Word Count

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | dmb busy | dmb rdata 17 | dmb wdata 16 | dmb wdcnt 11 | dmb wdcnt 10 | dmb wdcnt 9 | dmb wdcnt 8 | dmb wdcnt 7 | dmb wdcnt 6 | dmb wdcnt 5 | dmb wdcnt 4 | dmb wdcnt 3 | dmb wdcnt 2 | dmb wdcnt 1 | dmb wdcnt 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [11:00] | R | dmb_wdcnt[11: 0] | 0 | Raw hits RAM VME word count |
| [13:12] | R | dmb_rdata[17:16]; | 0 | Raw hits RAM VME read data MSBs |
| [14] | R | dmb_busy | 0 | Raw hits RAM VME |
| [15] | R | -- | 0 | Unassigned |

#### Adr 84	ADR_DMB_RAM_RDATA	Sequencer RAM Read Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dmb rdata 15 | dmb rdata 14 | dmb rdata 13 | dmb rdata 12 | dmb rdata 11 | dmb rdata 10 | dmb rdata 9 | dmb rdata 8 | dmb rdata 7 | dmb rdata 6 | dmb rdata 5 | dmb rdata 4 | dmb rdata 3 | dmb rdata 2 | dmb rdata 1 | dmb rdata 16 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | dmb_rdata[15:0] | 0 | Raw hits RAM VME read data (msb in adr 7A) |

#### Adr 86	ADR_TMB_TRIG	TMB Trigger Configuration / MPC Accept

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mpc oe | mpc idle blank | mpc sel_ttc bx0 | mpc reserved 1 | mpc reserved 0 | mpc accept 1 | mpc accept 0 | mpc delay 3 | mpc delay 2 | mpc delay 1 | mpc delay 0 | allow clct+alct match | allow clct only | allow alct only | sync err en 1 | sync err en 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [01:00] | RW | tmb_sync_err_en[1:0] | 112 | Allow sync_err to MPC for either muon |
| [02] | RW | tmb_allow_alct | 0 | Allow ALCT-only L1A (not used in current version) |
| [03] | RW | tmb_allow_clct | 1 | Allow CLCT0-only L1A |
| [04] | RW | tmb_allow_match | 1 | Allow ALCT+CLCT match pre-trigger |
| [08:05] | RW | mpc_rx_delay[3:0] | 7 | MPC accept response delay |
| [10:09] | R | mpc_accept[1:0] | - | MPC accept latched after delay |
| [12:11] | R | mpc_reserved[1:0] | - | MPC reserved latched after delay |
| [13] | RW | mpc_sel_ttc_bx0 | 1 | 1=MPC gets ttc_bx0, 0=bx0_local |
| [14] | RW | mpc_idle_blank | 0 | 1=blank mpc data & bx0 except when triggered |
| [15] | RW | mpc_oe | 1 | 1=enable outputs to MPC, 0=aset FFs to 1s |

#### Adr 88	ADR_MPC0_FRAME0	MPC0 Frame0 Data Sent to MPC

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st vpf | lct 1st q 3 | lct 1st q 2 | lct 1st q 1 | lct 1st q 0 | clct 1st pat 3 | clct 1st pat 2 | clct 1st pat 1 | clct 1st pat 0 | alct 1st wg 6 | alct 1st wg 5 | alct 1st wg 4 | alct 1st wg 3 | alct 1st wg 2 | alct 1st wg 1 | alct 1st wg 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [06:00] | R | alct_first_key[6:0] | 0-11110 | ALCT first key wire-group |
| [10:07] | R | clct_first_pat[3:0] | 0-10 | CLCT first pattern number |
| [14:11] | R | lct_first_quality[3:0] | 8 | LCT first muon quality |
| [15] | R | first_vpf | 1 | First valid pattern flag |

#### Adr 8A	ADR_MPC0_FRAME1	MPC0 Frame1 Data Sent to MPC

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| csc id 3 | csc id 2 | csc id 1 | csc id 0 | tmb bx0 local | alct 1st bxn 0 | sync err | clct 1st bend | clct 1st key 7 | clct 1st key 6 | clct 1st key 5 | clct 1st key 4 | clct 1st key 3 | clct 1st key 2 | clct 1st key 1 | clct 1st key 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | clct_first_key[7:0] | 0-15910 | CLCT first muon key ½-strip |
| [08] | R | clct_first_bend | 0 | CLCT first muon bend direction |
| [09] | R | sync_err | 0 | BXN does not match at BX0 |
| [10] | R | alct_first_bxn[0] | 0-1 | ALCT first muon bunch crossing number |
| [11] | R | clct_first_bx0_local | 0-1 | 1=TMBs bxn[11:0]==0 |
| [15:12] | R | csc_id[3:0] | 1-9 | CSC chamber ID |

#### Adr 8C	ADR_MPC1_FRAME0	MPC1 Frame0 Data Sent to MPC

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2nd vpf | lct 2nd q 3 | lct 2nd q 2 | lct 2nd q 1 | lct 2nd q 0 | clct 2nd pat 3 | clct 2nd pat 2 | clct 2nd pat 1 | clct 2nd pat 0 | alct 2nd wg 6 | alct 2nd wg 5 | alct 2nd wg 4 | alct 2nd wg 3 | alct 2nd wg 2 | alct 2nd wg 1 | alct 2nd wg 0 |

5

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [06:00] | R | alct_second_key[6:0] | 0-11110 | ALCT second key wire-group |
| [10:07] | R | clct_second_pat[3:0] | 0-10 | CLCT second pattern number |
| [14:11] | R | lct_second_quality[3:0] | 8 | LCT second muon quality |
| [15] | R | second_vpf | 1 | Second valid pattern flag |

#### Adr 8E	ADR_MPC1_FRAME1	MPC1 Frame1 Data Sent to MPC

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| csc id 3 | csc id 2 | csc id 1 | csc id 0 | tmb bx0 local | alct 2nd bxn 0 | sync err | clct 2nd bend | clct 2nd key 7 | clct 2nd key 6 | clct 2nd key 5 | clct 2nd key 4 | clct 2nd key 3 | clct 2nd key 2 | clct 2nd key 1 | clct 2nd key 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | clct_second_key[7:0] | 0-15910 | CLCT second muon key ½-strip |
| [08] | R | clct_second_bend | 0 | CLCT second muon bend direction |
| [09] | R | sync_err | 0 | BXN does not match at BX0 |
| [10] | R | alct_second_bxn[0] | 0-1 | ALCT second muon bunch crossing number |
| [11] | R | clct_second_bx0_local | 0-1 | 1=TMBs bxn[11:0]==0 |
| [15:12] | R | csc_id[3:0] | 1-9 | CSC chamber ID |

#### Adr 90	ADR_MPC_INJ	MPC Injector Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mpc inj clct bx0 | mpc inj alct bx0 | mpc reserv 1 | mpc reserv 0 | mpc accept 1 | mpc accept 0 | ttc inj enable | mpc inject | mpc nfram 7 | mpc nfram 6 | mpc nfram 5 | mpc nfram 4 | mpc nfram 3 | mpc nfram 2 | mpc nfram 1 | mpc nfram 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | mpc_nframes[7:0] | 5 | Number frames to inject |
| [08] | RW | mpc_inject | 0 | 1=Start MPC test pattern injector |
| [09] | RW | ttc_mpc_inj_en | 1 | 1=Enable injector start by TTC command |
| [11:10] | R | mpc_accept[1:0] | - | MPC accept stored at injector RAM address |
| [13:12] | R | mpc_reserved[1:0] | - | MPC reserved stored at injector RAM address |
| [14] | RW | mpc_inj_alct_bx0 | 0 | 1=Fire alct_bx0 one-shot |
| [15] | RW | mpc_inj_clct_bx0 | 0 | 1=Fire clct_bx0 one-shot |

#### Adr 92	ADR_MPC_RAM_ADR	MPC Injector RAM Address

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mpc adr 7 | mpc adr 6 | mpc adr 5 | mpc adr 4 | mpc adr 3 | mpc adr 2 | mpc adr 1 | mpc adr 0 | mpc ren 3 | mpc ren 2 | mpc ren 1 | mpc ren 0 | mpc wen 3 | mpc wen 2 | mpc wen 1 | mpc wen 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | mpc_wen[3:0] | 0 | Select RAM to write |
| [07:04] | RW | mpc_ren[3:0] | 0 | Select RAM to read |
| [15:08] | RW | mpc_adr[7:0] | 0 | Injector RAM read/write address |

#### Adr 94	ADR_MPC_RAM_WDATA	MPC Injector RAM Write Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mpc wdata 7 | mpc wdata 6 | mpc wdata 5 | mpc wdata 4 | mpc wdata 3 | mpc wdata 2 | mpc wdata 1 | mpc wdata 0 | mpc wdata 3 | mpc wdata 2 | mpc wdata 1 | mpc wdata 0 | mpc wdata 3 | mpc wdata 2 | mpc wdata 1 | mpc wdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | mpc_wdata[15:0] | 0 | MPC Injector RAM write data |

#### Adr 96	ADR_MPC_RAM_RDATA	MPC Injector RAM Read Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mpc rdata 7 | mpc rdata 6 | mpc rdata 5 | mpc rdata 4 | mpc rdata 3 | mpc rdata 2 | mpc rdata 1 | mpc rdata 0 | mpc rdata 3 | mpc rdata 2 | mpc rdata 1 | mpc rdata 0 | mpc rdata 3 | mpc rdata 2 | mpc rdata 1 | mpc rdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | mpc_rdata[15:0] | 0 | MPC Injector RAM read data |

#### Adr 98	ADR_SCP_CTRL	Scope Control [see Adr9A p52  SCP_RDATA and adr CE p62 SCP_TRIG]

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | trig done | waitin for trigger | ram sel 3 | ram sel 2 | ram sel 1 | ram sel 0 | tbins 2 | tbins 1 | tbins 0 | no write | auto | force trig | run stop | trig en |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | scp_ch_trig_en | 1 | 1=Enable channel triggers, see AdrCE p62 for ch |
| [01] | RW | scp_runstop | 0 | 1=Run, 0=Stop |
| [02] | RW | scp_force_trig | 0 | 1=Force a trigger (to trig: set 0,1,0 in 3 writes) |
| [03] | RW | scp_auto | 0 | Sequencer readout mode 1=insert in DMB data |
| [04] | RW | scp_nowrite | 0 | 1=Preserve initial RAM test pattern for debug |
| [07:05] | RW | scp_tbins[2:0] | 4 | Auto mode tbins per channel code, actual tbins/ch=(scp_tbins+1)*64, spans 64-512 |
| [11:08] | RW | scp_ram_sel[3:0] | 0 | RAM bank address 0-9 for VME readout |
| [12] | R | scp_waiting | - | Scope waiting for trigger |
| [13] | R | scp_trig_done | - | Scope triggered, ready for readout |
| [15:14] | RW | - |  | Unassigned |

#### Adr 9A	ADR_SCP_RDATA	Scope Read data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rdata 15 | rdata 14 | rdata 13 | rdata 12 | rdata 11 | rdata 10 | rdata 9 | radr8 rdata8 | radr7 rdata7 | radr6 rdata6 | radr5 rdata5 | radr4 rdata4 | radr3 rdata3 | radr2 rdata2 | radr1 rdata1 | radr0 rdata0 |

| Bits | Dir | Signal |  | Description |
| --- | --- | --- | --- | --- |
| [08:00] | W | scp_radr[8:0] |  | Scope data read address=tbin for selected RAM |
| [15:00] | R | scp_rdata[15:0] |  | See channel assignments below |

#### Scope Channel Assignments

// Pre-trigger to DMB

assign scp_ch[0]		= (clct_sm == pretrig);	// Trigger alignment marker, scope triggers on this ch usually

assign scp_ch[1]		= triad_tp[0];		// Triad test point at input to raw hits RAM

assign scp_ch[2]		= any_cfeb_hit;		// Any CFEB over threshold

assign scp_ch[3]		= active_feb_flag;	// Active feb flag to DMB

assign scp_ch[8:4]	= active_feb_list[4:0];	// Active feb list to DMB

// Pre-trigger CLCT*ALCT matching

assign scp_ch[9]		= alct_active_feb;		// ALCT active feb flag, should precede alct0_vpf

assign scp_ch[10]		= alct_pretrig_window;	// ALCT*CLCT pretrigger matching window

// Pre-trigger Processing

assign scp_ch[13:11]	= clct_sm_vec[2:0];	// Pre-trigger state machine

assign scp_ch[14]		= wr_buf_ready;		// Write buffer ready

assign scp_ch[15]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[27:16]	= bxn_counter[11:0];	// BXN counter

assign scp_ch[28]		= discard_nowrbuf;	// Event discard, no write buffer

// CLCT Pattern Finder Output

assign scp_ch[29]		= 0;

assign scp_ch[30]		= 0;

assign scp_ch[31]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[34:32]	= hs_hit_1st[2:0];		// CLCT0 number hits after drift

assign scp_ch[38:35]	= hs_pid_1st[3:0];	// CLCT0 Pattern number

assign scp_ch[46:39]	= hs_key_1st[7:0];	// CLCT0 ½-strip ID number

assign scp_ch[47]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[50:48]	= hs_hit_2nd[2:0];	// CLCT1 number hits after drift

assign scp_ch[54:51]	= hs_pid_2nd[3:0];	// CLCT1 Pattern number

assign scp_ch[62:55]	= hs_key_2nd[7:0];	// CLCT1 ½-strip ID number

assign scp_ch[63]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

// CLCT Builder

assign scp_ch[64]		= clct0_really_valid;	// CLCT0 is over threshold, not forced by an external trigger

assign scp_ch[65]		= clct0_vpf;		// CLCT0 vpf

assign scp_ch[66]		= clct1_vpf;		// CLCT1 vpf

assign scp_ch[67]		= clct_push_xtmb;	// CLCT sent to TMB matching

assign scp_ch[68]		= discard_invp;		// CLCT discarded, below threshold after drift

// TMB Matching

assign scp_ch[69]		= alct0_valid;		// ALCT0 vpf direct from 80MHz receiver, before alct_delay

assign scp_ch[70]		= alct1_valid;		// ALCT1 vpf direct from 80MHz receiver, before alct_delay

assign scp_ch[71]		= alct_vpf_tprt;		// ALCT vpf in TMB after pipe delay, unbuffered real time

assign scp_ch[72]		= clct_vpf_tprt;		// CLCT vpf in TMB

assign scp_ch[73]		= clct_window_tprt;	// CLCT matching window in TMB

assign scp_ch[77:74]	= tmb_match_win[3:0];	// Location of alct in clct window

assign scp_ch[78]		= tmb_alct_discard;	// ALCT pair was not used for LCT

assign scp_ch[79]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,15

assign scp_ch[80]		= tmb_clct_discard;	// CLCT pair was not used for LCT

// TMB Match Results

assign scp_ch[81]		= tmb_trig_pulse;		// TMB Triggered on ALCT or CLCT or both

assign scp_ch[82]		= tmb_trig_keep;		// ALCT or CLCT or both triggered, and trigger is allowed

assign scp_ch[83]		= tmb_match;		// ALCT and CLCT matched in time

assign scp_ch[84]		= tmb_alct_only;		// Only ALCT triggered

assign scp_ch[85]		= tmb_clct_only;		// Only CLCT triggered

assign scp_ch[86]		= discard_tmbreject;	// TMB discarded event

// MPC

assign scp_ch[87]		= mpc_xmit_lct0;		// MPC LCT0 sent

assign scp_ch[88]		= mpc_xmit_lct1;		// MPC LCT1 sent

assign scp_ch[89]		= mpc_response_ff;	// MPC accept is ready

assign scp_ch[91:90]	= mpc_accept_ff[1:0];	// MPC muon accept response

// L1A

assign scp_ch[92]		= l1a_pulse;		// L1A strobe from ccb or internal

assign scp_ch[93]		= l1a_window_open;	// L1A window open duh

assign scp_ch[94]		= l1a_match;		// L1A strobe match in window

assign scp_ch[95]		= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

// Buffer push at L1A

assign scp_ch[96]		= buf_push;		// Allocate write buffer space for this event

assign scp_ch[103:97]	= buf_push_adr[6:0];	// Address of write buffer to allocate

// DMB Readout

assign scp_ch[104]	= dmb_dav;		// DAV to DMB

assign scp_ch[105]	= dmb_busy;		// Readout in progress

assign scp_ch[110:106]	= read_sm_vec[4:0];	// Readout state machine

assign scp_ch[111]	= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[126:112]	= seq_wdata[14:0];	// DMB dump image, very cool

assign scp_ch[127]	= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[128]	= seq_wdata[15];		// DMB dump image, very cool

// CLCT+TMB Pipelines

assign scp_ch[132:129]	= wr_buf_adr[3:0];	// Event address counter

assign scp_ch[133]	= wr_push_xtmb;		// Buffer write strobe after drift time

assign scp_ch[137:134]	= wr_adr_xtmb[3:0];	// Buffer write address after drift time

assign scp_ch[138]	= wr_push_rtmb;		// Buffer write strobe at TMB matching time

assign scp_ch[142:139]	= wr_adr_rtmb[3:0];	// Buffer write address at TMB matching time

assign scp_ch[143]	= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

assign scp_ch[144]	= wr_push_xmpc;	// Buffer write strobe at MPC xmit to sequencer

assign scp_ch[148:145]	= wr_adr_xmpc[3:0];	// Buffer write address at MPC xmit to sequencer

assign scp_ch[149]	= wr_push_rmpc;		// Buffer write strobe at MPC received

assign scp_ch[153:150]	= wr_adr_rmpc[3:0];	// Buffer write address at MPC received

// Buffer pop at readout completion

assign scp_ch[154]	= buf_pop;		// Specified buffer is to be released

assign scp_ch[158:155]	= buf_pop_adr[3:0];	// Address of read buffer to release

assign scp_ch[159]	= (clct_sm == pretrig);	// Skip channels 15,31,47,63,79,95,111,127,143,159

#### Adr 9C	ADR_CCB_CMD	CCB TTC Command Generator (Internal)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ccb cmd 7 | ccb cmd 6 | ccb cmd 5 | ccb cmd 4 | ccb cmd 3 | ccb cmd 2 | ccb cmd 1 | ccb cmd 0 | 0 | fmm state 2 | fmm state 1 | fmm state 0 | subadr strobe | data strobe | brcst strobe | dis con ccb |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | vme_ccb_cmd_enable | 0 | 1=Disconnect CCB backplane ccb_cmd[7:0] |
| [01] | RW | vme_ccb_cmd_strobe | 0 | 1=Assert internal ccb_cmd brcst strobe |
| [02] | RW | vme_ccb_data_strobe | 0 | 1=Assert internal ccb_cmd data strobe |
| [03] | RW | vme_ccb_subaddr_strobe | 0 | 1=Assert internal ccb_cmd sub-adr strobe |
| [06:04] | R | fmm_state[2:0] | - | FMM machine states: 0: fmm_startup 1: fmm_resync 2: fmm_stop 3: fmm_wait_bx0 4 : fmm_run |
| [07] | RW | - |  | Unassigned |
| [15:08] | RW | vme_ccb_cmd[7:0] | 0 | TTC command to generate |

#### Adr 9E	ADR_BUF_STAT0	Buffer Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| buffer disp 7 | buffer disp 6 | buffer disp 5 | buffer disp 4 | buffer disp 3 | buffer disp 2 | buffer disp 1 | buffer disp 0 | buf stalled once | buf q adrerr | buf q udferr | buf q ovferr | buf q empty | buf q full | buf stalled | wr buf ready |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | wr_buf_ready | 1 | Write buffer is ready |
| [01] | R | buf_stalled | 0 | Buffer write pointer hit a fence and stalled |
| [02] | R | buf_q_full | 0 | All raw hits ram in use, ram writing must stop |
| [03] | R | buf_q_empty | 0 | No fences remain in buffer queue |
| [04] | R | buf_q_ovf_err | 0 | Tried to push new event when queue full |
| [05] | R | buf_q_udf_err | 0 | Tried to pop event when queue empty |
| [06] | R | buf_q_adr_err | 0 | Fence adr popped from queue doesn’t match expected adr |
| [07] | R | buf_stalled_once | 0 | Buffer stalled at least once since last resync |
| [15:08] | R | buf_display[7:0] | 0 | Buffer fraction in use, for in-board LED display |

#### Adr A0	ADR_BUF_STAT1	Buffer Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | buf adr 10 | buf adr 9 | buf adr 8 | buf adr 7 | buf adr 6 | buf adr 5 | buf adr 4 | buf adr 3 | buf adr 2 | buf adr 1 | buf adr 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [10:00] | R | wr_buf_adr[10:0] | - | Current address of event & header write buffer |
| [15:11] | R | - | 0 | Unassigned |

#### Adr A2	ADR_BUF_STAT2	Buffer Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | buf fence dist 10 | buf fence dist 9 | buf fence dist 8 | buf fence dist 7 | buf fence dist 6 | buf fence dist 5 | buf fence dist 4 | buf fence dist 3 | buf fence dist 2 | buf fence dist 1 | buf fence dist 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [10:00] | R | buf_fence_dist[10:0] | - | Distance to 1st fence address |
| [15:11] | R | - | 0 | Unassigned |

#### Adr A4	ADR_BUF_STAT3	Buffer Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | buf fence cntr 11 | buf fence cnt 10 | buf fence cnt 9 | buf fence cnt 8 | buf fence cnt 7 | buf fence cnt 6 | buf fence cnt 5 | buf fence cnt 4 | buf fence cnt 3 | buf fence cnt 2 | buf fence cnt 1 | buf fence cnt 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [11:00] | R | buf_fence_cnt[11:0] | - | Number of fences in fence RAM currently |
| [15:12] | R | - | 0 | Unassigned |

#### Adr A6	ADR_BUF_STAT4	Buffer Status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | buf fence peak 11 | buf fence peak 10 | buf fence peak 9 | buf fence peak 8 | buf fence peak 7 | buf fence peak 6 | buf fence peak 5 | buf fence peak 4 | buf fence peak 3 | buf fence peak 2 | buf fence peak 1 | buf fence peak 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [11:00] | R | buf_fence_cnt_peak [11:0] | - | Peak number of fences in fence RAM |
| [15:12] | R | - | 0 | Unassigned |

#### Adr A8	ADR_ALCTFIFO1	ALCT Raw Hits RAM Control

(Split with Adr 3E ADR_ALCT_FIFO0 and Adr A4 ADR_ALCT_FIFO2)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | alct demux mode | 0 | alct radr 10 | alct radr 9 | alct radr 8 | alct radr 7 | alct radr 6 | alct radr 5 | alct radr 4 | alct radr 3 | alct radr 2 | alct radr 1 | alct radr 0 | alct raw reset |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | alct_raw_reset | 0 | Reset ALCT raw hits FIFO controller |
| [11:01] | RW | alct_raw_radr[10:0] | 0 | ALCT raw hits RAM read address or demux wd |
| [12] | RW | -- | 0 | Unassigned |
| [13] | RW | alct_demux_mode | 0 | 0=alctfifo2 has RAM data, 1=fifo2=demux data |
| [15:14] | RW | -- | 0 | Unassigned |

#### Adr AA	ADR_ALCTFIFO2	ALCT Raw Hits RAM data (LSBs)

(Split with Adr 3E ADR_ALCT_FIFO0 and Adr A2 ADR_ALCT_FIFO1)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alct fifo 15 | alct fifo 14 | alct fifo 13 | alct fifo 12 | alct fifo 11 | alct fifo 10 | alct fifo 9 | alct fifo 8 | alct fifo 7 | alct fifo 6 | alct fifo 5 | alct fifo 4 | alct fifo 3 | alct fifo 2 | alct fifo 1 | alct fifo 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | alct_raw_rdata[15:0] OR alct_1st_vme[14:1] alct_1st_vme[28:15] alct_2nd_vme[14:1] alct_2nd_vme[28:15] |  | ALCT FIFO data (msbs in adr_alct_fifo) alct_raw_radr=0 and alct_demux_mode=1 alct_raw_radr=1 alct_raw_radr=2 alct_raw_radr=3 |

#### Adr AC	ADR_SEQMOD	Sequencer Trigger Modifiers

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| event clear vme | active feb src | Scint veto state | Clear scint veto | L1A alct only | L1A tmb nol1a | L1A no tmb | L1A tmb trig | valid clct requir | wr buf requir | hdr wr cont | wrbuf auto clr | flush timer 3 | flush timer 2 | flush timer 1 | flush timer 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | clct_flush_delay[3:0] | 1 | Trigger sequencer flush state timer |
| [04] | RW | wr_buf_autoclr_en | 1 | 1=Enable frozen buffer auto clear |
| [05] | RW | hdr_wr_continuous | 0 | 1=allow continuous header buffer writing for invalid triggers |
| [06] | RW | wr_buf_required | 1 | Require wr_buffer available to pre-trigger |
| [07] | RW | valid_clct_required | 1 | Require valid CLCT after drift delay |
| [08] | RW | l1a_allow_match | 1 | Readout allows tmb trig pulse in L1A window |
| [09] | RW | l1a_allow_notmb | 0 | Readout allows notmb trig pulse in L1A window |
| [10} | RW | l1a_allow_nol1a | 0 | Readout allows tmb trig pulse outside L1A wind |
| [11] | RW | l1a_allow_alct_only | 0 | Allow ALCT-only events to readout at L1A |
| [12] | RW | scint_veto_clr | 0 | Clear scintillator veto FF |
| [13] | R | scint_veto_vme | 0 | Scintillator veto FF state |
| [14] | RW | active_feb_src | 0 | Active feb flag source, 0=pretrig, 1=tmb match |
| [15] | RW | event_clear_vme | 0 | Event clear for aff,clct,mpc VME diagnostic registers |

#### Adr AE	ADR_SEQSM	Sequencer Machine State

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | buf_q adr err | buf_q ovf | buf_q empty | buf_q full | read sm 2 | read sm 4 | read sm 3 | read sm 2 | read sm 1 | read sm 0 | clct sm 2 | clct sm 1 | clct sm 0 |

| Bits | Dir | Signal |  | Description |
| --- | --- | --- | --- | --- |
| [02:00] | R | clct_sm[2:0] |  | CLCT Trigger machine state |
| [07:03] | R | read_sm[4:0] |  | Readout machine state |
| [08] | R | buf_q_full |  | All raw hits ram in use, ram writing must stop |
| [09] | R | buf_q_empty |  | No fences remain in buffer queue |
| [10] | R | buf_q_ovf_err |  | Tried to push new event when queue full |
| [11] | R | buf_q_adr_err |  | Tried to pop event when queue empty |
| [15:12] | R | -- | 0 | Unassigned |

#### Adr B0	ADR_SEQCLCTM	Sequencer CLCT (MSBs)

(Split with Adr 78 ADR_SEQCLCT0 and Adr 7A ADR_SEQCLCT1)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sync err | clock lock lost | 0 | 0 | 0 | 0 | active cfeb 6 | active cfeb 5 | active cfeb 4 | active cfeb 3 | active cfeb 2 | active cfeb 1 | active cfeb 0 | sync err | clct bxn 1 | clct bxn 0 |

| Bits | Dir | Signal | Typ | Description |
| --- | --- | --- | --- | --- |
| [01:00] | R | clctc[1:0] bxn_counter | - | Bunch crossing number at pretrigger, common to clct0/1 |
| [02] | R | clctc[2] sync_err | 0 | BX0 disagrees with BXN counter, common to clct0/1 |
| [9:3] | R | clctf[6:0] | - | Active feb list latched at TMB alct*clct matching time |
| [13:10] | R | -- | 0 | Unassigned |
| [14] | R | clock_lock_lost FF | 0 | 40MHz main clock lost lock, global_reset asserted |
| [15] | R | sync_err (direct) | 0 | Sync error: bxn counter==0 does not match bx0 |

#### Adr B2	ADR_TMBTIM	TMB Timing for ALCT*CLCT Coincidence

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | mpctx delay 3 | mpctx delay 2 | mpctx delay 1 | mpctx delay 0 | clct wind 3 | clct wind 2 | clct wind 1 | clct wind 0 | alct delay 3 | alct delay 2 | alct delay 1 | alct delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | alct_delay[3:0] | 1 | Delay ALCT for CLCT match window |
| [07:04] | RW | clct_window[3:0] | 3 | CLCT match window width |
| [11:08] | RW | mpc_tx_delay[3:0] | 0 | MPC transmit delay |
| [15:12] | RW | -- | 0 | Unassigned |

#### Adr B4	ADR_LHC_CYCLE	LHC Cycle Period, Maximum BXN Count

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | lhc cycle 11 | lhc cycle 10 | lhc cycle 9 | lhc cycle 8 | lhc cycle 7 | lhc cycle 6 | lhc cycle 5 | lhc cycle 4 | lhc cycle 3 | lhc cycle 2 | lhc cycle 1 | lhc cycle 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [11:00] | RW | lhc_cycle[11:0] | 3564 | Maximum bxn+1 3564(hDEC) for LHC 924(h39C) for beam test |
| [15:12] | RW | -- | 0 | Unassigned |

#### Adr B6	ADR_RPC_CFG	RPC Configuration

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | rpc done | read bxn 2 | read bxn 1 | read bxn 0 | read bank 1 | read bank 0 | bxn offset3 | bxn offset2 | bxn offset1 | bxn offset0 | read enable | 0 | 0 | rpc1 exists | rpc0 exists |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [01:00] | RW | rpc_exists[1:0] | 3 | Bit (n) = 1 = RPC(n) Exists |
| [03:02] | RW | -- | 0 | Unused |
| [04] | RW | rpc_read_enable | 1 | 1=Include Existing RPCs in DMB Readout |
| [08:05] | RW | rpc_bxn_offset[3:0] | 0 | RPC BXN offset |
| [10:09] | RW | rpc_bank[1:0] | 0 | RPC bank address, for reading rdata sync mode |
| [13:11] | R | rpc_rbxn[2:0] | - | RPC rdata[18:16] msbs for sync mode, adr 1E |
| [14] | R | rpc_done | 1 | RPC FPGA reports configuration done |
| [15] | RW | -- | 0 | Unassigned |

#### Adr B8	ADR_RPC_RDATA	RPC Raw Hits Sync Mode Read Data, See Adr 1E p33

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rdata 15 | rdata 14 | rdata 13 | rdata 12 | rdata 11 | rdata 10 | rdata 9 | rdata 8 | rdata 7 | rdata 6 | rdata 5 | rdata 4 | rdata 3 | rdata 2 | rdata 1 | rdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | rpc_rdata[15:0] | - | RPC RAM read data for sync mode |

#### Adr BA	ADR_RPC_RAW_DELAY	RPC Raw Hits Data Delay + RPC BXN Differences

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rpc1 bxn diff3 | rpc1 bxn diff2 | rpc1 bxn diff1 | rpc1 bxn diff0 | rpc0 bxn diff3 | rpc0 bxn diff2 | rpc0 bxn diff1 | rpc0 bxn diff0 | rpc1 delay 3 | rpc1 delay 2 | rpc1 delay 1 | rpc1 delay 0 | rpc0 delay 3 | rpc0 delay 2 | rpc0 delay 1 | rpc0 delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | rpc0_delay[3:0] | 1 | RPC0 Raw hits data delay |
| [07:04] | RW | rpc1_delay[3:0] | 1 | RPC1 Raw hits data delay |
| [11:08] | R | rpc0_bxn_diff[3:0]; | - | RPC bxn – Offset (in adr B6) |
| [15:12] | R | rpc1_bxn_diff[3:0]; | - | RPC bxn – Offset (in adr B6) |

#### Adr BC	ADR_RPC_INJ	RPC Injector Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | rpc tbins test | inj rdata 18 | inj rdata 17 | inj rdata 16 | inj wdata 18 | inj wdata 17 | inj wdata 16 | inj sel | delay rat 3 | delay rat 2 | delay rat 1 | delay rat 0 | mask rpc | mask rat | mask all |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | rpc_mask_all | 1 | 1=Enable RPC Inputs from RAT, 0=disable all |
| [01] | RW | injector_mask_rat | 0 | 1=Enable RAT for injector fire |
| [02] | RW | injector_mask_rpc | 1 | 1=Enable RPC injector RAM for injector fire |
| [06:03] | RW | inj_delay_rat[3:0] | 0 | CFEB/RPC injectors wait for RAT |
| [07] | RW | rpc_inj_sel | 0 | 1=Enable injector RAM write |
| [10:08] | RW | rpc_inj_wdata[18:16] | 0 | RPC injector write data MSBs, see adr C0 p60 |
| [13:11] | R | rpc_inj_rdata[18:16] | - | RPC injector read data MSBs, see adr C0 p60 |
| [14] | RW | rpc_tbins_test | 0 | Set write_data=address test mode |
| [15] | RW | -- | 0 | Unassigned |

#### Adr BE	ADR_RPC_INJ_ADR	RPC Injector RAM Addresses

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj adr 7 | inj adr 6 | inj adr 5 | inj adr 4 | inj adr 3 | inj adr 2 | inj adr 1 | inj adr 0 | 0 | 0 | inj ren 1 | inj ren 0 | 0 | 0 | inj wen 1 | inj wen 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [01:00] | RW | rpc_inj_wen[1:0] | 0 | 1=Write enable injector RAMn |
| [03:02] | RW | -- | 0 | Unused |
| [05:04] | RW | rpc_inj_ren[1:0] | 0 | 1=Read enable Injector RAMn |
| [15:06] | RW | inj_rwadr[9:0] | 0 | Injector RAM read/write address |

#### Adr C0	ADR_RPC_INJ_WDATA	RPC Injector Write Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj wdata 15 | inj wdata 14 | inj wdata 13 | inj wdata 12 | inj wdata 11 | inj wdata 10 | inj wdata 9 | inj wdata 8 | inj wdata 7 | inj wdata 6 | inj wdata 5 | inj wdata 4 | inj wdata 3 | inj wdata 2 | inj wdata 1 | inj wdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | rpc_inj_wdata[15:0] | 0 | RPC RAM write data LSBs (see Adr BC msbs p60) |

#### Adr C2	ADR_RPC_INJ_RDATA	RPC Injector Read Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inj rdata 15 | inj rdata 14 | inj rdata 13 | inj rdata 12 | inj rdata 11 | inj rdata 10 | inj rdata 9 | inj rdata 8 | inj rdata 7 | inj rdata 6 | inj rdata 5 | inj rdata 4 | inj rdata 3 | inj rdata 2 | inj rdata 1 | inj rdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | rpc_inj_rdata[15:0] | - | RPC RAM read data LSBs (see Adr BC msbs p60) |

#### Adr C4	ADR_RPC_TBINS	RPC FIFO Time Bins

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | rpc de couple | rpc pre 4 | rpc pre 3 | rpc pre 2 | rpc pre 1 | rpc pre 0 | rpc tbins 4 | rpc tbins 3 | rpc tbins 2 | rpc tbins 1 | rpc tbins 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [04:00] | RW | fifo_tbins_rpc[4:0] | 7 | Number RPC FIFO time bins to read out |
| [09:05] | RW | fifo_pretrig_rpc[4:0] | 2 | Number RPC FIFO time bins before pretrigger |
| [10] | RW | rpc_decouple | 0 | 1=Independent rpc tbins, 0=copy cfeb tbins |
| [15:11] | RW | -- | 0 | Unused |

#### Adr C6	ADR_RPC0_HCM	RPC0 Hot Channel Mask

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| enable pad15 | enable pad14 | enable pad13 | enable pad12 | enable pad11 | enable pad10 | enable pad9 | enable pad8 | enable pad7 | enable pad6 | enable pad5 | enable pad4 | enable pad3 | enable pad2 | enable pad1 | enable pad0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | rpc0_hcm[15:0] | FFFF | Bit(n)=1=Enable RPC Pad(n), FFFF=enable all |

#### Adr C8	ADR_RPC1_HCM	RPC1 Hot Channel Mask

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | rpc1_hcm[15:0] | FFFF | Bit(n)=1=Enable RPC Pad(n), FFFF=enable all |

#### Adr CA	ADR_BX0_DELAY	BX0 to MPC Delays

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | bx0 match | bx0 vpf test | alct bx0 enable | clct bx dly 3 | clct bx dly 2 | clct bx dly 1 | clct bx dly 0 | alct bx dly 3 | alct bx dly 2 | alct bx dly 1 | alct bx dly 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | alct_bx0_delay[3:0] | 0 | ALCT bx0 delay to mpc transmitter |
| [07:04] | RW | clct_bx0_delay[3:0] | 0 | CLCT bx0 delay to mpc transmitter |
| [08] | RW | alct_bx0_enable | 1 | 1=Enable using alct bx0, else copy clct bx0 |
| [09] | RW | bx0_vpf_test | 0 | Sets clct_bx0=lct0_vpf for bx0 alignment tests |
| [10] | R | bx0_match | 1 | 1=alct_bx0==clct_bx0, latched at clct_bx0 |
| [15:11] | RW | -- | - | Unused |

#### Adr CC	ADR_NON_TRIG_RO	Non-Triggering Event Enables + ME1A/B Reversal

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| csc type 3 | csc type 2 | csc type 1 | csc type 0 | 0 | 0 | reverse me1b | reverse me1a | reverse csc | stagger csc | csc me1ab | cnt non me1ab | mpc me1a block | allow match ro | allow clct ro | allow alct ro |

| Bits | Dir | Signal | Default Type A | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | tmb_allow_alct_ro | 0 | 1=Allow ALCT-only non-triggering readout |
| [01] | RW | tmb_allow_clct_ro | 0 | 1=Allow CLCT-only non-triggering readout |
| [02] | RW | tmb_allow_match_ro | 1 | 1=Allow ALCT*CLCT non-triggering readout |
| [03] | RW | mpc_me1a_block | 1 | Block ME1A LCTs from MPC, still queue for readout |
| [04] | RW | cnt_non_me1ab_en | 1 | Allow clct pretrig counters count non me1ab |
| [05] | R | csc_me1ab | 0 | 1= CSC is ME1A or ME1B. 0=normal CSC |
| [06] | R | stagger_hs_csc | 1 | 1=Staggered CSC, 0=non-staggered |
| [07] | R | reverse_hs_csc | 0 | 1=Reversed staggered CSC, non-me1 |
| [08] | R | reverse_hs_me1a | 0 | 1=reversed me1a hstrips |
| [09] | R | reverse_hs_me1b | 0 | 1=reversed me1b hstrips |
| [11:10] | RW | -- | 0 | Free 2 |
| [15:12] | R | csc_type[3:0] | A | Firmware compile type A=Normal CSC B=Reversed CSC C=Normal ME1B, Reversed ME1A D=Reversed ME1B, Normal ME1A |

Firmware Compile Type Codes A,B,C,D:

| CSC_type | stagger_hs_csc | reverse_hs_csc | reverse_hs_me1a | reverse_hs_me1b | csc_me1ab |
| --- | --- | --- | --- | --- | --- |
| A | 1 | 0 | 0 | 0 | 0 |
| B | 0 | 1 | 0 | 0 | 0 |
| C | 0 | 0 | 1 | 0 | 1 |
| D | 0 | 0 | 0 | 1 | 1 |

#### Adr CE	ADR_SCP_TRIG	Scope Trigger Source Channel

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | ch6 | ch5 | ch4 | ch3 | ch2 | ch1 | ch0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [06:00] | RW | trigger_ch[6:0] | 0 | ch0=trigger on sequencer pretrig |
| [14:07] | RW | -- | 0 | Unassigned |
| [15] | RW | scp_ch_overlay | 0 | 0=normal ch assignments, 1=debug assigments |

#### Adr D0	ADR_CNT_CTRL	Status Counter Control [See Adr D2 p66]

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cnt select 6 | cnt select 5 | cnt select 4 | cnt select 3 | cnt select 2 | cnt select 1 | cnt select 0 | cnt adr lsb | hdr clr resync | cnt clr resync | en alct debug | seq cnt ovf | alct cnt ovf | stop on ovf | snap shot | clear all |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | cnt_all_clear | - | Clear VME counters (also clr on ccb_evcntres) |
| [01] | RW | cnt_snapshot | - | Take snapshot of current counter state |
| [02] | RW | cnt_stop_on_ovf | 0 | Stop all counters if any overflows |
| [03] | R | cnt_any_ovf_alct | - | At least 1 alct counter overflowed |
| [04] | R | cnt_any_ovf_seq | - | At least 1 sequencer counter overflowed |
| [05] | RW | cnt_alct_debug | 1 | 1=enable alct_lct_err counter |
| [06] | RW | cnt_clear_on_resync | 0 | 1=Clear VME counters on ttc_resync |
| [07] | RW | hdr_clear_on_resync | 1 | 1=Clear header counters on ttc_resync |
| [08] | RW | cnt_adr_lsb | - | Counter half select, 0=bits[15:0], 1=bits[29:16] |
| [15:09] | RW | cnt_select[6:0] | - | Counter select address |

Counter Select Addresses (even cnt_ad_lsb=LSBs, odd cnt_adr_lsb=MSBs)

| Address10 | Counter Description | Bits | VME Clears |
| --- | --- | --- | --- |
| 00 | ALCT: alct0 vpf received | 30 | Y |
| 01 | ALCT: alct1 vpf received | 30 | Y |
| 02 | ALCT: alct data structure error | 30 | Y |
| 03 | ALCT: trigger path ECC 1-bit error corrected | 30 | Y |
| 04 | ALCT: trigger path ECC 2-bit error not corrected | 30 | Y |
| 05 | ALCT: trigger path ECC >2-bit error not corrected | 30 | Y |
| 06 | ALCT: trigger path ECC 2-bit error not corrected, ALCT data blanked | 30 | Y |
| 07 | ALCT: alct replied ECC 1-bit error corrected | 30 | Y |
| 08 | ALCT: alct replied ECC 2-bit error not corrected | 30 | Y |
| 09 | ALCT: alct replied ECC >2-bit error not corrected | 30 | Y |
| 10 | ALCT: raw hits readout | 30 | Y |
| 11 | ALCT: raw hits readout CRC error | 30 | Y |
| 12 |  | 30 | Y |
| 13 | CLCT: Pre-trigger was on any CFEB | 30 | Y |
| 14 | CLCT: Pre-trigger includes a CLCT on CFEB0 | 30 | Y |
| 15 | CLCT: Pre-trigger includes a CLCT on CFEB1 | 30 | Y |
| 16 | CLCT: Pre-trigger includes a CLCT on CFEB2 | 30 | Y |
| 17 | CLCT: Pre-trigger includes a CLCT on CFEB3 | 30 | Y |
| 18 | CLCT: Pre-trigger includes a CLCT on CFEB4 | 30 | Y |
| 19 | CLCT: Pre-trigger includes a CLCT on CFEB5 | 30 | Y |
| 20 | CLCT: Pre-trigger includes a CLCT on CFEB6 | 30 | Y |
| 21 | CLCT: Pre-trigger was on ME1A cfeb4 only | 30 | Y |
| 22 | CLCT: Pre-trigger was on ME1B cfebs0-3 only | 30 | Y |
| 23 | CLCT: Pretrig discarded, no wrbuf available, buffer stalled | 30 | Y |
| 24 | CLCT: Pretrig discarded, no alct in window | 30 |  |
| 25 | CLCT: CLCT discarded, clct0 had invalid pattern after drift | 30 | Y |
| 26 | CLCT: CLCT0 passed hit thresh but failed pid thresh after drift | 30 | Y |
| 27 | CLCT: CLCT1 passed hit thresh but failed pid thresh after drift | 30 | Y |
| 28 | CLCT: Bx pre-trigger machine waited for triads to dissipate before rearm | 30 | Y |
| 29 | CLCT: clct0 sent to TMB matching section | 30 | Y |
| 30 | CLCT: clct1 sent to TMB matching section | 30 | Y |
| 31 | TMB: TMB matching accepted a match, alct-only, or clct-only event | 30 | Y |
| 32 | TMB: CLCT*ALCT matched trigger | 30 | Y |
| 33 | TMB: ALCT-only trigger | 30 | Y |
| 34 | TMB: CLCT-only trigger | 30 | Y |
| 35 | TMB: TMB matching rejected event | 30 | Y |
| 36 | TMB: TMB matching rejected event, but queued for non-trigger readout | 30 | Y |
| 37 | TMB: TMB matching discarded an ALCT pair | 30 | Y |
| 38 | TMB: TMB matching discarded a CLCT pair | 30 | Y |
| 39 | TMB: TMB matching discarded CLCT0 from ME1A | 30 | Y |
| 40 | TMB: TMB matching discarded CLCT1 from ME1A | 30 | Y |
| 41 | TMB: Matching found no ALCT | 30 | Y |
| 42 | TMB: Matching found no CLCT | 30 | Y |
| 43 | TMB: Matching found One ALCT | 30 | Y |
| 44 | TMB: Matching found One CLCT | 30 | Y |
| 45 | TMB: Matching found Two ALCTs | 30 | Y |
| 46 | TMB: Matching found Two CLCTs | 30 | Y |
| 47 | TMB: ALCT0 copied into ALCT1 to make 2nd LCT | 30 | Y |
| 48 | TMB: CLCT0 copied into CLCT1 to make 2nd LCT | 30 | Y |
| 49 | TMB: LCT1 has higher quality than LCT0, error | 30 | Y |
| 50 | TMB: Transmitted LCT0 to MPC | 30 | Y |
| 51 | TMB: Transmitted LCT1 to MPC | 30 | Y |
| 52 | TMB: MPC accepted LCT0 | 30 | Y |
| 53 | TMB: MPC accepted LCT1 | 30 | Y |
| 54 | TMB: MPC rejected both LCT0 & LCT1 | 30 | Y |
| 55 | L1A: L1A received | 30 | Y |
| 56 | L1A: L1A received, TMB in L1A window | 30 | Y |
| 57 | L1A: L1A received, no TMB in window | 30 | Y |
| 58 | L1A: TMB triggered, no L1A in window | 30 | Y |
| 59 | L1A: TMB readouts completed | 30 | Y |
| 60 | L1A: TMB readouts lost by 1-event-per-L1A limit | 30 | Y |
| 61 | STAT: CLCT Triads skipped | 30 | Y |
| 62 | STAT: Raw hits buffer had to be reset due to ovf, error | 30 | Y |
| 63 | STAT: TTC Resyncs received | 30 | Y |
| 64 | STAT: Sync error, bxn!=offset at bx0 arrival or no bx0 at bxn==offset | 30 | Y |
| 65 | STAT: Parity error in CFEB or RPC raw hits RAM, possible SEU | 30 | Y |
|  | Event counters that follow are in the TMB header: They are cleared via TTC commands, such as event counter reset, and are not via direct VME command to Adr D0. |  |  |
| 66 | HDR: Pre-trigger counter | 30 | N |
| 67 | HDR: CLCT counter | 30 | N |
| 68 | HDR: TMB trigger counter | 30 | N |
| 69 | HDR: ALCTs received counter | 30 | N |
| 70 | HDR: L1As received from ccb counter, 12 bits | 12 | N |
| 71 | HDR: Readout counter, 12 bits | 12 | N |
| 72 | HDR: Orbit counter | 30 | N |
| 73 | ALCT: Structure error, Expected alct0[10:1]=0 when alct0_vpf=0 | 8 | Y |
| 74 | ALCT: Structure error, Expected alct1[10:1]=0 when alct1_vpf=0 | 8 | Y |
| 75 | ALCT: Structure error, Expected alct0_vpf=1 when alct1_vpf=1 | 8 | Y |
| 76 | ALCT: Structure error, Expected alct0[10:1]>0 when alct0_vpf=1 | 8 | Y |
| 77 | ALCT: Structure error, Expected alct1[10:1]>0 when alct1_vpf=1 | 8 | Y |
| 78 | ALCT: Structure error, Expected alct1!=alct0 when alct0_vpf=1 | 8 | Y |
| 79 | CCB: TTCrx lock lost | 8 | Y |
| 80 | CCB: qPLL lock lost | 8 | Y |
| 81 | GTX: Optical receiver error gtx_rx_err_count0 | 16 | N |
| 82 | GTX: Optical receiver error gtx_rx_err_count1 | 16 | N |
| 83 | GTX: Optical receiver error gtx_rx_err_count2 | 16 | N |
| 84 | GTX: Optical receiver error gtx_rx_err_count3 | 16 | N |
| 85 | GTX: Optical receiver error gtx_rx_err_count4 | 16 | N |
| 86 | GTX: Optical receiver error gtx_rx_err_count5 | 16 | N |
| 87 | GTX: Optical receiver error gtx_rx_err_count6 | 16 | N |

#### Adr D2	ADR_CNT_RDATA	Status Counter Data [See Adr D0 p62]

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rdata 15 | rdata 14 | rdata 13 | rdata 12 | rdata 11 | rdata 10 | rdata 9 | rdata 8 | rdata 7 | rdata 6 | rdata 5 | rdata 4 | rdata 3 | rdata 2 | rdata 1 | rdata 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | cnt_rdata[15:0] | - | Data for selected counter (see adr D0 p62) |

#### Adr D4	ADR_JTAGSM0	JTAG State Machine Control (reads JTAG PROM)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| throt 3 | throt 2 | throt 1 | throt 0 | dis | jsm jtag oe | jsm ok | vme ready | jsm tck ok | jsm wdcnt ok | jsm cksum ok | jsm abort | jsm busy | jsmsel/ vsm jtag auto | jsm reset | jsm start |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | jsm_start | 0 | Manual cycle start command |
| [01] | RW | jsm_sreset | 0 | Status signal reset |
| [02] | RW | jsm_sel | 0 | 1=select new JTAG format, 0=select old format |
| [03] | R | jsm_busy | 0 | State machine busy writing |
| [04] | R | jsm_aborted | 0 | State machine aborted reading PROM |
| [05] | R | jsm_cksum_ok | 1 | Check-sum matches PROM contents |
| [06] | R | jsm_wdcnt_ok | 1 | Word count matches PROM contents |
| [07] | R | jsm_tck_fpga_ok | 1 | FPGA jtag tck detected |
| [08] | R | vme_ready | 1 | TMB VME registers done loading from PROM |
| [09] | R | jsm_ok | 1 | JTAG state machine completed without errors |
| [10] | R | jsm_jtag_oe | 0 | Enable jtag drivers else tri-state |
| [11] | RW | wr_usr_jtag_dis | 0 | 1=disable write access to ADR_USR_JTAG adr10 |
| [15:12] | RW | jsm_throttle[3:0] | 0 | JTAGspeed, 0=fastest,20MHz read,10MHz TCK |

#### Adr D6	ADR_JTAGSM1	JTAG State Machine Word Count

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wdcnt 15 | wdcnt 14 | wdcnt 13 | wdcnt 12 | wdcnt 11 | wdcnt 10 | wdcnt 9 | wdcnt 98 | wdcnt 7 | wdcnt 6 | wdcnt 5 | wdcnt 4 | wdcnt 3 | wdcnt 2 | wdcnt 1 | wdcnt 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | jsm_wdcnt[15:0]; | - | JTAG PROM word-count bits [15:0] |

#### Adr D8	ADR_JTAGSM2	JTAG State Machine Checksum

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| chain ok | header ok | end ok | tck cnt OK | tck cnt 3 | tck cnt 2 | tck cnt 1 | tck cnt 0 | cksum 7 | cksum 6 | cksum 5 | cksum 4 | cksum 3 | cksum 2 | cksum 1 | cksum 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | jsm_cksum[7:0]; | - | jtag state machine checksum |
| [11:08] | R | tck_fpga_cnt[3:0] | - | fpga jtag chain tck counter |
| [12] | R | jsm_tckcnt_ok | 1 | Total TCKs sent maches PROM trailer tck_cnt |
| [13] | R | jsm_end_ok | 1 | jtag PROM end marker detected |
| [14] | R | jsm_header_ok | 1 | jtag PROM header marker detected |
| [15] | R | jsm_chain_ok | 1 | jtag PROM chain marker detected |

#### Adr DA	ADR_VMEM0	VME State Machine Control (reads VME PROM)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| throt 3 | throt 2 | throt 1 | throt 0 | vsm phaser auto | vsm path ok | vsm ok | vme ready | vsm jtag auto | vsm wdcnt ok | vsm cksum ok | vsm abort | vsm busy | vsm auto start | vsm sreset | vsm start |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | vsm_start | 0 | Manual cycle start command |
| [01] | RW | vsm_sreset | 0 | Status signal reset |
| [02] | R | vsm_autostart | 1 | Auto-start after hard-reset |
| [03] | R | vsm_busy | 0 | State machine busy writing |
| [04] | R | vsm_aborted | 0 | State machine aborted reading PROM |
| [05] | R | vsm_cksum_ok | 1 | Check-sum matches PROM contents |
| [06] | R | vsm_wdcnt_ok | 1 | Word count matches PROM contents |
| [07] | RW* | vsm_jtag_auto | 1 | JTAG SM autostart after vmesm completes |
| [08] | R | vme_ready | 1 | TMB VME registers done loading from PROM |
| [09] | R | vsm_ok | 1 | State machine completed without errors |
| [10] | R | vsm_path_ok | 1 | State machine wrote 55AAh to ADR_VMESM4 |
| [11] | RW | vsm_phaser_auto | 1 | Digital phase shifter autostart after vmesm done |
| [15:12] | RW | vsm_throttle[3:0] | 0 | VME PROM-read speed control, 0=fastest |

* vsm_jtag_auto should be set to 0 to enable U76 testing, otherwise jtagsm will run, and erase U76 data.

#### Adr DC	ADR_VMESM1	VME State Machine Word Count

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wdcnt 15 | wdcnt 14 | wdcnt 13 | wdcnt 12 | wdcnt 11 | wdcnt 10 | wdcnt 9 | wdcnt 98 | wdcnt 7 | wdcnt 6 | wdcnt 5 | wdcnt 4 | wdcnt 3 | wdcnt 2 | wdcnt 1 | wdcnt 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | vsm_wdcnt[15:0]; | - | VME PROM word-count bits [15:0] |

#### Adr DE	ADR_VMESM2	VME State Machine Checksum

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | jtag vec 1 | jtag vec 0 | fmt err 4 | fmt err 3 | fmt err 2 | fmt err 1 | fmt err 0 | cksum 7 | cksum 6 | cksum 5 | cksum 4 | cksum 3 | cksum 2 | cksum 1 | cksum 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | vsm_cksum[7:0]; | - | VME state machine checksum |
| [08] | R | vsm_fmt_err[0] | 0 | Missing BC header-begin marker |
| [09] | R | vsm_fmt_err[1] | 0 | Missing EC header-end marker |
| [10] | R | vsm_fmt_err[2] | 0 | Missing FC data-end marker |
| [11] | R | vsm_fmt_err[3] | 0 | Missing FF prom-end marker |
| [12] | R | vsm_fmt_err[4] | 0 | Word counter overflow |
| [14:13] | R | jtag_sm_vec[1:0] | 0 | JSM JTAG signal State Machine vector |
| [15] | R | -- | - | unassigned |

#### Adr E0	ADR_VMESM3	Number of VME Addresses Written by VMESM

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | format sm 2 | format sm 1 | format sm 0 | prom sm 3 | prom sm 2 | prom sm 1 | prom sm 0 | nvme writes 7 | nvme writes 6 | nvme writes 5 | nvme writes 4 | nvme writes 3 | nvme writes 2 | nvme writes 1 | nvme writes 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | vsm_nvme_writes[7:0] | - | Number of vme addresses written |
| [11:8] | R | jsm_prom_sm_vec[3:0] | - | JSM PROM State Machine state vector |
| [14:12] | R | jsm_format_sm_vec[2:0] | - | JSM Data Format Machine state vector |
| [15] | R | -- | 0 | unassigned |

#### Adr E2	ADR_VMESM4	VME State Machine Write-Data Check

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| data 15 | data 14 | data 13 | data 12 | data 11 | data 10 | data 9 | data 8 | data 7 | data 6 | data 5 | data 4 | data 3 | data 2 | data 1 | data 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [15:00] | R | vmesm4_rd[15:0] | 55AAh | vsm_path_ok=1 if vsm writes 55aa to this adr |

#### Adr E4	ADR_DDDSM	RAT 3D3444 State Machine Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| verify dly1 | verify dly0 | rx phase | link tmb | oe 3 | oe 2 | oe 1 | oe o | verify ok | sm busy | auto start | serial from | serial to | adr latch | dddr clock | dddr start |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | dddr_start_vme | 0 | Start DDDR State Machine |
| [01] | RW | dddr_clock | 0 | DDDR manual-mode clock |
| [02] | RW | dddr_adr_latch | 1 | DDDR manual-mode address latch, active low |
| [03] | RW | dddr_serial_in | 0 | Serial data to DDDR chip |
| [04] | R | dddr_serial_out | 0 | Serial data from DDDR chip |
| [05] | RW | dddr_auto_start | 1 | DDDR State Machine autostart state |
| [06] | R | dddr_busy | 0 | DDDR State Machine busy |
| [07] | R | dddr_verify_ok | 1 | DDDR data read back verified OK |
| [11:08] | RW | dddr_oe[3:0] | 00112 | 3D3444 output enables, 1=enable |
| [12] | RW | dddr_linktmb | 1 | 1=start RAT machine when starting TMB machine |
| [13] | RW | dddr_rxphase | 1 | 1=use negative clock edge to latch verify data, 0=posedge |
| [15:14] | RW | dddr_verify_dly[1:0] | 3 | Delay before latching verify data |

#### Adr E6	ADR_DDDR0	RAT 3D3444 RPC Delays, 1 step = 2ns

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | delay_ch0[3:0] | 3 | RPC0 rx clock |
| [07:04] | RW | delay_ch1[3:0] | 3 | RPC1 rx clock |
| [11:08] | RW | delay_ch2[3:0] | 0 | RAT2 rx clock, not used |
| [15:12] | RW | delay_ch3[3:0] | 0 | RAT2 rx clock, not used |

#### Adr E8	ADR_UPTIME	Uptime Counter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | up time 14 | up time 13 | up time 12 | up time 11 | up time 10 | up time 9 | up time 8 | up time 7 | up time 6 | up time 5 | up time 4 | up time 3 | up time 2 | up time 1 | up time 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [04:00] | R | uptime[14:0] | - | Seconds since last hard-reset |
| [15] | R | -- | 0 | unassigned |

#### Adr EA	ADR_BDSTATUS	Board Status Summary (copy of raw-hits header)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| status 15 | status 14 | status 13 | status 12 | status 11 | status 10 | status 9 | status 8 | status 7 | status 6 | status 5 | status 4 | status 3 | status 2 | status 1 | status 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | bdstatus_ok | 1 | Voltages OK, temperature OK, prom-load OK |
| [01] | R | vstat_5p0V | 1 | Voltage Comparator +5.0V, 1=OK |
| [02] | R | vstat_3p3v | 1 | Voltage Comparator +3.3V, 1=OK |
| [03] | R | vstat_1p8v | 1 | Voltage Comparator +1.8V, 1=OK |
| [04] | R | vstat_1p5v | 1 | Voltage Comparator +1.5V, 1=OK |
| [05] | R | /t_crit | 1 | Temperature ADC Tcritical, 1=OK |
| [06] | R | vsm_ok | 1 | VME Machine ran without errors |
| [07] | R | vsm_aborted | 0 | VME State machine aborted reading PROM |
| [08] | R | vsm_cksum_ok | 1 | VME Check-sum matches PROM contents |
| [09] | R | vsm_wdcnt_ok | 1 | VME Word count matches PROM contents |
| [10] | R | jsm_ok | 1 | JTAG State machine completed without errors |
| [11] | R | jsm_aborted | 0 | JTAG State machine aborted reading PROM |
| [12] | R | jsm_cksum_ok | 1 | JTAG Check-sum matches PROM contents |
| [13] | R | jsm_wdcnt_ok | 1 | JTAG Word count matches PROM contents |
| [14] | R | jsm_tck_fpga_ok | 1 | JTAG tck loopback detected on chain adr C |
| [15] | R | jsm_tckcnt_ok | 1 | JTAG state machine TCK count matches PROM |

#### Adr EC	ADR_BXN_CLCT	CLCT BXN At CLCT-Pretrigger

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | clct bxn 11 | clct bxn 10 | clct bxn 9 | clct bxn 8 | clct bxn 7 | clct bxn 6 | clct bxn 5 | clct bxn 4 | clct bxn 3 | clct bxn 2 | clct bxn 1 | clct bxn 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [11:00] | R | bxn_clct_vme[11:0] | - | CLCT BXN latched at last CLCT pretrigger |
| [15:12] | R | -- | 0 | unassigned |

#### Adr EE	ADR_BXN_ALCT	ALCT BXN At ALCT-Valid-Pattern-Flag

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | alct bxn 4 | alct bxn 3 | alct bxn 2 | alct bxn 1 | alct bxn 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [4:00] | R | bxn_alct_vme[4:0] | - | ALCT BXN latched at last ALCT vpf |
| [15:5] | R | -- | 0 | unassigned |

#### Adr F0	ADR_LAYER_TRIG	Layer-Trigger Mode

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clct throt 7 | clct throt 6 | clct throt 5 | clct throt 4 | clct throt 3 | clct throt 2 | clct throt 1 | clct throt 0 | 0 | nlayrs hit 2 | nlayrs hit 1 | nlayrs hit 0 | layer thresh 2 | layer thresh 1 | layer thresh 0 | layer trig en |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | layer_trigger_en | 0 | 1=Enable layer trigger mode(see adr 68 p44) |
| [03:01] | RW | lyr_thresh_pretrig[2:0] | 4 | layer-trigger threshold |
| [06:04] | R | nlayers_hit_vme[2:0] | -- | number layers hit on last layer-trigger |
| [07] | RW | -- | -- | Unassigned |
| [15:08] | RW | clct_throttle[7:0] | 0 | CLCT Pre-trigger rate throttle |

#### Adr F2	ADR_ISE_VERSION	ISE Version

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ver 7 | ver 6 | ver 5 | ver 4 | ver 3 | ver 2 | ver 1 | ver 0 | minor 3 | minor 2 | minor 1 | minor 0 | spack 3 | spack 2 | spack 1 | spack 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [03:00] | R | ise_version[3:0] | 03h | ISE Service Pack |
| [07:04] | R | ise_version[7:4] | 01h | ISE Minor Version |
| [15:08] | R | ise_version[15:8] | 10h | ISE Major Version |

#### Adr F4	ADR_TEMP0	Pattern Finder Pre-Trigger

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| adj cfeb dist 5 | adj cfeb dist 4 | adj cfeb dist 3 | adj cfeb dist 2 | adj cfeb dist 1 | adj cfeb dist 0 | pid thresh post 3 | pid thresh post 2 | pid thresh post 1 | pid thresh post 0 | pid thresh pretrig 3 | pid thresh pretrig 2 | pid thresh pretrig 1 | pid thresh pretrig 0 | 0 | clct blank |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | clct_blanking | 1 | 1=blank non-vpf clct output [requires alct-only or l1a-notmb mode enabled to change this to 0] |
| [01] | RW | -- | 0 | Unassigned |
| [05:02] | RW | pid_thresh_pretrig[3:0] | 0 | Minimum pattern ID 0x0-0xA for pre-trigger |
| [09:06] | RW | pid_thresh_postdrift[3:0] | 0 | Minimum pattern ID 0x0-0xA after drift delay |
| [15:10] | RW | adjcfeb_dist[5:0] | 5 | Distance from key on CFEBn to CFEBn+1 to set active feb flag on CFEBn+1 for DMB Setting to 5 enables hs0,1,2,3,4 and hs31,30,29,29,28,27. |

#### Adr F6	ADR_TEMP1	CLCT Separation

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clct sep 7 | clct sep 6 | clct sep 5 | clct sep 4 | clct sep 3 | clct sep 2 | clct sep 1 | clct sep 0 | 0 | clct sep ram sel_ab | clct sep ram adr 3 | clct sep ram adr 2 | clct sep ram adr 1 | clct sep ram adr 0 | clct sep ram we | clct sep src |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | clct_sep_src | 1 | CLCT separation source 1=vme, 0=ram |
| [01] | RW | clct_sep_ram_we | 0 | 1=enable CLCT separation ram for writing |
| [05:02] | RW | clct_sep_ram_adr[3:0] | 0 | CLCT separation RAM rw address 0-F |
| [06] | RW | clct_sep_ram_sel_ab | 0 | 1=read me1a RAM, 0=me1b RAM or std RAM |
| [07] | RW | -- | -- | Unassigned |
| [15:08] | RW | clct_sep_vme[7:0] | 10 | Minimum CLCT separation in key ½-strips |

#### Adr F8	ADR_TEMP2	CLCT Separation RAM Data

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pspan 7 | pspan 6 | pspan 5 | pspan 4 | pspan 3 | pspan 2 | pspan 1 | pspan 0 | nspan 7 | nspan 6 | nspan 5 | nspan 4 | nspan 3 | nspan 2 | nspan 1 | nspan 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW/R | clct_sep_ram_wr[7:0] | 10 | nspan CLCT separation RAM data, blanks keys from 1st key to keys>=key-nspan |
| [15:08] | RW/R | clct_sep_ram_wr[15:8] | 10 | pspan CLCT separation RAM data, blanks keys from 1st key to keys<=key+pspan |

#### Adr FA	ADR_PARITY	SEU Parity Errors

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| perr mux 5 | perr mux 4 | perr mux 3 | perr mux 2 | perr mux 1 | perr mux 0 | 0 | 0 | perr reset | perr ff | perr | perr en | perr adr 3 | perr adr 2 | perr adr 1 | perr adr 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | perr_adr[3:0] | 0 | Parity data bank select address |
| [4] | R | perr_en | 1 | Parity error latch enabled |
| [5] | R | perr | 0 | Parity error summary |
| [6] | R | perr_ff | 0 | Parity error summary, latched |
| [7] | RW | perr_reset | 0 | Parity error reset |
| [9:8] | RW | -- | 0 | Unassigned |
| [15;10] | R | parity_rd_mux[5:0] | 0 | Parity data multiplexer, selected by perr_adr[] |

adr 0:	parity_rd_mux <= perr_ram_ff[ 5: 0];			// R	cfeb0 rams

adr 1:	parity_rd_mux <= perr_ram_ff[11: 6];			// R	cfeb1 rams

adr 2:	parity_rd_mux <= perr_ram_ff[17:12];			// R	cfeb2 rams

adr 3:	parity_rd_mux <= perr_ram_ff[23:18];			// R	cfeb3 rams

adr 4:	parity_rd_mux <= perr_ram_ff[29:24];			// R	cfeb4 rams

adr 5:	parity_rd_mux <= perr_ram_ff[35:30];			// R	cfeb5 rams

adr 6:	parity_rd_mux <= perr_ram_ff[41:36];			// R	cfeb6 rams

adr 7:	parity_rd_mux <= {1’b0,perr_ram_ff[46:42]};		// R	rpc   rams

adr 8:	parity_rd_mux <= {4’h0,perr_ram_ff[48:47]};		// R	mini  rams

adr 9:	parity_rd_mux <= {1’b0,perr_cfeb[6:0]};		// R	cfeb parity errors

adr 10:	parity_rd_mux <= {1’b0,perr_cfeb_ff[6:0]};		// R	cfeb parity errors,latched

adr 11:	parity_rd_mux <= {4’h0,perr_rpc_ff,perr_rpc};	// R	rpc  parity errors,latched

adr12:	parity_rd_mux <= {4’h0,perr_mini_ff, perr_mini};	// R	mini parity errors,latched

Adr FC	ADR_CCB_STAT1	CCB Status Register Continued from Adr 2E

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | ccb_ttcrx_lock_never | 0 | TTCrx lock never achieved |
| [01] | R | ccb_ttcrx_lost_ever | 0 | TTCrx lock was lost at least once |
| [02] | R | ccb_qpll_lock_never | 0 | QPLL lock never achieved |
| [03] | R | ccb_qpll_lost_ever | 0 | QPLL lock was lost at least once |
| [15:04] | R | -- | 0 | Unassigned |

#### Adr FE	ADR_BXN_L1A	CLCT BXN at L1A Arrival

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | l1a bxn 11 | l1a bxn 10 | l1a bxn 9 | l1a bxn 8 | l1a bxn 7 | l1a bxn 6 | l1a bxn 5 | l1a bxn 4 | l1a bxn 3 | l1a bxn 2 | l1a bxn 1 | l1a bxn 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [11:00] | R | bxn_l1a_vme[11:0] | - | CLCT BXN latched at last L1A arrival |
| [15:12] | R | -- | 0 | unassigned |

#### Adr 100	ADR_L1A_LOOKBACK	L1A Lookback Distance

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l1a win pri en | inj rdata 17 | inj rdata 16 | inj wdata 17 | inj wdata 16 | l1a look back 10 | l1a look back 9 | l1a look back 8 | l1a look back 7 | l1a look back 6 | l1a look back 5 | l1a look back 4 | l1a look back 3 | l1a look back 2 | l1a look back 1 | l1a look back 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [10:00] | RW | l1a_lookback[10:0] | 128 | bx to look back from L1As wr_buf_adr for L1A-only readouts |
| [12:11] | RW | inj_wdata[17:16] | 0 | Injector RAM write data MSBs |
| [14:13] | R | inj_rdata[17:16] | 0 | Injector RAM read data MSBs |
| [15] | RW | l1a_win_pri_en | 1 | 1=Limit TMB to 1 event readout per L1A |

#### Adr 102	ADR_SEQ_DEBUG	Sequencer Debug Signals

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | l1a bxn 11 | l1a bxn 10 | l1a bxn 9 | l1a bxn 8 | l1a bxn 7 | l1a bxn 6 | l1a bxn 5 | l1a bxn 4 | l1a bxn 3 | l1a bxn 2 | l1a bxn 1 | l1a bxn 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | seqdeb_adr[3:0] | - | Sequencer signal address 0-15 |
| [15:04] | R | seqdeb_rd_mux[11:0] | - | Multiplexed sequencer data for debugging |

adr 0:	deb_wr_buf_adr[10:0]		Buffer write address at last pretrig

adr 1:	deb_buf_push_adr[10:0]		Queue push address at last push

adr 2:	deb_buf_pop_adr[10:0]		Queue pop  address at last pop

adr 3:	deb_buf_push_data[11:0]	Queue push data at last push

adr 4:	deb_buf_push_data[23:12]	  +Queue push data at last push

adr 5:	deb_buf_push_data[31:24]	  +Queue push data at last push

adr 6:	deb_buf_pop_data[11:0]		Queue pop data at last pop

adr 7:	deb_buf_pop_data[23:12] 	  +Queue pop data at last pop

adr 8:	deb_buf_pop_data[31:24] 	  +Queue pop data at last pop

#### Adr 104	ADR_ALCT_SYNC_CTRL			ALCT Sync Mode Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sync pre 3 | sync pre 2 | sync pre 1 | sync pre 0 | 0 | 0 | sync 2nd err ff | sync 1st err ff | sync 2nd err | sync 1st err | sync clr | sync rand | sync dly 3 | sync dly 2 | sync dly 1 | sync dly 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | alct_sync_rxdata_dly [3:0] | 0 | Sync mode delay pointer to valid data |
| [04] | RW | alct_sync_tx_random | 0 | Sync mode tmb transmits random data to alct |
| [05] | RW | alct_sync_clr_err | 0 | ALCT sync mode clear rng error FFs |
| [06] | R | alct_sync_1st_err | 0 | 1st-in-time match ok, alct-to-tmb |
| [07] | R | alct_sync_2nd_err | 0 | 2nd-in-time match ok, alct-to-tmb |
| [08] | R | alct_sync_1st_err_ff | 0 | 1st-in-time match ok, alct-to-tmb, latched |
| [09] | R | alct_sync_2nd_err_ff | 0 | 2nd-in-time match ok, alct-to-tmb, latched |
| [11:10] | RW | -- | 0 | Unassigned |
| [15:12] | RW | alct_sync_rxdata_pre[3:0] | 9 | Sync mode pre-delay pointer to valid data |

#### Adr 106	ADR_ALCT_SYNC_TXDATA_1ST	ALCT Sync Mode Transmit Data 1st

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | sync 1st 9 | sync 1st 8 | sync 1st 7 | sync 1st 6 | sync 1st 5 | sync 1st 4 | sync 1st 3 | sync 1st 2 | sync 1st 1 | sync 1st 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [09:00] | RW | alct_sync_txdata_1st[9:0] | - | Sync mode data to send for loopback 1st in time |
| [15:10] | RW | -- | 0 | Unassigned |

#### Adr 108	ADR_ALCT_SYNC_TXDATA_2ND	ALCT Sync Mode Transmit Data 2nd

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | sync 2nd 9 | sync 2nd 8 | sync 2nd 7 | sync 2nd 6 | sync 2nd 5 | sync 2nd 4 | sync 2nd 3 | sync 2nd 2 | sync 2nd 1 | sync 2nd 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [09:00] | RW | alct_sync_txdata_1st[9:0] | - | Sync mode data to send for loopback 1st in time |
| [15:10] | RW | -- | 0 | Unassigned |

#### Adr 10A	ADR_SEQ_OFFSET1	Sequencer Counter Offsets Continued [from Adr076]

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | bxn offset 11 | bxn offset 10 | bxn offset 9 | bxn offset 8 | bxn offset 7 | bxn offset 6 | bxn offset 5 | bxn offset 4 | bxn offset 3 | bxn offset 2 | bxn offset 1 | bxn offset 0 |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [11:00] | RW | bxn_offset_l1a[11:0] | - | L1A bxn offset preset value |
| [15:12] | RW | -- | 0 | Unassigned |

#### Adr 10C	ADR_MINISCOPE	Internal 16 Channel Digital Miniscope

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | tbins pretrig 4 | tbins pretrig 3 | tbins pretrig 2 | tbins pretrig 1 | tbins pretrig 0 | tbins 4 | tbins 3 | tbins 2 | tbins 1 | tbins 0 | tbins word nsert | tbins test mode | read enable |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | mini_read_enable | 1 | Enable Miniscope readout to DMB |
| [01] | RW | mini_tbins_test | 0 | Miniscope data=write_address, for testing |
| [02] | RW | mini_tbins_word | 1 | Insert tbins and pretrig tbins in 1st word |
| [07:03] | RW | fifo_tbins_mini[4:0] | 22 | Number Mini FIFO time bins to read out, must multiple of 2 but not of 4 |
| [12:08] | RW | fifo_pretrig_mini[4:0] | 4 | Number Mini FIFO time bins before pre-trigger |
| [15:13] | RW | -- | 0 | Unassigned |

#### Adr 10E	ADR_PHASER0	ALCT rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alct rxd delay 7 | alct rxd delay 6 | alct rxd delay 5 | alct rxd delay 4 | alct rxd delay 3 | alct rxd delay 2 | alct rxd delay 1 | alct rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_alct_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_alct_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_alct_rxd | 0 | Phase shifter busy |
| [03] | R | lock_alct_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_alct_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | alct_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | alct_rxd_delay[7:0] | 32 | Phase delay to latch data received from ALCT approximately 0.1ns steps (clock period/256) |

#### Adr 110	ADR_PHASER1	ALCT txd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alct txd delay 7 | alct txd delay 6 | alct txd delay 5 | alct txd delay 4 | alct txd delay 3 | alct txd delay 2 | alct txd delay 1 | alct txd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_alct_txd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_alct_txd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_alct_txd | 0 | Phase shifter busy |
| [03] | R | lock_alct_txd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_alct_txd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | alct_txd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | alct_txd_delay[7:0] | 32 | Phase delay for data transmitted to ALCT approximately 0.1ns steps (clock period/256) |

#### Adr 112	ADR_PHASER2	CFEB0 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb0_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb0_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb0_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb0_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb0_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb0_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb0_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 114	ADR_PHASER3	CFEB1 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb1_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb1_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb1_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb1_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb1_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb1_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb1_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 116	ADR_PHASER4	CFEB2 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb2_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb2_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb2_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb2_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb2_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb2_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb2_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 118	ADR_PHASER5	CFEB3 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb3_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb3_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb3_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb3_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb3_rxd[2:0] | 0 | Phase shifter machine state |
| [07] | RW | cfeb3_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb3_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 11A	ADR_PHASER6	CFEB4 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb4_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb4_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb4_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb4_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb4_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb4_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb4_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 11C	ADR_DELAY0_INT	CFEB DDR RxD Interstage Delays

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb3 delay 3 | cfeb3 delay 2 | cfeb3 delay 1 | cfeb3 delay 0 | cfeb2 delay 3 | cfeb2 delay 2 | cfeb2 delay 1 | cfeb2 delay 0 | cfeb1 delay 3 | cfeb1 delay 2 | cfeb1 delay 1 | cfeb1 delay 0 | cfeb0 delay 3 | cfeb0 delay 2 | cfeb0 delay 1 | cfeb0 delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | cfeb0_rxd_int_delay[3:0] | 0 | Delay data received from CFEB0 by integer bx |
| [07:04] | RW | cfeb1_rxd_int_delay [3:0] | 0 | Delay data received from CFEB1 by integer bx |
| [11:08] | RW | cfeb2_rxd_int_delay [3:0] | 0 | Delay data received from CFEB2 by integer bx |
| [15:12] | RW | cfeb3_rxd_int_delay [3:0] | 0 | Delay data received from CFEB3 by integer bx |

#### Adr 11E	ADR_DELAY1_INT	CFEB DDR RxD Interstage Delays Continued

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | cfeb6 delay 3 | cfeb6 delay 2 | cfeb6 delay 1 | cfeb6 delay 0 | cfeb5 delay 3 | cfeb5 delay 2 | cfeb5 delay 1 | cfeb5 delay 0 | cfeb4 delay 3 | cfeb4 delay 2 | cfeb4 delay 1 | cfeb4 delay 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [03:00] | RW | cfeb4_rxd_int_delay 3:0] | 0 | Delay data received from CFEB4 by integer bx |
| [07:04] | RW | cfeb5_rxd_int_delay 3:0] | 0 | Delay data received from CFEB5 by integer bx |
| [11:08] | RW | cfeb6_rxd_int_delay 3:0] | 0 | Delay data received from CFEB6 by integer bx |
| [15:12] | RW | -- | 0 | Unassigned |

#### Adr 120	ADR_SYNC_ERR_CTRL	Synchronization Error Control

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sync err forced | clock lock err | bx0 match err | alct ecc tx err | alct ecc rx err | clct bx0 err | sync err | err stops L1As | err stops pretrig | err blanks LCTs | clock lock lost en | bx0 match en | alct tx ecc en | alct rx ecc en | clct bx0 en | sync err reset |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | sync_err_reset | 0 | VME sync error reset |
| Sync error source enables: |  |  |  |  |
| [01] | RW | clct_bx0_sync_err_en | 1 | TMB clock pulse count err: bxn!=0+offset at ttc_bx0 arrival |
| [02] | RW | alct_ecc_rx_err_en | 0 | ALCT uncorrected ECC error in data ALCT received from TMB |
| [03] | RW | alct_ecc_tx_err_en | 0 | ALCT uncorrected ECC error in data ALCT transmitted to TMB |
| [04] | RW | bx0_match_err_en | 0 | ALCT alct_bx0 != clct_bx0 in LCT to MPC |
| [05] | RW | clock_lock_lost_err_en | 0 | 40MHz main clock lost lock |
| Sync error action enables: |  |  |  |  |
| [06] | RW | sync_err_blanks_mpc_en | 0 | Sync error blanks LCTs to MPC |
| [07] | RW | sync_err_stops_pretrig_en | 0 | Sync error stops CLCT pre-triggers |
| [08] | RW | sync_err_stops_readout_en | 0 | Sync error stops L1A readouts |
| Sync error types latched for VME readout: |  |  |  |  |
| [09] | R | sync_err | 0 | Sync error OR of enabled types of error |
| [10] | R | clct_bx0_sync_err_ff | 0 | TMB clock pulse count err: bxn!=0+offset at ttc_bx0 arrival |
| [11] | R | alct_ecc_rx_err_ff | 0 | ALCT uncorrected ECC error in data ALCT received from TMB |
| [12] | R | alct_ecc_tx_err_ff | 0 | ALCT uncorrected ECC error in data ALCT transmitted to TMB |
| [13] | R | bx0_match_err_ff | 0 | ALCT alct_bx0 != clct_bx0 in LCT to MPC |
| [14] |  | clock_lock_lost_err_ff | 0 | 40MHz main clock lost lock |
| [15] | RW | sync_err_forced | 0 | Force sync_err=1 |

See p48 Adr86[1:0] for tmb_sync_err_en[1:0]    Allow sync_err to MPC for either muon

See p38 Adr38[2]    for alct_ecc_err_blank          Blank alct muons having uncorrected ecc errors

#### Adr 122	ADR_CFEB_BADBITS_CTRL	CFEB Bad Bits Control/Status (see 0x15C for V6)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb blocked | found cfeb4 | found cfeb3 | found cfeb2 | found cfeb1 | found cfeb0 | block cfeb4 | block cfeb3 | block cfeb2 | block cfeb1 | block cfeb0 | reset cfeb4 | reset cfeb3 | reset cfeb2 | reset cfeb1 | reset cfeb0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [04:00] | RW | cfeb_badbits_reset[4:0] | 0 | 0x1F=Reset bad cfeb bits FFs for cfeb[n] |
| [09:05] | RW | cfeb_badbits_block[4:0] | 0 | 0x1F=Block bad cfeb bits in cfeb[n] |
| [14:10] | R | cfeb_badbits_found[4:0] | 0 | CFEB[n] has at least 1 bad bit |
| [15] | R | cfeb_badbits_blocked | 0 | At least one CFEB has a bad bit that was blocked |

#### Adr 124	ADR_CFEB_BADBITS_TIMER	CFEB Bad Bits Check Interval

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nbx 15 | nbx 14 | nbx 13 | nbx 12 | nbx 11 | nbx 10 | nbx 9 | nbx 8 | nbx 7 | nbx 6 | nbx 5 | nbx 4 | nbx 3 | nbx 2 | nbx 1 | nbx 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | cfeb_badbits_nbx [15:0] | 3564 | Check Interval for CFEB bad bits, bx units |

#### Adr 126	ADR_CFEB0_BADBITS_LY01	CFEB0 Ly0,Ly1 Bad Bits List

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | cfeb0_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| [15:08] | R | cfeb0_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 128	ADR_CFEB0_BADBITS_LY23	CFEB0 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb0_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb0_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 12A	ADR_CFEB0_BADBITS_LY45	CFEB0 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb0_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb0_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 12C	ADR_CFEB1_BADBITS_LY01	CFEB1 Ly0,Ly1 Bad Bits List

| [07:00] | R | cfeb1_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb1_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 12E	ADR_CFEB1_BADBITS_LY23	CFEB1 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb1_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb1_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 130	ADR_CFEB1_BADBITS_LY45	CFEB1 Ly4,Ly5 Bad Bits List

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | cfeb1_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| [15:08] | R | cfeb1_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 132	ADR_CFEB2_BADBITS_LY01	CFEB2 Ly0,Ly1 Bad Bits List

| [07:00] | R | cfeb2_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb2_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 134	ADR_CFEB2_BADBITS_LY23	CFEB2 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb2_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb2_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 136	ADR_CFEB2_BADBITS_LY45	CFEB2 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb2_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb2_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 138	ADR_CFEB3_BADBITS_LY01	CFEB3 Ly0,Ly1 Bad Bits List

| [07:00] | R | cfeb3_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb3_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 13A	ADR_CFEB3_BADBITS_LY23	CFEB3 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb3_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb3_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 13C	ADR_CFEB3_BADBITS_LY45	CFEB3 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb3_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb3_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 13E	ADR_CFEB4_BADBITS_LY01	CFEB4 Ly0,Ly1 Bad Bits List

| [07:00] | R | cfeb4_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb4_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 140	ADR_CFEB4_BADBITS_LY23	CFEB4 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb4_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb4_ly3_badbits [7:0] | 000000002 | 1=1=CFEB rx bit[n] went bad |

#### Adr 142	ADR_CFEB4_BADBITS_LY45	CFEB4 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb4_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb4_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 144	ADR_ALCT_STARTUP_DELAY	     ALCT startup delay milliseconds for Spartan-6

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d[15] | d[14] | d[13] | d[12] | d[11] | d[10] | d[9] | d[8] | d[7] | d[6] | d[5] | d[4] | d[3] | d[2] | d[1] | d[0] |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [15:00] | RW | alct_startup_delay [15:0] | 116 | Msec to wait after TMB powers up before Initializing DDD delays and ALCT JTAG. This setting is only used after TMB first powers up or has a hard reset, so changes need to be stored in VME PROM. ALCT Spartan-6 takes 212msec to configure. TMB Virtex-2 takes 100msec. This register holds the number of msec to wait after TMB configures, so a value of 116 corresponds to 100ms + 116ms =216ms after a TMB hard reset. |

#### Adr 146	ADR_ALCT_STARTUP_STATUS		ALCT startup delay machine status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | alct start done | alct wait cfg | alct wait vme | alct wait dll | alct start msec | vsm ready | power up | global reset |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | R | global_reset | 0 | Global reset |
| [01] | R | power_up | 1 | DLL clock locked, we wait for it |
| [02] | R | vsm_ready | 1 | Injector RAM read data MSBs |
| [03] | R | alct_startup_msec | 0 | Startup machine millisecond pulse, width=25ns |
| [04] | R | alct_wait_dll | 0 | Startup machine waiting for TMB DLL lock |
| [05] | R | alct_wait_vme | 0 | Startup machine waiting for TMB VME user PROM |
| [06] | R | alct_wait_cfg | 0 | Startup machine waiting for ALCT FPGA to config |
| [07] | R | alct_startup_done | 1 | Startup machine done ALCT FPGA assumed configured |
| [15:08] | R | -- | 0 | Unassigned |

#### Adr 148	ADR_V6_SNAP12_QPLL	Virtex-6 SNAP12 Serial interface + QPLL status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | r12 fok | r12 sdat | r12 sclk | 0 | qpll err | qpll lock | qpll nrst |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | qpll_nrst | 1 | nReset QPLL, 0=reset |
| [01] | R | qpll_lock | 0 | QPLL locked status |
| [02] | R | qpll_err | 0 | QPLL error status |
| [03] | RW | -- | 0 | Unassigned |
| [04] | R | r12_sclk | 1 | SNAP12 Serial interface clock, drive high |
| [05] | R | r12_sdat | 0 | SNAP12 Serial interface data |
| [06] | R | r12_fok | 0 | SNAP12 Serial interface status |
| [15:07] | R | -- | 0 | Unassigned |

#### Adr 14A	ADR_V6_GTX_RX_ALL	Virtex-6 master GTX control and status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gtx err sum bit 7 | gtx err sum bit 6 | gtx err sum bit 5 | gtx err sum bit 4 | gtx err sum bit 3 | gtx err sum bit 2 | gtx err sum bit 1 | gtx err sum bit 0 | gtx pol swap | gtx link bad | gtx linkhad err | gtx link good | gtx sync done | gtx en prbs | gtx reset | gtx en |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | gtx_rx_enable_all | 1 | Enable all GTX optical inputs, disables copper CFEBs |
| [01] | RW | gtx_rx_reset_all | 0 | Reset all GTX |
| [02] | RW | gtx_rx_en_prbs_test_all | 0 | Select all GTX for PRBS test input mode |
| [03] | R | &gtx_rx_sync_done[6:0] | 1 | All GTX are ready |
| [04] | R | &gtx_link_good[6:0] | 1 | All GTX links are locked (over 15 BX with clean structure) |
| [05] | R | ¦gtx_link_had_err[6:0] | 0 | At least one GTX link had an error since last reset |
| [06] | R | ¦gtx_link_bad[6:0] | 0 | At least one GTX link had over 100 errors |
| [07] | R | ¦gtx_rx_pol_swap[6:0] | 0 | GTX 5,6 [ie dcfeb 4,5] have swapped rx board routes |
| [15:8] | R | gtx_rx_err_count_sum_all | 0 | Sum of GTX link error counts (full scale count is hex FE) |

#### Adr 14C-158	    ADR_V6_GTX_RX0-6	Virtex-6 individual GTX (idcfeb[6:0]) control and status

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gtx err countbit 7 | gtx err countbit 6 | gtx err countbit 5 | gtx err countbit 4 | gtx err countbit 3 | gtx err countbit 2 | gtx err countbit 1 | gtx err countbit 0 | gtx pol swap | gtx link bad | gtx linkhad err | gtx link good | gtx sync done | gtx en prbs | gtx reset | gtx en |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | gtx_rx_enable[idcfeb] | 1 | Enable this GTX optical input, disables copper input |
| [01] | RW | gtx_rx_reset[idcfeb] | 0 | Reset this GTX |
| [02] | RW | gtx_rx_en_prbs_test[idcfeb] | 0 | Select this GTX for PRBS test input mode |
| [03] | R | gtx_rx_sync_done[idcfeb] | 1 | GTX ready |
| [04] | R | gtx_link_good | 1 | GTX link is locked (over 15 BX with clean data frames) |
| [05] | R | gtx_link_had_err | 0 | GTX link had an error (bad data frame) since last reset |
| [06] | R | gtx_link_bad | 0 | GTX link had over 100 errors since last reset |
| [07] | R | gtx_rx_pol_swap[idcfeb] | 0 | GTX 5,6 [ie dcfeb 4,5] have swapped rx board routes |
| [15:8] | R | gtx_rx_err_count[idcfeb] | 0 | GTX link error count (full scale count is hex E0) |

#### Adr 15A	ADR_V6_SYSMON	Virtex-6 Sysmon ADC

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | gtx err | gtx pol swal | gtx sync done | gtx match | gtx valid | gtx fc | gtx start | gtx en prbs | gtx reset err cnt | gtx reset | gtx en |

| Bits | Dir | Signal | Typical | Description |
| --- | --- | --- | --- | --- |
| [04:00] | RW | adc_adr[4:0] | 0 | ADC channel |
| [05] | R | adc_valid | 0 | ADC RAM has valid data for this adc_adr, readonly |
| [05] | W | adc_reset | 0 | Reset Sysmon module, writeonly |
| [15:6] | R | adc_data[15:6] | 0 | ADC counts for this adc_adr |

#### Virtex-6 Sysmon ADC Channel Assignments

adr	Source		Units		   Conversion Factor

0	Temperature		Degrees C	= (ADC code × 503.975)/1024 – 273.15

1	VccINT		Volts		= (ADC Code / 1024) x 3V

2	VccAUX		Volts		= (ADC Code / 1024) x 3V

4	Vref  1.25V		Volts		= (ADC Code / 1024) x 3V

5	Vzero 0.00V		Volts		= (ADC Code / 1024) x 3V

#### Adr 15C	ADR_V6_CFEB_BADBITS_CTRL	CFEB Bad Bits Control/Status (See Adr 0x122)

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | found cfeb6 | found cfeb5 | block cfeb6 | block cfeb5 | reset cfeb6 | reset cfeb5 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [01:00] | RW | cfeb_badbits_reset[6:5] | 0 | 0x1F=Reset bad cfeb bits FFs for cfeb[n] |
| [03:02] | RW | cfeb_badbits_block[6:5] | 0 | 0x1F=Block bad cfeb bits in cfeb[n] |
| [05:04] | R | cfeb_badbits_found[6:5] | 0 | CFEB[n] has at least 1 bad bit |
| [15:06] | RW | -- | 0 | Unassigned |

#### Adr 15E	ADR_ V6_CFEB5_BADBITS_LY01	CFEB5 Ly0,Ly1 Bad Bits List

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | R | cfeb5_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| [15:08] | R | cfeb5_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 160	ADR_ V6_CFEB5_BADBITS_LY23	CFEB5 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb5_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb5_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 162	ADR_ V6_CFEB5_BADBITS_LY45	CFEB5 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb5_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb5_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 164	ADR_ V6_CFEB6_BADBITS_LY01	CFEB6 Ly0,Ly1 Bad Bits List

| [07:00] | R | cfeb6_ly0_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb6_ly1_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 166	ADR_ V6_CFEB6_BADBITS_LY23	CFEB6 Ly2,Ly3 Bad Bits List

| [07:00] | R | cfeb6_ly2_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb6_ly3_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 168	ADR_ V6_CFEB6_BADBITS_LY45	CFEB6 Ly4,Ly5 Bad Bits List

| [07:00] | R | cfeb6_ly4_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |
| --- | --- | --- | --- | --- |
| [15:08] | R | cfeb6_ly5_badbits [7:0] | 000000002 | 1=CFEB rx bit[n] went bad |

#### Adr 16A	ADR_V6_PHASER7	CFEB5 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb0_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb0_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb0_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb0_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb0_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb0_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb0_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 16C	ADR_V6_PHASER8	CFEB6 rxd Digital Phase Shifter

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cfeb rxd delay 7 | cfeb rxd delay 6 | cfeb rxd delay 5 | cfeb rxd delay 4 | cfeb rxd delay 3 | cfeb rxd delay 2 | cfeb rxd delay 1 | cfeb rxd delay 0 | pos neg | sm 2 | sm 1 | sm 0 | lock | busy | reset | fire |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [00] | RW | fire_cfeb0_rxd | 0 | Set new phase, software sets then unsets |
| [01] | RW | reset_cfeb0_rxd | 0 | Reset current phase to 32 |
| [02] | R | phaser_busy_cfeb0_rxd | 0 | Phase shifter busy |
| [03] | R | lock_cfeb0_rxd | 1 | DCM lock status |
| [06:04] | R | phaser_sm_cfeb0_rxd[2:0] | 0 | Phase shifter machine state vector |
| [07] | RW | cfeb0_rxd_posneg | 0 | 0=latch inter-stage on falling main clock edge 1=latch inter-stage on rising main clock edge |
| [15:08] | RW | cfeb0_rxd_delay[7:0] | 32 | Phase delay to latch data received from CFEB approximately 0.1ns steps (clock period/256) |

#### Adr 16E	ADR_V6_HCM501	CFEB5 Ly0,Ly1 Hot Channel Mask

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ly1 distrip 7 | ly1 distrip 6 | ly1 distrip 5 | ly1 distrip 4 | ly1 distrip 3 | ly1 distrip 2 | ly1 distrip 1 | ly1 distrip 0 | ly0 distrip 7 | ly0 distrip 6 | ly0 distrip 5 | ly0 distrip 4 | ly0 distrip 3 | ly0 distrip 2 | ly0 distrip 1 | ly0 distrip 0 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [07:00] | RW | cfeb5_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| [15:08] | RW | cfeb5_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 170	ADR_V6_HCM523	CFEB5 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb5_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb5_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 172	ADR_V6_HCM545	CFEB5 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb5_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb5_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 174	ADR_V6_HCM601	CFEB6 Ly0,Ly1 Hot Channel Mask

| [07:00] | RW | cfeb6_ly0_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 0 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb6_ly1_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 1 |

#### Adr 176	ADR_V6_HCM623	CFEB6 Ly2,Ly3 Hot Channel Mask

| [07:00] | RW | cfeb6_ly2_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 2 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb6_ly3_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 3 |

#### Adr 178	ADR_V6_HCM645	CFEB6 Ly4,Ly5 Hot Channel Mask

| [07:00] | RW | cfeb6_ly4_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 4 |
| --- | --- | --- | --- | --- |
| [15:08] | RW | cfeb6_ly5_hcm[7:0] | 111111112 | 1=Enable DiStrip[7:0] Layer 5 |

#### Adr 17A	ADR_V6_EXTEND	DCFEB 7-bit extensions to 5 bit fields in 0x42, 0x68

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | cfeben 6 read | cfeben 5 read | cfeben 6 vme | cfeben 5 vme | inj mask 6 | inj mask 5 | inj febsel 6 | inj febsel 5 | mask all cfeb6 | mask all cfeb5 |

| Bits | Dir | Signal | Default | Description |
| --- | --- | --- | --- | --- |
| [01:00] | RW | mask_all[6:5] | 112 | Extend 0x42[4:0] = mask_all[4:0] |
| [03:02] | RW | inj_febsel[6:5] | 0 | Extend 0x42[9:5] = inj_febsel[4:0] |
| [05:04] | RW | injector_mask[6:5] | 112 | Extend 0x42[14:10] = injector_mask_cfeb[4:0] |
| [07:06] | RW | cfeb_en_vme[6:5] | 112 | Extend 0x68[14:10] = cfeb_en_vme[4:0] |
| [09:08] | R | cfeb_en[6:5] | 112 | Extend 0x68[14:10] = cfeb_en[4:0] readback |
| [15:10] | RW | -- | 0 | Unassigned |

### TTC Commands:

Fast Control Bus ccb_cmd[5..0] Decoding Scheme

| Signal | Code (hex) | Decoded by TMB | Description |
| --- | --- | --- | --- |
| BX0 (*) | 01 | Y | Bunch Crossing Zero |
| L1 Reset (*) | 03 | Y | Reset L1 readout buffers and resynchronize optical links |
| Hard_reset (*) | 04 |  | Reload all FPGAs from EPROMs |
| Start Trigger | 06 | Y | Go to trigger run , wait for bx0, may be disabled by Adr 2C[7] |
| Stop Trigger | 07 | Y | Go to stop state, wait for bx0, may be disabled by Adr 2C[7] |
| Test Enable | 08 |  |  |
| Private Gap | 09 |  |  |
| Private Orbit | 0A |  |  |
| Tmb_hard_reset (*) | 10 |  | Reload TMB FPGAs from EPROM |
| Alct_hard_reset (*) | 11 |  | Reload ALCT FPGAs from EPROM |
| Dmb_hard_reset (*) | 12 |  | Reload DMB FPGAs from EPROM |
| Mpc_hard_reset (*) | 13 |  | Reload MPC FPGAs from EPROM |
| Dmb_cfeb_calibrate0 (*) | 14 |  | CFEB Calibrate Pre-Amp Gain |
| Dmb_cfeb_calibrate1 (*) | 15 |  | CFEB Trigger Pattern Calibration |
| Dmb_cfeb_calibrate2 (*) | 16 |  | CFEB Pedestal Calibration |
| Dmb_cfeb_initiate (*) | 17 |  | Initiate CFEB calibration (Hold next L1ACC and Pretriggers) |
| Alct_adb_pulse_sync (*) | 18 |  | Pulse Anode Discriminator, synchronous |
| Alct_adb_pulse_async (*) | 19 |  | Pulse Anode Discriminator, asynchronous |
| Clct_external_trigger (*) | 1A |  | External Trigger All CLCTs |
| Alct_external_trigger (*) | 1B |  | External Trigger All ALCTs |
| Soft_reset (*) | 1C |  | Initializes the FPGA on DMB, TMB and MPC boards |
| DMB_soft_reset (*) | 1D |  | Initializes the FPGA on a DMB |
| TMB_soft_reset (*) | 1E |  | Initializes the FPGA on a TMB |
| MPC_soft_reset (*) | 1F |  | Initializes the FPGA on a MPC |
| Send_bcnt[7..0] (*) | 20 |  | Send Bunch_Counter[7..0] to ccb_data[7..0] bus |
| Send_evcnt[7..0] (*) | 21 |  | Send Event_Counter[7..0] to ccb_data[7..0] bus |
| Send_evcnt[15..8] (*) | 22 |  | Send Event_Counter[15..8] to ccb_data[7..0] bus |
| Send_evcnt[23..16] (*) | 23 |  | Send Event_Counter[23..16] to ccb_data[7..0] bus |
| Inject patterns from TMBs | 24 | Y | Injects patterns from TMB’s internal RAM to MPC |
| Alct_adb_pulse (*) | 25 |  | Generate sync and async anode discriminator pulses |
| Inject patterns from MPCs | 30 |  | Injects patterns from MPC’s input FIFO to SP |
| Inject patterns from MS | 31 |  | Injects patterns from MS input FIFO to Global Muon Trigger |
| tmb_bxreset | 32 | Y | Reset TMB/ALCT BXN, do not reset L1A counters |

(*) – decoded by CCB

# TMB Board Status Operations

()

### ID Registers

id_slot=15					VME Slot

id_rev =D					Firmware version

id_type=C					Firmware Type

id_date=06/08/2004			Firmware Compile Date

id_rev =38CA=06/10/04 xc2v3000	Firmware Revcode

### Digital Serial Numbers

Digital Serial for TMB CRC=DC DSN=00000A237E7F MFG=01 OK

Digital Serial for Mez CRC=BF DSN=000007E06194 MFG=01 OK

Digital Serial for RAT CRC=52 DSN=00000AB39AAD MFG=01 OK

### Power Supply ADC

TMB2005E Comparators

5.0V  status=OK

3.3V  status=OK

1.8V  status=OK

1.5V  status=OK

Tcrit status=OK

TMB2005E ADC

+5.0 TMB      5.004 V     0.305 A

+3.3 TMB      3.221 V     1.160 A

+1.5 TMBcore  1.488 V     0.795 A

+1.5 GTLtt    1.492 V     0.230 A

+1.0 GTLref   1.004 V     0.000 A

+3.3 RAT      3.221 V     0.250 A

+1.8 RATcore  1.797 V     8.985 A

+vref/2       2.047 V     0.000 A

+vzero        0.000 V     0.000 A

+vref         4.095 V     0.000 A

TMB2005E Temperature IC

T tmb pcb    73.4   F    23.    C  Tcrit=261./127.

T tmb fpga   95.0   F    35.    C  Tcrit=261./127.

RAT2005E Temperature IC

T rat pcb    68.0   F    20.    C  Tcrit=261./127.

T rat xstr   69.8   F    21.    C  Tcrit=261./127.

### Clock Delays

Current 3D3444 Delay Settings	02/27/2006

Ch0   8steps  16ns  ALCT  tx clock	alct_tof_delay in muonic firmware versions

Ch1   1steps   2ns  ALCT  rx clock 	not used in muonic firmware versions

Ch2   2steps   4ns  DMB   tx clock

Ch3   9steps  20ns  RPC   tx clock

Ch4   0steps   0ns  TMB1  rx clock 	not used in muonic firmware versions

Ch5   0steps   0ns  MPC   rx clock

Ch6   0steps   0ns  DCC   tx clock 	cfeb_tof_delay in muonic firmware versions

Ch7   7steps  14ns  CFEB0 tx clock

Ch8   7steps  14ns  CFEB1 tx clock

Ch9   7steps  14ns  CFEB2 tx clock

ChA   7steps  14ns  CFEB3 tx clock

ChB   7steps  14ns  CFEB4 tx clock

### JTAG Chains

Chain Select Address (X=don’t care)

3210	Base	Function

00SS	0	ALCT: SS=00(Slow user) SS=01(Slow prom) SS=10(Mez user) SS=11(Mez prom)

01XX	4	TMB Mezzanine FPGA+PROMs

10XX	8	TMB User PROMs

1100	C	FPGA Monitor (for TMB self-test)

1101	D	RAT Module FPGA+PROM

### RAT Module Status Register USER1

RAT FPGA device 0 Idcode=  20A10093

RAT PROM device 1 Idcode=  05024093

RAT FPGA device 0 USERcode=02232006

RAT PROM device 1 USERcode=02232006

RAT USER1=E00000007FFFFFFFFE02300336205650565E400CCC989C20060223EB

rs_begin          B

rs_version        E

rs_monthday       0223

rs_year           2006

rs_syncmode       0

rs_posneg         0

rs_loop           1

rs_rpc_en         3

rs_clk_active     0

rs_locked_tmb     1

rs_locked_rpc0    0

rs_locked_rpc1    0

rs_locklost_tmb   0

rs_locklost_rpc0  1

rs_locklost_rpc1  1

rs_txok           0

rs_rxok           0

rs_ntcrit         1

rs_rpc_free       0

rs_dsn            0

rs_dddoe_wr       3

rs_ddd_wr         0033

rs_ddd_auto       1

rs_ddd_start      0

rs_ddd_busy       0

rs_ddd_verify_ok  1

rs_rpc0_parity_ok 1

rs_rpc1_parity_ok 1

rs_rpc0_cnt_perr  0565

rs_rpc1_cnt_perr  0565

rs_last_opcode    02

rw_rpc_en         3

rw_ddd_start      0

rw_ddd_wr         0033

rw_dddoe_wr       3

rw_perr_reset     0

rw_parity_odd     1

rw_perr_ignore    0

rw_rpc_future     00

rs_rpc0_pdata     7FFFF

rs_rpc1_pdata     7FFFF

rs_unused         0000000

rs_end            E

### RAT Module Control Register USER2

RAT USER2=0118019B

ws_rpc_en      3

ws_ddd_start   0

ws_ddd_wr      0033

ws_dddoe_wr    3

ws_perr_reset  0

ws_parity_odd  1

ws_perr_ignore 0

ws_rpc_future  00

# User PROM Programming

TMB has two on-board  erasable PROMs that contain non-volatile data for modifying VME registers and for initializing JTAG chains.  The PROMs are read automatically by firmware state machines after power-up or hard-reset.

Both PROMs are Xilinx XC18V256 devices, which have a capacity of 256K bits stored 1-byte wide in 32K addresses.  Some late-production TMBs may have one or two XC18V512 devices, because the 256K product was discontinued by the manufacturer.

PROM contents are programmed and verified either via the front-panel JTAG connector or from the VME backplane using TMBs boot register. The data may be read out 8-wide though TMBs PROM-port register ADR_PROM (12h) or it may be read 1-bit serial via JTAG.

TDI    TDO

PROM 0

VME

Data

TDI

TDI    TDO

PROM 1

JTAG

Data

TDO

JTAG Chain

ID=8h

Mezzanine

FPGA

Shared 8-bit bus

Increment Address

### Register Initialization

Initialization after power-up or hard reset proceeds as follows:

1) FPGA loads its firmware from mezzanine PROMs (100msec)

2) Delay Locked Loops (DLLs) acquire lock on the 40MHz TTC clock (lock time not yet tested)

3) VME registers load their default values (100ns)

4) The VMEsm state machine reads PROM 0 and writes any new data to the specified VME registers.

5) JTAGsm state machine reads PROM 1 and write JTAG data to the specified chains.

### VME PROM-0

After TMB loads default values into its VME registers, the VMEsm state machine reads PROM 0 and updates the specified VME registers with PROM data.  The PROM contains a 16-word header, followed by an arbitrary number of 24-bit VME-addresses and their associated 16-bit data word, then followed by a 3-byte trailer sequence that contains the checksum and end-of-PROM marker.

The VMEsm state machine expects PROM-0 data to be organized in a specific format and it may either terminate the read-process or indicate an error condition if the format is incorrect.  If the BCh header-begin marker is missing, the state machine terminates reading immediately.

The 16-byte header consists of the BC “begin  CLCT data” marker, a 2-byte word count, and the EC end marker. There are no restrictions on the values of the other 12 header bytes.  VMEsm assembles the two word-count bytes into a 16-bit number that it uses to recognize when to stop reading the PROM.

The word-count includes every PROM address between the BC and FF markers, inclusive.  If there are n VME addresses to be written, the word count would be 16 header + 5n vme data + 3 trailer =1910 +5n. A counter in ADR_VMESM3 records the number of VME addresses what were actually written.  A value of 55Aah should be written to VME address ADR_VMESM4 to test the PROM-to-FPGA data path.

VME PROM-0 data format:

Adr	7654 3210	Hex	Description

---	---- ----   -- 	-----------

0	1011 1100	BC	Begin CLCT Header Marker, if missing state machine stops

1	tttt oooo	LL	Word count [7:0] kkkk hhhh tttt oooo from BC to FF

2	kkkk hhhh	HH	Word count [15:8]

3	tttt oooo	12	Month (month/day in “hex-ascii” December 31, 2006)

4	tttt oooo	31	Day

5	tttt oooo	06	Year	2006   = kkkk hhhh tttt oooo

6	kkkk hhhh	20	Year

7	vvvv vvvv	XX	Version number = vvvv vvvv

8	xxxx xxxx	AA	Option (suggest AA to test even bits)

9	xxxx xxxx	55	Option (suggest 55 to test odd  bits)

A	xxxx xxxx	XX	Option

B	xxxx xxxx	XX	Option

C	xxxx xxxx	XX	Option

D	xxxx xxxx	XX	Option

E	xxxx xxxx	XX	Option

F	1110 1100	EC	End Header Marker

10	aaaa aaaa	LL	VME adr[7:0]		This 5-byte structure is repeated

11	aaaa aaaa	MM	VME adr[15:8]		for every VME address that is

12	aaaa aaaa	HH	VME adr[23:16]	to be modified

13	dddd dddd	LL	VME data[7:0]

14	dddd dddd	HH	VME data[15:8]

L-2	1111 1100	FC	End of CLCT VME data Marker

L-1	cccc cccc	cc	Check sum  [7:0] includes addresses 0 to L-2

L	1111 1111	FF	End of PROM data Marker

If VMEsm detects an error condition, status information can be read from VME address

ADR_VMESM2 (0xDE):

fmt_err[0] = Missing BC header-begin marker

fmt_err[1] = Missing EC header-end marker

fmt_err[2] = Missing FC data-end marker

fmt_err[3] = Missing FF prom-end marker

fmt_err[4] = Word counter overflow

### JTAG PROM-1

After the VMEsm state machine completes successful, the JTAGsm machine is started.  It reads

PROM-1 and writes to the JTAG chains specified by the PROM data.  The PROM contains a 16-word header, followed by an arbitrary number of 3-byte Chain Blocks, then followed by a 8-byte trailer sequence that contains the tck-count, word-count, checksum and end-of-PROM marker.

Header

The JTAGsm state machine expects PROM-1 data to be organized in a specific format and it may either terminate the read-process or indicate an error condition if the format is incorrect.  If the Bah header-begin marker is missing, the state machine terminates reading immediately.

The 16-byte header consists of the BA “begin  ALCT data” marker and the EA end marker. The state machine skips over the next 15 bytes, so are no restrictions on the header contents.

Adr	7654 3210	Hex	Description

---	---- ----	--	-----------

0	10111010	BA	Begin ALCT Marker, if “BA” missing state machine stops

1	0000aaaa	03	ALCT  MSD 3	Type (288,384,672)

2	0000aaaa	08	ALCT      8

3	0000aaaa	04	ALCT  LSD 4

4	0000mmmm	00	Month MSD 0  in “hex-ascii” June 9, 2008

5	0000mmmm	06	Month LSD 6

6	0000dddd	00	Day   MSD 0

7	0000dddd	09	Day   LSD 9

8	0000yyyy	02	Year  MSD 2

9	0000yyyy	00	Year      0

A	0000yyyy	00	Year      0

B	0000yyyy	08	Year  LSD 8

C	0000vvvv	01	Version number [3:0]

D	0000xxxx	00	Future use

E	0000xxxx	00	Future use

F	00101010	EA	End ALCT Header Marker

Chain Block

A Chain Block marker “Cs” signals the start of a new sequence to generate TCK,TMS, and TDI for a JTAG chain.  The “s” in the “Cs” marker is the chain address SEL[3:0].  Normally, TCK will be held high after the last TMS/TDI pair is sent, unless the “C” marker is replaced by a “D” maker.  After Cs, there are two more bytes specifying the number of TCK clock pulses to send to the chain.

Following the TCK count are JTAG data bytes that contain packed TMS and TDI bits for up to 4 TCKs.  The number of data bytes is tck_count/4+1.

Chain Block Format [7:0]:

76543210

1100ssss	Cs	Chain Block begin marker chain address ssss =SEL[3:0]. Cs=hold tck high, Ds= do not

tttttttt	ww	TCK count [15:8]

tttttttt	ww	TCK count [7:0]

1sisisisi	si	JTAG data [7:0], I=TDI bit, s=TMS bit, defined below

4 TCKs packed per byte format, data[7:0]:

Bit	Signal

[0]	TDI[0]

[1]	TMS[0]

[2]	TDI[1]

[3]	TMS[1]

[4]	TDI[2]

[5]	TMS[2]

[6]	TDI[3]

[7]	TMS[3]

SEL[3:0] Selects the active JTAG chain:

SEL[3:0]

Hex	3210	Function

0	0000	ALCT Slow Control FPGA user registers

1	0001	ALCT Slow Control FPGA PROM)

2	0010	ALCT Mezzanine FPGA user registers

3	0011	ALCT Mezzanine FPGA PROMs

4	01XX	TMB Mezzanine FPGA+PROMs

8	10XX	TMB User PROMs

C	1100	TMB FPGA Monitor (for TMB self-test)

D	1101	RAT Module FPGA+PROM

Chain Block, Continued

The state machine copies the signals TMS, and TDI to the JTAG chain selected by SEL[3:0].  It automatically generates the JTAG clock TCK and also arranges for asserting the next TMS and TDI values while TCK is low.

TMB hardware pulls TCK, TMS, and TDI high for chains that are not currently selected.  When changing to a new chain ID, it is recommend that the last PROM word for that chain sets the signals to a logic high.

The number of Chain Blocks is limited only by the memory capacity of the PROM.  Each block begins with a Cs marker, and the last block is indicated by an “FA” marker.

Trailer

An 8-byte trailer sequence contains the total-tck-count, PROM word count, data checksum and end-of-PROM marker.  The tck-count is the number of TCKs the state machine sent to all chains. The word-count includes every PROM address between the BA and FA markers, inclusive, as well as the 3 TCK-count bytes.  Checksum includes every PROM address from the BA marker to the last word-count byte.

Adr	7654 3210	Hex	Description

---	---- ----	-- 	-----------

T-1	11111010	FC	End of JTAG data Marker	Also set chain address to C

T+0	tttttttt	tt	TCK Count Total [17:16]	Includes tcks sent for all chain blocks

T+1	tttttttt	tt	TCK Count Total [15:8]

T+2	tttttttt	tt	TCK Count Total [7:0]

T+3	wwwwwwww	ww	Word Count [15:8]		Includes Adr 0 and end JTAG marker at T+2

T+4	wwwwwwww	ww	Word Count [7:0]

T+5	cccccccc	cc	Check sum  [7:0]		Includes addresses 0 and T+5

T+6	11111111	FF	End of PROM data Marker

If the tck-count, word-count or checksum are incorrect, or if the last-word marker is missing, the state machine indicates the an error by setting jsm_tck_fpga_ok=0.

TCK Throttle

The JTAGsm state machine reads header and trailer bytes from the PROM at 20MHz, then slows to 2.5MHz for processing JTAG data bytes.  This results in TCK being pulsed at 10MHz (because there are 4 TCKs per byte, each 50ns high + 50ns low).

If this rate is too high for ALCT JTAG chains, the state machine can be throttled to a lower speed by setting jsm_throttle[3:0] in ADR_VMESM0 (D4h).  A jsm_throttle value of 0 corresponds firing TCK at 10MHz full speed. Increasing jsm_throttle by ‘n’ increases the TCK period by 25ns*n, and maintains a 50% duty cycle.

State Machine Status

Automatic operation of the state machine can be verified by including TCK writes to chain address C.  That chain effectively loops-back TCK, TMS and TDI to the FPGA.  If at least 1 TCK transition is seen on chain C, then signal jsm_tck_fpga_ok  will be a logic 1 in VME register ADR_JTAGSM0, and tck_fpga_cnt[3:0] in ADR_JTAGSM2 counts the number of TCKs.  Demo software exists that converts old PROM data files to this new format and automatically inserts a C-chain Block if there is not one already.

JTAG PROM-1 data format Example:

Adr	7654 3210	Hex	Description

---	---- ----   -- 	-----------

0	10111010	BA	Begin ALCT Marker, if “BA” missing state machine stops

1	0000aaaa	03	ALCT  MSD 3	Type (288,384,672)

2	0000aaaa	08	ALCT      8

3	0000aaaa	04	ALCT  LSD 4

4	0000mmmm	00	Month MSD 0  in “hex-ascii” June 9, 2008

5	0000mmmm	06	Month LSD 6

6	0000dddd	00	Day   MSD 0

7	0000dddd	09	Day   LSD 9

8	0000yyyy	02	Year  MSD 2

9	0000yyyy	00	Year      0

A	0000yyyy	00	Year      0

B	0000yyyy	08	Year  LSD 8

C	0000vvvv	01	Version number [3:0]

D	0000xxxx	00	Future use

E	0000xxxx	00	Future use

F	00101010	EA	End ALCT Header Marker

10	1100ssss	C3	Chain Block Markder for chain adr 3 (or Ds to set TCK low)

11	tttttttt	ww	TCK count [15:8]

12	tttttttt	ww	TCK count [7:0]

13	sisisisi	si	JTAG data

14	sisisisi	si	JTAG data

15	sisisisi	si	JTAG data

T-1	11111010	FC	End of JTAG data Marker

T+0 tttttttt	tt	TCK Count Total [17:16]	Includes tcks sent for all chain blocks

T+1	tttttttt	tt	TCK Count Total [15:8]

T+2	tttttttt	tt	TCK Count Total [7:0]

T+3 wwwwwwww	ww	Word Count [15:8]		Includes Adr 0 and end JTAG marker at T+2

T+4 wwwwwwww	ww	Word Count [7:0]

T+5	cccccccc	cc	Check sum  [7:0]		Includes addresses 0 and T+5

T+6	11111111	FF	End of PROM data Marker

# DMB Readout

### Full-Readout and Local-Readout Format (Long Header):

1	DB0C	header		Beginning Of Cathode Data

7	 event	header		Non-buffered event data

e	 clct	header		Cathode LCTs

e	 tmb	header		TMB match result

e	 mpc	header		MPC frames

e	 rpc	header		RPC status

e	 buf	header		Buffer status

e	6E0B	header		End of header block

n	 hits			CFEB0 raw hits

n	 hits			CFEB1 raw hits

n	 hits			CFEB2 raw hits

n	 hits			CFEB3 raw hits

n	 hits			CFEB4 raw hits

1	6B04			Start of RPC raw hits marker	(optional)

m	hits			RPC0 raw hits			(optional)

m	hits			RPC1 raw hits			(optional)

1	6E04			End of RPC raw hits marker	(optional)

1	6B05			Beginning scope data		(optional)

s	scope data		Scope data 			(optional)

1	6E05			End of scope data 		(optional)

1	6B07			Beginning miniscope data		(optional)

22	miniscope data		Miniscope data 22 words		(optional)

1	6E07			End of miniscope data 		(optional)

1	6BCB			Beginning blocked CFEB list	(optional)

20	blocked cfebs list		Blocked CFEBs list 20 words	(optional)

1	6ECB			End blocked CFEB list		(optional)

1	6E0C			End of raw hits

1	2AAA			Make word count x4		(inserted only if needed)

1	5555			Make word count x4		(inserted only if needed)

1	DE0F			End of Frame

1	Dcrc0			CRC22[10:0]

1	Dcrc1			CRC22[21:11]

1	Dwordcount		Total words in transmission	(inclusive)

------

Word Count =	   42(nheaders)

+ 1(E0B)

+ ncfebs*(6*ntbins)

+1(B04)			(if RPC readout enabled)

+ nrpcs*(2*ntbins) 	(if RPC readout enabled)

+1(E04)			(if RPC readout enabled)

+1(B05)			(if Scope readout enabled)

+nch(128)/16*256	(if Scope readout enabled)

+1(E05)			(if Scope readout enabled)

+1(B07)			(if miniscope readout enabled)

+22			(if miniscope readout enabled)

+1(E07)			(if miniscope readout enabled)

+1(BCB)		(if blocked cfebs list readout enabled)

+ncfebs*4		(if blocked cfebs list readout enabled)

+1(ECB)		(if blocked cfebs list readout enabled)

+1(EOC)

+2(2AAA 5555)		(if needed to make word count multiple of 4)

+1(E0F)

+2(crc)

+1(wordcount)

### Long Header-only Format:

1	DB0C	header 		Beginning of Cathode Data

7	 event	header		Non-buffered event data

e	 clct	header		Cathode LCTs

e	 tmb	header		TMB match result

e	 mpc	header		MPC frames

e	 buf	header		Buffer status

e	 rpc	header		RPC status

1	6E0B			End of header block

1	6E0C			End of raw hits

1	DE0F			End of Frame

1	Dcrc0			CRC22

1	Dcrc1			CRC22

1	Dwordcount		Total words in transmission (inclusive)

------

40 words = nheaders(34)+E0B+E0C+2crc+E0F+wdcnt

### Short Header-only Format:

1	DB0C	header		Beginning of Cathode Data

7	 event	header		Non-buffered event data

1	DEEF			End of Frame

1	Dcrc0			CRC22

1	Dcrc1			CRC22

1	Dframe wordcount	Total words in transmission (inclusive)

------

12 words = 8headers+2crc+EEF+wdcnt

### Miniscope:

TMB contains a smaller version of the main digital scope that is intended to be included in the DMB readout stream by default.  The minscope displays a time history of  ALCTs, CLCTs, and L1As that helps reproduce TMB behavior offline.

Miniscope data is written continuously to Block RAM, in the same fashion as CFEB raw hits are stored.

When there is a CLCT pre-trigger, miniscope data starts at fifo_pretrig_mini time bins before the pre-trigger occurred.  If there is no CLCT pre-trigger, mininscope data can still be read out in an L1A-only event, using the L1A look-back mode.

The miniscope is enabled by setting:

0x10C [00]    mini_read_enable = 1  Turn miniscope on

0x10C [01]    mini_tbins_test  = 1  Turn debug mode off

0x10C [02]    mini_tbins_word  = 1  Insert tbins and pre-trig tbins settings in 1st word

0x10C [07:03] fifo_tbins_mini  = 22	Number of time bins to read out, must be a multiple of 2, but not of 4

0x10C [12:08] fifo_pretrig_mini = 4 Number of time bins before pre-trigger

Once enabled, miniscope data is included in every event readout that contains a full header.

Events the contain miniscope data will have

1 )  header19[14] mini_read_enable = 1 to indicate the event includes miniscope data.

2 ) 06B07  begin minscope marker in the data stream, located after RPC and main-scope data

3)  Typically 22 miniscope time-bin data words

4) 06E07  end miniscope marker

The number of minscope time-bin data words depends on the VME register 0x10C[7:3].

Because the total number of words in the DMB readout is required to be a multiple of 4, and 2 words are used for 06E07|06E07 markers, the number of miniscope time-bin words must be a multiple of 2, but not a multiple of 4.

The first word after the 06B07 marker contains the number of minscope time-bins, which along with the mini_read_enable bit in header19[14] allows event-unpacking software to locate the miniscope data markers and predict the total event word count.

Miniscope data is inserted between RPC and Blocked-bits list

Adr=  282 Data= 06E04

Adr=  283 Data= 06B07	  begin miniscope marker

Adr=  284 Data= 00416	  tbin count 0x16 and tbins before pre-trigger 0x04

Adr=  285 Data= 00002	  14 minicope channels at tbin=1

Adr=  286 Data= 00002

Adr=  287 Data= 00002

Adr=  288 Data= 00403

Adr=  289 Data= 01805

Adr=  290 Data= 01009

Adr=  291 Data= 01009

Adr=  292 Data= 02009

Adr=  293 Data= 00009

Adr=  294 Data= 00158

Adr=  295 Data= 00302

Adr=  296 Data= 00102

Adr=  297 Data= 00002

Adr=  298 Data= 00002

Adr=  299 Data= 00002

Adr=  300 Data= 00002

Adr=  301 Data= 00002

Adr=  302 Data= 00002

Adr=  303 Data= 00002

Adr=  304 Data= 00002

Adr=  305 Data= 00002

Adr=  306 Data= 06E07	 end miniscope marker

Adr=  307 Data= 06BCB	 blocked bits marker, if blocked bits are in the readout

Miniscope Channel Assignments:

ch[00]	any_cfeb_hit	At least 1 CFEB meets pre-trigger layer-hit threshold

ch[03:01]	clct_sm_vec[2:0]	CLCT pre-trigger state machine vector

ch[04]	clct0_vpf		Valid pattern flag for 1st  best CLCT

ch[05]	clct1_vpf		Valid pattern flag for 2nd best CLCT

ch[06]	alct0_vpf_tprt	Valid pattern flag for 1st  best ALCT, after pipe delay

ch[07]	alct1_vpf_tprt	Valid pattern flag for 2nd best ALCT, after pipe delay

ch[08]	clct_window		CLCT match window

ch[09]	wr_push_rtmb	ALCT*CLCT match signal

ch[10]	tmb_push_dly	L1A signals are l1a_delay+2bx later

ch[11]	l1a_pulse		L1A from CCB

ch[12]	l1a_window_open	L1A window

ch[13]	l1a_push_me		L1A queued for readout signal

ch[14]	tmb_special		Always 0 in DMB readout, required by unpacker

ch[15]	ddu_special		Always 0 in DMB readout, required by DDU

CLCT Pre-trigger State Machine Vector:

0   s:    Startup wait after hard-reset

1   I:	Idle, waiting for pre-trigger

2   p:	Pre-triggered

3   f:	Flushing triad one-shots	[checks any_cfeb_hit, waits n-bx, returns to idle, n may be 0,  n=1 here]

4   t:	Trigger-rate throttle		[ optional fixed delay before returning to idle]

5   h:	Halted

Decoded Readout Example:

000000000000000111111		 Time bins

123456789ABCDEF012345

ch 00  any_cfeb_hit       ___------____________		 CLCT signals are in pre-trigger time domain

ch 03  clct_state_machine 111124444411111111111

ch 03  clct_state_machine iiiipfffffiiiiiiiiiii		 CLCT Pre-trigger machine state

ch 04  clct0_vpf          _________-___________

ch 05  clct1_vpf          _____________________

ch 06  alct0_vpf          _________-___________		 ALCT arrival is needed to check TMB offline

ch 07  alct1_vpf          _____________________

ch 08  clct_window        _________---_________

ch 09  wr_push_rtmb       __________-__________		 ALCT*CLCT match signal

ch 10  tmb_push_dly       ___-_________________		 L1A signals are l1a_delay+2bx later

ch 11  l1a_pulse          ____-________________		 L1A from CCB merged with other L1A

ch 12  l1a_window_open    ____---______________

ch 13  l1a_push_me        _______-_____________		 L1A queued for readout signal

ch 14  tmb_special        _____________________		 Unpacker prevents TMB from using 15th bit

ch 15  ddu_special        _____________________		 DDU prevents TMB from using 16th bit

### Blocked CFEB DiStrips List Format:

1	6BCB   marker		Beginning of blocked cfeb distrip list

1	CFEB0 word 0		[14:12]=cfebid[2:0]=0   [11:0]=cfeb0 distrips[11:0]

1	CFEB0 word 1		[14:12]=cfebid[2:0]=0   [11:0]=cfeb0 distrips[23:12]

1	CFEB0 word 2		[14:12]=cfebid[2:0]=0   [11:0]=cfeb0 distrips[35:24]

1	CFEB0 word 3		[14:12]=cfebid[2:0]=0   [11:0]=cfeb0 distrips[47:36]

1	CFEB1 word 0		[14:12]=cfebid[2:0]=1   [11:0]=cfeb1 distrips[11:0]

1	CFEB1 word 1		[14:12]=cfebid[2:0]=1   [11:0]=cfeb1 distrips[23:12]

1	CFEB1 word 2		[14:12]=cfebid[2:0]=1   [11:0]=cfeb1 distrips[35:24]

1	CFEB1 word 3		[14:12]=cfebid[2:0]=1   [11:0]=cfeb1 distrips[47:36]

1	CFEB2 word 0		[14:12]=cfebid[2:0]=2   [11:0]=cfeb2 distrips[11:0]

1	CFEB2 word 1		[14:12]=cfebid[2:0]=2   [11:0]=cfeb2 distrips[23:12]

1	CFEB2 word 2		[14:12]=cfebid[2:0]=2   [11:0]=cfeb2 distrips[35:24]

1	CFEB2 word 3		[14:12]=cfebid[2:0]=2   [11:0]=cfeb2 distrips[47:36]

1	CFEB3 word 0		[14:12]=cfebid[2:0]=3   [11:0]=cfeb3 distrips[11:0]

1	CFEB3 word 1		[14:12]=cfebid[2:0]=3   [11:0]=cfeb3 distrips[23:12]

1	CFEB3 word 2		[14:12]=cfebid[2:0]=3   [11:0]=cfeb3 distrips[35:24]

1	CFEB3 word 3		[14:12]=cfebid[2:0]=3   [11:0]=cfeb3 distrips[47:36]

1	CFEB4 word 0		[14:12]=cfebid[2:0]=4   [11:0]=cfeb4 distrips[11:0]

1	CFEB4 word 1		[14:12]=cfebid[2:0]=4   [11:0]=cfeb4 distrips[23:12]

1	CFEB4 word 2		[14:12]=cfebid[2:0]=4   [11:0]=cfeb4 distrips[35:24]

1	CFEB4 word 3		[14:12]=cfebid[2:0]=4   [11:0]=cfeb4 distrips[47:36]

1	6ECB  marker		End of blocked cfeb distrip list

------

22 words

Blocked DiStrips list includes:

DiStrips turned off via VME Hot Channel Mask

DiStrips turned off via mask_all applied to the entire CFEB

DisStrips marked as bad by automatic bad-bits detection

CFEB DiStrip Bit Packing:

Each CFEB has 6 layers of 8 DiStrips = 48 bits

block_distrip_list[ 7: 0] = Layer0 Ds[7:0]

block_distrip_list[15: 8] = Layer1 Ds[7:0]

block_distrip_list[23:16] = Layer2 Ds[7:0]

block_distrip_list[31:24] = Layer3 Ds[7:0]

block_distrip_list[39:32] = Layer4 Ds[7:0]

block_distrip_list[47:40] = Layer5 Ds[7:0]

Which are packed into 4 readout words, 12 bits per word = 48 bits

CFEBn word 0  = block_distrip_list[11: 0]

CFEBn word 1  = block_distrip_list[23:12]

CFEBn word 2  = block_distrip_list[35:24]

CFEBn word 3  = block_distrip_list[47:36]

### Header Word Descriptions:

First 4 header words must conform to DDU format specification:

header00_[11:0] 	12’hB0C			Beginning of Cathode record marker

header00_[14:12]	3’b101				DDU code for TMB/ALCT

header00_[15]		1				DDU special-word flag

header01_[11:0]	pop_bxn_counter[11:0] 	BXN pushed into L1A queue at L1A arrival

header01_[14:12]	3’b101				DDU code for TMB/ALCT

header01_[15]		1				DDU special-word flag

header01 notes:

- bxn_counter contains the value of the 12-bit BXN counter at the time

L1A arrives, and is typically 128bx later than the pre-trigger BXN (see header08).

Readouts will always have bxn_counter, but may or may not have pre-trigger data.

header02_[11:0]	pop_l1a_rx_counter[11:0]	L1As received by TMB

header02_[14:12]	3’b101				DDU code for TMB/ALCT

header02_[15]		1				DDU special-word flag

header03_[11:0]	readout_counter[11:0]		Counts L1A readouts

header03_[14:12]	3’b101				DDU code for TMB/ALCT

header03_[15]		1				DDU special-word flag

Next 4 words for short-header mode or full-header:

header04_[4:0]	board_id[4:0]			TMB module ID number = VME slot number 1-20

header04_[8:5]	csc_id[3:0]			Chamber ID number, set by VME register

header04_[12:9]	run_id[3:0]			Run info, set by VME register

header04_[13]	=	buf_q_ovf_err			Tried to push new event when queue full

header04_[14]	=	r_sync_err			BXN sync error

header04 notes:

- board_id defaults to the VME crate slot number, unless overridden via VME.

(2)	csc_id is a user-set value to identify the CSC connected to this TMB

(3) 	run_id is a user-set value to identify the current data run.

(4)	buf_q_ovf indicates that more L1As arrived than TMB was able to push into its

readout processing queue.  In this case the average trigger rate is probably higher than the

readout data path can tolerate.

(5)	sync_err indicates that bx0 did not arrive when the BXN counter turned

over to the bxn-preset value.  Either bx0 is not functioning, or the 40 MHz clock

gained or lost counts.  A sync_err is latched-on until ttc_resync or ttc_bxreset.

header05_[5:0]	r_nheaders[5:0]		Number of header words

header05_[8:6]	fifo_mode[2:0]		Raw hits fifo readout mode set via VME

header05_[10:9]	r_type[1:0]			Record type: dump, nodump, full header, short header

header05_[12:11]	l1a_type[1:0]			L1A Pop type code: buffers, no buffers, clct/alct_only

header05_[13]		r_has_buf			Event has clct and rpc buffer data

header05_[14]		r_buf_stalled			Buffer write pointer hit a fence and stalled

header05 notes:

- nheaders indicates the length of the current header block, including the BOC marker to

1 frame before the EOB marker.

In current firmware it will be 8 for short headers and 42 for full.

(2) FIFO Modes:

mode	raw hits		header

0	no		full 	(if buffer was available at pre-trigger)

1	all 5 CFEBs 	full	(if buffer was available at pre-trigger)

2	local		full 	(if buffer was available at pre-trigger), local=sparsified cfebs

3	no		short

4	no		no

(3) Record Type Codes:

r-type	raw hits		header

0	no		full

1	full		full

2	local		full

3	no		short	(no buffer was available at pre-trigger)

(4) L1A Type Codes:

l1a-type

0	Normal CLCT trigger with buffer data and L1A window match

1	ALCT-only trigger, no data buffers 				(not usually read out)

2	L1A-only, no matching TMB trigger, no buffer data		(not usually read out)

3	TMB triggered, no L1A-window match, event has buffer data	(not usually read out)

header06_[14:0]	bd_status[14:0]		Board status summary

header06 notes:

(1)		bd_status[ 0]	bd_status_ok		Board all-OK: voltages OK, temperature OK, prom-load OK

bd_status[ 1]	vstat_5p0v		Voltage Comparator +5.0V, 1=OK

bd_status[ 2]	vstat_3p3v		Voltage Comparator +3.3V, 1=OK

bd_status[ 3]	vstat_1p8v		Voltage Comparator +1.8V, 1=OK

bd_status[ 4]	vstat_1p5v		Voltage Comparator +1.5V, 1=OK

bd_status[ 5]	*t_crit			Temperature ADC Tcritical 1=OK

bd_status[ 6]	vsm_ok			VME Machine ran without errors

bd_status[ 7]	vsm_aborted		VME	State machine aborted reading PROM

bd_status[ 8]	vsm_cksum_ok		VME	Check-sum  matches PROM contents

bd_status[ 9]	vsm_wdcnt_ok		VME	Word count matches PROM contents

bd_status[10]	jsm_ok			JTAG state machine completed without errors

bd_status[11]	jsm_aborted		JTAG	State machine aborted reading PROM

bd_status[12]	jsm_cksum_ok		JTAG	Check-sum  matches PROM contents

bd_status[13]	jsm_wdcnt_ok		JTAG	Word count matches PROM contents

bd_status[14]	jsm_tck_fpga_ok		FPGA     jtag tck detected correctly

header07_[14:0]	revcode[14:0]			Firmware version date code

header07 notes:

(1)		revcode[04:00]	day 1-31

revcode[08:05]	month 1-12

revcode[12:09]	years after 2000

revcode[14:13]	fpga type, 1 for xc2v3000, 2 for xc2v4000

Full Header-mode words 8-EOB: Event Counters

header08_[11:0]	r_bxn_counter_ff[11:0]	CLCT Bunch Crossing number at pre-trigger, 0-3563

header08_[12]		r_tmb_clct0_discard;		TMB discarded clct0 from ME1A

header08_[13]		r_tmb_clct1_discard;		TMB discarded clct1 from ME1A

header08_[14]		clock_lock_lost		Main DLL clock lost lock

header09_[14:0]	r_pretrig_counter[14:0]	Counts CLCT pre-triggers [stops on ovf]

header10_[14:0]	r_pretrig_counter[29:15]

header11_[14:0]	r_clct_counter[14:0]		Counts CLCTs post-drift [stops on ovf]

header12_[14:0]	r_clct_counter[29:15]

header13_[14:0]	r_trig_counter[14:0]		Counts TMB triggers to MPC, L1A request to CCB,

header14_[14:0]	r_trig_counter[29:15]		[stops on ovf]

header15_[14:0]	r_alct_counter[14:0]		Counts ALCTs received from ALCT board [stops on ovf]

header16_[14:0]	r_alct_counter[29:15]

header17_[14:0]	r_orbit_counter[14:0]		BX0s since last hard reset [stops on ovf]

header18_[14:0]	r_orbit_counter[29:15]

CLCT Raw Hits Size:

header19_[2:0]	r_ncfebs[2:0]			Number of CFEBs read out

header19_[7:3]	r_fifo_tbins[4:0]		Number of time bins per CFEB in dump

header19_[12:8]	fifo_pretrig[4:0]		Number of time bins before pre-trigger

header19_[13]		scope_data_exists		Readout includes logic analyzer scope data

header19_[14]		mini_read_enable		Readout includes miniscope data, 22wds+2markers

CLCT Configuration:

header20_[2:0]	hit_thresh_pretrig[2:0]	Hits on pattern template pre-trigger threshold

header20_[6:3]	pid_thresh_pretrig[3:0]	Pattern shape ID pre-trigger threshold

header20_[9:7]	hit_thresh_postdrift[2:0]	Hits on pattern post-drift threshold

header20_[13:10]	pid_thresh_postdrift[3:0]	Pattern shape ID post-drift threshold

header20_[14]		stagger_hs_csc		CSC Staggering ON

header21_[3:0]	triad_persist[3:0]		CLCT Triad persistence

header21_[6:4]	dmb_thresh_pretrig[2:0]	DMB pre-trigger threshold for active-cfeb list

header21_[10:7]	alct_delay[3:0]		Delay ALCT for CLCT match window

header21_[14:11]	clct_window[3:0]		CLCT match window width

CLCT Trigger Status:

header22_[8:0]	r_trig_source_vec[8:0]	Pre-trigger source vector

header22_[14:9]	r_layers_hit_vec[5:0]		CSC layers hit on layer trigger

header22 notes:

(1)	trig_source [ 0]	CLCT pattern triggered sequencer

trig_source [ 1]	ALCT pattern triggered sequencer

trig_source [ 2]	ALCT*CLCT pattern triggered sequencer

trig_source [ 3]	ADB external triggered sequencer

trig_source [ 4]	DMB external triggered sequencer

trig_source [ 5]	CLCT (CCB scintillator) external triggered sequencer

trig_source [ 6]	ALCT (CCB) external triggered sequencer

trig_source [ 7]	VME triggered sequencer

trig_source [ 8]	Layer-mode trigger

header23_[4:0]	r_active_feb_mux[4:0]	Active CFEB list sent to DMB

header23_[9:5]	r_cfebs_read[4:0]		CFEBs read out for this event

header23_[13:10]	pop_l1a_match_win[3:0]	Position of l1a in window

header23_[14]		active_feb_src			Active CFEB list source, 0=pretrig, 1=at TMB match

CLCT+ALCT Match Status:

header24_[0]		r_tmb_match			ALCT and CLCT matched in time, pushed into L1A queue

header24_[1]		r_tmb_alct_only		Only ALCT triggered, pushed into L1A queue

header24_[2]		r_tmb_clct_only		Only CLCT triggered, pushed into L1A queue

header24_[6:3]	r_tmb_match_win[3:0]	Location of alct in clct window, pushed into L1A queue

header24_[7]		r_no_alct_tmb;		No ALCT

header24_[8]		r_one_alct_tmb;		One ALCT

header24_[9]		r_one_clct_tmb;		One CLCT

header24_[10]		r_two_alct_tmb;		Two ALCTs

header24_[11]		r_two_clct_tmb;		Two CLCTs

header24_[12]		r_dupe_alct_tmb;		ALCT0 copied into ALCT1 to make 2nd LCT

header24_[13]		r_dupe_clct_tmb;		CLCT0 copied into CLCT1 to make 2nd LCT

header24_[14]		r_rank_err_tmb;		LCT1 has higher quality than LCT0, error

CLCT Trigger Data:

header25_[14:0]	r_clct0_tmb[14:0]		CLCT0 after drift lsbs

header26_[14:0]	r_clct1_tmb[14:0]		CLCT1 after drift lsbs

header27_[0]		r_clct0_tmb[15]		CLCT0 after drift msbs

header27_[1]		r_clct1_tmb[15]		CLCT1 after drift msbs

header27_[4:2]	r_clctc_tmb[2:0]		CLCT0/1 common after drift msbs

header27_[5]		r_clct0_invp			CLCT0 had invalid pattern after drift delay

header27_[6]		r_clct1_invp			CLCT1 had invalid pattern after drift delay

header27_[7]		r_clct1_busy			2nd CLCT busy, logic error indicator

header27_[12:8]	perr_cfeb_ff[4:0]		CFEB raw hits RAM parity error, latched, SEU detection

header27_[13]	0	perr_rpc_ff | perr_mini_ff	RPC   raw hits RAM parity error, latched, SEU detection

header27_[14]	0	perr_ff				Raw hits RAM parity error summary, latched, SEU

header25-27 notes:

- clct0, clct1, clctc packing format:

clct0[0]		clct_1st_valid		Valid pattern flag

clct0[3:1]	hs_hit_1st[2:0]		Hits on pattern 0-6

clct0[7:4]	hs_pid_1st[3:0]		Pattern shape 0-A

clct0[15:8]	hs_key_1st[7:0]		½-strip ID number

clct1[0]		clct_2nd_valid		Valid pattern flag

clct1[3:1]	hs_hit_2nd[2:0]		Hits on pattern 0-6

clct1[7:4]	hs_pid_2nd[3:0]		Pattern shape 0-A

clct1[15:8]	hs_key_2nd[7:0]		½-strip ID number

clctc[1:0]	bxn_counter_ff[1:0]	Bunch crossing number at pretrigger, common to clct0/1

clctc[2]		sync_err			BX0 disagrees with BXN count, common to clct0/1

ALCT Trigger Data:

header28_[0]		alct_1st_valid			ALCT0 valid pattern flag

header28_[2:1]	alct_1st_quality[1:0]		ALCT0 quality

header28_[3]		alct_1st_amu			ALCT0 accelerator muon flag

header28_[10:4]	alct_1st_key[6:0]		ALCT0 key wire group

header28_[14:11]	alct_pretrig_win[3:0]		ALCT active_feb_flag position in pretrig window

header29_[0]		alct_2nd_valid			ALCT1 valid pattern flag

header29_[2:1]	alct_2nd_quality[1:0]		ALCT1 quality

header29_[3]		alct_2nd_amu			ALCT1 accelerator muon flag

header29_[10:4]	alct_2nd_key[6:0]		ALCT1 key wire group

header29_[12:11]	drift_delay[1:0]		CLCT drift delay

header29_[13]		bcb_read_enable		CFEB blocked DiStrip bits list included in readout

header29_[14]		hs_layer_trig			Layer-mode trigger

header30_[4:0]	alct_bxn[4:0]			ALCT0/1 bxn

header30_[6:5]	alct_ecc_err[1:0]		ALCT trigger path ECC error code

header30_[11:7]	cfeb_badbits_found[4:0]	Bad distrip bits detected in cfeb[n]

header30_[12]		cfeb_badbits_blocked		At least one CFEB has a bad bit that was blocked

header30_[13]		alct_cfg_done			ALCT FPGA configuration done

header30_[14]		bx0_match			alct_bx0==clct_bx0, latched at clct_bx0 time

MPC Frames:

header31_[14:0]	r_mpc0_frame0_ff[14:0]	MPC muon 0 frame 0 LSBs

header32_[14:0]	r_mpc0_frame1_ff[14:0]	MPC muon 0 frame 1 LSBs

header33_[14:0]	r_mpc1_frame0_ff[14:0]	MPC muon 1 frame 0 LSBs

header34_[14:0]	r_mpc1_frame1_ff[14:0]	MPC muon 1 frame 1 LSBs

header35_[0]	=	r_mpc0_frame0_ff[15]	MPC muon 0 frame 0 MSB

header35_[1]	=	r_mpc0_frame1_ff[15]	MPC muon 0 frame 1 MSB

header35_[2]	=	r_mpc1_frame0_ff[15]	MPC muon 1 frame 0 MSB

header35_[3]	=	r_mpc1_frame1_ff[15]	MPC muon 1 frame 1 MSB

header35_[7:4]	mpc_tx_delay[3:0]		MPC transmit delay

header35_[9:8]	r_mpc_accept[1:0]		MPC muon accept response

header35_[14:10]	cfeb_en[4:0]			CFEBs enabled for triggering (didn’t fit elsewhere)

header31-35 notes:

- MPCframe packing format:

mpc0_frame0[6:0]	=	alct0_key[6:0];

mpc0_frame0[10:7]	=	clct0_pat[3:0];

mpc0_frame0[14:11]	=	lct0_quality[3:0];

mpc0_frame0[15]	=	lct0_vpf;

mpc0_frame1[7:0]	=	{clct0_cfeb[2:0],clct0_key[4:0]};

mpc0_frame1[8]		=	clct0_bend;

mpc0_frame1[9]		=	clct_sync_err & tmb_sync_err_en[0];

mpc0_frame1[10]	=	alct0_bxn[0];

mpc0_frame1[11]	=	clct_bx0;     // bx0 gets replaced after mpc_tx_delay, keep here to mollify xst

mpc0_frame1[15:12]	=	csc_id[3:0];

mpc1_frame0[6:0]	=	alct1_key[6:0];

mpc1_frame0[10:7]	=	clct1_pat[3:0];

mpc1_frame0[14:11]	=	lct1_quality[3:0];

mpc1_frame0[15]	=	lct1_vpf;

mpc1_frame1[7:0]	=	{clct1_cfeb[2:0],clct1_key[4:0]};

mpc1_frame1[8]		=	clct1_bend;

mpc1_frame1[9]		=	clct_sync_err & tmb_sync_err_en[1];

mpc1_frame1[10]	=	alct1_bxn[0];

mpc1_frame1[11]	=	alct_bx0;     // bx0 gets replaced after mpc_tx_delay, keep here to mollify xst

mpc1_frame1[15:12]	=	csc_id[3:0];

RPC Configuration:

header36_[1:0]	rd_list_rpc[1:0]		RPCs included in read out

header36_[3:2]	r_nrpcs_read[1:0]		Number of RPCs in readout, 0,1,2, 0 if header-only

header36_[4]	=	rpc_read_enable		RPC readout enabled

header36_[9:5]	fifo_tbins_rpc[4:0]		Number RPC FIFO time bins to read out

header36_[14:10]	fifo_pretrig_rpc[4:0]		Number RPC FIFO time bins before pretrigger

Buffer Status:

header37_[10:0]	r_wr_buf_adr[10:0]		Buffer RAM write address at pretrigger

header37_[11]		r_wr_buf_ready		Write buffer was ready at pretrig

header37_[12]		wr_buf_ready			Write buffer ready now

header37_[13]		buf_q_full			All raw hits ram in use, ram writing must stop

header37_[14]		buf_q_empty			No fences remain on buffer stack

header38_[10:0]	r_buf_fence_dist[10:0]	Distance to 1st fence address at pretrigger

header38_[11]		buf_q_ovf_err			Tried to push when stack full

header38_[12]		buf_q_udf_err			Tried to pop when stack empty

header38_[13]		buf_q_adr_err			Fence adr popped from stack doesn’t match rls adr

header38_[14]		buf_stalled_once		Buffer stalled at least once since last resync

header39_[11:0]	buf_fence_cnt[11:0]		Number of fences in fence RAM currently

header39_[12]		reverse_hs_csc		1=Reverse staggered CSC, non-me1

header39_[13]		reverse_hs_me1a		1=ME1A hstrip order reversed

header39_[14]		reverse_hs_me1b		1=ME1B hstrip order reversed

header40_[1:0]	active_feb_mux[6:5];		Extend Hdr23[4:0]    Active CFEB list sent to DMB

header40_[3:2]	r_cfebs_read[6:5];		Extend Hdr23[9:5]     CFEBs read out for this event

header40_[5:4] 	perr_cfeb_ff[6:5];		Extend Hdr27[12:8]   CFEB RAM parity error, latched

header40_[7:6] 	cfeb_badbits_found[6:5];	Extend Hdr30[11:7]   CFEB[n] has at least 1 bad bit

header40_[9:8]	cfeb_en[6:5];			Extend Hdr35[14:10] CFEBs enabled for triggering

header40_[10]		buf_fence_cnt_is_peak;	Current fence is peak number of fences in RAM

header40_[11]		(MXCFEB==7);		TMB has 7 DCFEBs so hdr40_[10:0] are active

header40_[13:12]	r_trig_source_vec[10:9]	Pre-trigger source vector for ME1A/B

header40_[14]		r_tmb_trig_pulse		TMB trig_pulse signal matched rtmb_push

Spare Frame:

header41_[0]		tmb_allow_alct		Allow ALCT-only  tmb-matching

header41_[1]		tmb_allow_clct		Allow CLCT-only  tmb-matching

header41_[2]		tmb_allow_match		Allow Match-only tmb-matching

header41_[3]		tmb_allow_alct_ro		Allow ALCT-only  tmb-matching, non-trigger readout

header41_[4]		tmb_allow_clct_ro		Allow CLCT-only  tmb-matching, non-trigger readout

header41_[5]		tmb_allow_match_ro		Allow Match-only tmb-matching, non-trigger readout

header41_[6]		r_tmb_alct_only_ro		Only ALCT trig, pushed into L1A queue, non-triggering

header41_[7]		r_tmb_clct_only_ro		Only CLCT trig, pushed into L1A queue, non-triggering

header41_[8]		r_tmb_match_ro		ALCT*CLCT match, pushed into L1A queue, non-triggering

header41_[9]		r_tmb_trig_keep		This is a triggering readout event

header41_[10]		r_tmb_non_trig_keep		This is a non-triggering readout event

header41_[13:11]	lyr_thresh_pretrig[2:0]	Layer pre-trigger threshold

header41_[14]		layer_trig_en			Layer-trigger mode enabled

# TMB Data Format: Short Header Mode

| FIFO Control |  |  |  | DDU | TMB Data [14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame # | /write fifo | DAV Data Available | last word | d15 DDU special | d14 |  | d13 | d12 | d11 | d10 | d9 |  | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 | d0 |
| No Write | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | B0C16 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | BXN Counter at L1A arrival [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | L1A Rx Counter [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | Readout Counter[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 0 | 0 | 0 | sync err | buf_q ovf |  | run_id[3:0] |  |  |  | csc_id[3:0] |  |  |  |  | board_id[4:0] |  |  |  |  |
| 5 | 0 | 0 | 0 | 0 | buf stalled | has buf |  | l1a_type[1:0] |  | rec_type[1:0] |  | fifo_mode[2:0] |  |  |  | nheader_words[5:0] |  |  |  |  |  |
| 6 | 0 | 0 | 0 | 0 | board_status[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 0 | 0 | 0 | 0 | firmware_revcode[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | EEF16 [11:0] (=E0F for full header events, EEF for short header) |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | 1 TMB | CRC22[10:0] |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  | 1 TMB | CRC22[21:11] |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 0 | 0 | 1 | 1 | DDU Code 1012 |  |  |  | 1 TMB | Word Count [10:0] |  |  |  |  |  |  |  |  |  |  |  |
| No Write | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frame # | /write fifo | DAV Data Available | last word | d15 | d14 |  | d13 | d12 | d11 | d10 | d9 |  | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 | d0 |

# ALCT Header/Trailer Format:

| FIFO Control |  |  |  | DDU | ALCT Data [14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame # | /write fifo | DAV Data Available | last word | d15 DDU special | d14 | d13 | d12 | d11 | d10 | d9 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 | d0 |
| No Write | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | B0A16 |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | BXN Counter at L1A arrival [11:0] |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | L1A Rx Counter [11:0] |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | Readout Counter[11:0] |  |  |  |  |  |  |  |  |  |  |  |

| n-3 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | E0D16 [11:0] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n-2 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | 0 ALCT | CRC22[10:0] |  |  |  |  |  |  |  |  |  |  |
| n-1 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  | 0 ALCT | CRC22[21:11] |  |  |  |  |  |  |  |  |  |  |
| n | 0 | 0 | 1 | 1 | DDU Code 1012 |  |  | 0 CRC OK=1 | Word Count [10:0] |  |  |  |  |  |  |  |  |  |  |
| No Write | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frame # | /write fifo | DAV Data Available | last word | d15 | d14 | d13 | d12 | d11 | d10 | d9 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 | d0 |

Notes:

[1] CRC OK=1 is inserted by TMB after it calculates the CRC for data received from ALCT, and compares it to the CRC words sent by ALCT

# TMB Data Format: Long Header-Only Mode

| FIFO Control |  |  |  | DDU | TMB Data [14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame # | /write fifo | DAV Data Available | last word | d15 DDU special | d14 | d13 |  | d12 |  | d11 |  | d10 |  | d9 |  |  | d8 |  |  | d7 |  |  |  | d6 | d5 | d4 |  |  | d3 |  | d2 | d1 |  | d0 |  |  |
| No Write | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | B0C16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | BXN Counter at L1A arrival [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | L1A Rx Counter [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | Readout Counter[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 0 | 0 | 0 | sync err | buf_q ovf |  | run_id[3:0] |  |  |  |  |  |  |  | csc_id[3:0] |  |  |  |  |  |  |  |  |  | board_id[4:0] |  |  |  |  |  |  |  |  |  |  |
| 5 | 0 | 0 | 0 | 0 | buf stalled | has buf |  | l1a_type[1:0] |  |  |  | rec_type[1:0] |  |  |  | fifo_mode[2:0] |  |  |  |  |  |  |  |  | nheader_words[5:0] |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 0 | 0 | 0 | 0 | board_status[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 0 | 0 | 0 | 0 | firmware_revcode[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 0 | 0 | 0 | 0 | lock lost | clct1 discard |  | clct0 discard | bxn_counter_ff[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 0 | 0 | 0 | 0 | pretrig_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 0 | 0 | 0 | 0 | pretrig_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 0 | 0 | 0 | 0 | clct_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 0 | 0 | 0 | 0 | clct_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 0 | 0 | 0 | 0 | trig_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 0 | 0 | 0 | 0 | trig_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 0 | 0 | 0 | 0 | alct_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 0 | 0 | 0 | 0 | alct_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 0 | 0 | 0 | 0 | uptime_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 0 | 0 | 0 | 0 | uptime_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 0 | 0 | 0 | 0 | miniscope read ena | scope esixts | fifo_pretrig[4:0] |  |  |  |  |  |  |  |  |  |  |  |  | fifo_tbins[4:0] |  |  |  |  |  |  |  |  |  |  | ncfebs[2:0] |  |  |  |  |  |
| 20 | 0 | 0 | 0 | 0 | stagger csc | pid_thresh_postdrift[3:0] |  |  |  |  |  |  |  | hit_thresh_postdrift[2:0] |  |  |  |  |  |  |  |  | pid_thresh_pretrig[3:0] |  |  |  |  |  |  |  | hit_thresh_pretrig[2:0] |  |  |  |  |  |
| 21 | 0 | 0 | 0 | 0 | clct_window[3:0] |  |  |  |  |  |  | alct_delay[3:0] |  |  |  |  |  |  |  |  |  |  | dmb_thresh_pretrig[2:0] |  |  |  | triad_persist[3:0] |  |  |  |  |  |  |  |  |  |
| 22 | 0 | 0 | 0 | 0 | layers_hit_vec[5:0] |  |  |  |  |  |  |  |  |  |  | trig_source_vec[8:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | 0 | 0 | 0 | 0 | aff source | l1a_match_win[3:0] |  |  |  |  |  |  |  | cfebs_read[4:0] |  |  |  |  |  |  |  |  |  |  |  | active_cfeb[4:0] |  |  |  |  |  |  |  |  |  |  |
| 24 | 0 | 0 | 0 | 0 | lct rank err | dupe clct |  | dupe alct |  |  | two clct | two alct |  | one clct |  |  |  | one alct |  |  | no alct |  |  | match_win[3:0] |  |  |  |  |  |  | clct only |  | alct only |  | tmb match |  |
| 25 | 0 | 0 | 0 | 0 | clct0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 0 | 0 | 0 | 0 | clct1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | 0 | 0 | 0 | 0 | perr summary | perr rpc+mini |  | parity error cfeb ram[4:0] SEU |  |  |  |  |  |  |  |  |  |  |  | clct1 busy |  |  |  | clct1 invp | clct0 invp | clctc[2:0] |  |  |  |  |  | clct1[15] |  | clct0[15] |  |  |
| 28 | 0 | 0 | 0 | 0 | alct_pretrig_win[3:0] |  |  |  |  |  |  |  | alct0_key[6:0] |  |  |  |  |  |  |  |  |  |  |  |  | alct0 amu |  |  |  |  | alct0 quality[1:0] |  |  |  | alct0 valid |  |
| 29 | 0 | 0 | 0 | 0 | layer triggerd | bcb readout |  | drift_delay[1:0] |  |  |  |  | alct1_key[6:0] |  |  |  |  |  |  |  |  |  |  |  |  | alct1 amu |  |  |  |  | alct1 quality[1:0] |  |  |  | alct1 valid |  |
| 30 | 0 | 0 | 0 | 0 | bx0 match | alct cfg done |  | cfeb bits blocked |  |  | cfeb_badbits_found[4:0] |  |  |  |  |  |  |  |  |  |  |  |  | alct_ecc_err[1:0] |  | alct_bxn[4:0] |  |  |  |  |  |  |  |  |  |  |
| 31 | 0 | 0 | 0 | 0 | mpc0_frame0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | 0 | 0 | 0 | 0 | mpc0_frame1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 0 | 0 | 0 | 0 | mpc1_frame0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | 0 | 0 | 0 | 0 | mpc1_frame1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 | 0 | 0 | 0 | 0 | cfeb_en[4:0] |  |  |  |  |  |  |  |  | mpc_accept[1:0] |  |  |  |  |  |  |  | mpc_tx_delay[3:0] |  |  |  |  |  |  |  | mpc1fr1 [15] | mpc1fr0 [15] |  | mpc0fr1 [15] |  |  | mpc0fr0 [15] |
| 36 | 0 | 0 | 0 | 0 | fifo_pretrig_rpc[4:0] |  |  |  |  |  |  |  |  | fifo_tbins_rpc[4:0] |  |  |  |  |  |  |  |  |  |  |  | rpc read en |  |  |  | nrpcs[1:0] |  |  | rpc_list[1:0] |  |  |  |
| 37 | 0 | 0 | 0 | 0 | buf_q empty | buf_q full |  | wr_buf ready |  | r_wr_buf ready |  | r_wr_buf_adr[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | 0 | 0 | 0 | 0 | buf stalled ff | buf_q adr err |  | buf_q udf err |  | buf_q ovf err |  | r_buf_fence_dist[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 | 0 | 0 | 0 | 0 | reverse me1b | reverse me1a |  | reverse csc |  | buf_fence_cnt[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | 0 | 0 | 0 | 0 | tmb trig pulse | trig_src_vec[10:9] |  |  |  | mxcfeb=7 |  | peak fence |  | cfeb_en[6:5] |  |  |  |  | perr_cfeb[6:5] |  |  |  |  |  | cfeb_badbits_found[6:5 |  |  | cfebs_read[6:5] |  |  |  |  | active_cfeb[6:5] |  |  |  |
| 41 | 0 | 0 | 0 | 0 | layer trig enabled | lyr_thresh_pretrig[2:0] |  |  |  |  |  | non-trig readout |  | triggered readout | non-trig match ro |  |  |  | non-trig clct ro |  |  |  |  | non-trig alct o | allow match ro | allow clct ro |  | allow alct ro |  |  | allow match |  | allow clct | allow alct |  |  |
| 42 | 0 | 0 | 0 | 0 | 6 |  |  |  |  | EOB End Header Block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 | 0 | 0 | 0 | 0 | 6 |  |  |  |  | E0C End Cathode Block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | E0F16 [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 45 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | 1 TMB |  | CRC22[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 | 0 | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  | 1 TMB |  | CRC22[21:11] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 | 0 | 0 | 1 | 1 | DDU Code 1012 |  |  |  |  | 1 TMB |  | Word Count [10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Write | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frame # | /write fifo | DAV Data Available | last word | d15 | d14 | d13 |  | d12 |  | d11 |  | d10 |  | d9 |  |  | d8 |  |  | d7 |  |  |  | d6 | d5 | d4 |  |  | d3 |  | d2 | d1 |  | d0 |  |  |

TMB Data Format: Full-Readout Mode

| FIFO Control |  |  |  |  | DDU | TMB Data [14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame # | /write fifo |  | DAV Data Available | last word | d15 DDU special | d14 | d13 |  |  | d12 |  |  | d11 | d10 | d9 | d8 | d7 |  | d6 |  |  | d5 |  | d4 |  |  | d3 |  |  | d2 |  |  |  |  |  | d1 | d0 |  |
| No Write | 1 |  | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | B0C16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | BXN Counter at L1A arrival [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | L1A Rx Counter [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | Readout Counter[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 |  | 0 | 0 | 0 | sync err | buf_q ovf |  |  | run_id[3:0] |  |  |  |  |  | csc_id[3:0] |  |  |  |  |  |  |  | board_id[4:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | 0 |  | 0 | 0 | 0 | buf stalled | has buf |  |  | l1a_type[1:0] |  |  |  | rec_type[1:0] |  | fifo_mode[2:0] |  |  |  |  |  | nheader_words[5:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 0 |  | 0 | 0 | 0 | board_status[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 0 |  | 0 | 0 | 0 | firmware_revcode[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 0 |  | 0 | 0 | 0 | lock lost | clct1 discard |  |  | clct0 discard |  | bxn_counter_ff[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 0 |  | 0 | 0 | 0 | pretrig_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 0 |  | 0 | 0 | 0 | pretrig_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 0 |  | 0 | 0 | 0 | clct_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 0 |  | 0 | 0 | 0 | clct_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 0 |  | 0 | 0 | 0 | trig_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 0 |  | 0 | 0 | 0 | trig_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 0 |  | 0 | 0 | 0 | alct_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 0 |  | 0 | 0 | 0 | alct_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 0 |  | 0 | 0 | 0 | uptime_counter[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 0 |  | 0 | 0 | 0 | uptime_counter[29:15] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 0 |  | 0 | 0 | 0 | miniscoperead ena | scope esixts | fifo_pretrig[4:0] |  |  |  |  |  |  |  |  | fifo_tbins[4:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ncfebs[2:0] |  |  |  |  |  |
| 20 | 0 |  | 0 | 0 | 0 | stagger csc | pid_thresh_postdrift[3:0] |  |  |  |  |  |  |  | hit_thresh_postdrift[2:0] |  |  | pid_thresh_pretrig[3:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | hit_thresh_pretrig[2:0] |  |  |  |  |
| 21 | 0 |  | 0 | 0 | 0 | clct_window[3:0] |  |  |  |  |  |  |  | alct_delay[3:0] |  |  |  | dmb_thresh_pretrig[2:0] |  |  |  |  |  |  |  |  |  |  | triad_persist[3:0] |  |  |  |  |  |  |  |  |  |
| 22 | 0 |  | 0 | 0 | 0 | layers_hit_vec[5:0] |  |  |  |  |  |  |  |  |  | trig_source_vec[8:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | 0 |  | 0 | 0 | 0 | aff source | l1a_match_win[3:0] |  |  |  |  |  |  |  | febs_read[4:0] |  |  |  |  |  |  |  |  |  | active_feb[4:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | 0 |  | 0 | 0 | 0 | lct rank err | dupe clct |  | dupe alct |  |  |  | two clct | two alct | one clct | one alct | no alct |  |  | match_win[3:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  | clct only | alct only |  | tmb match |
| 25 | 0 |  | 0 | 0 | 0 | clct0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 0 |  | 0 | 0 | 0 | clct1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | 0 |  | 0 | 0 | 0 | perr summary | perr rpc+mini |  |  |  | parity error cfeb ram[4:0] SEU |  |  |  |  |  | clct1 busy |  | clct1 invp |  |  | clct0 invp |  | clctc[2:0] |  |  |  |  |  |  |  |  |  |  |  | clct1[15] | clct0[15] |  |
| 28 | 0 |  | 0 | 0 | 0 | alct_pretrig_win[3:0] |  |  |  |  |  |  |  | alct0_key[6:0] |  |  |  |  |  |  |  |  |  |  | alct0 amu |  |  |  |  |  |  | alct0 quality[1:0] |  |  |  |  |  | alct0 valid |
| 29 | 0 |  | 0 | 0 | 0 | layer triggerd | bcb readout |  |  | drift_delay[1:0] |  |  |  | alct1_key[6:0] |  |  |  |  |  |  |  |  |  |  | alct1 amu |  |  |  |  |  |  | alct1 quality[1:0] |  |  |  |  |  | alct1 valid |
| 30 | 0 |  | 0 | 0 | 0 | bx0 match | alct cfg done |  |  | cfeb bits blocked |  |  | cfeb_badbits_found[4:0] |  |  |  |  |  | alct_ecc_err[1:0] |  |  |  |  |  | alct_bxn[4:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 0 |  | 0 | 0 | 0 | mpc0_frame0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | 0 |  | 0 | 0 | 0 | mpc0_frame1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | 0 |  | 0 | 0 | 0 | mpc1_frame0[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | 0 |  | 0 | 0 | 0 | mpc1_frame1[14:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 | 0 |  | 0 | 0 | 0 | cfeb_en[4:0] |  |  |  |  |  |  |  |  | mpc_accept[1:0] |  | mpc_tx_delay[3:0] |  |  |  |  |  |  |  |  |  |  | mpc1fr1 [15] |  |  | mpc1fr0 [15] |  |  |  |  | mpc0fr1 [15] |  | mpc0fr0 [15] |
| 36 | 0 |  | 0 | 0 | 0 | fifo_pretrig_rpc[4:0] |  |  |  |  |  |  |  |  | fifo_tbins_rpc[4:0] |  |  |  |  |  |  |  | rpc read en |  |  |  |  | nrpcs[1:0] |  |  |  |  |  |  |  | rpc_list[1:0] |  |  |
| 37 | 0 |  | 0 | 0 | 0 | buf_q empty | buf_q full |  |  | wr_buf ready |  |  | r_wr_buf ready | r_wr_buf_adr[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | 0 |  | 0 | 0 | 0 | buf stalled ff | buf_q adr err |  |  | buf_q udf err |  |  | buf_q ovf err | r_buf_fence_dist[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 | 0 |  | 0 | 0 | 0 | reverse me1b | reverse me1a |  |  | reverse csc |  |  | buf_fence_cnt[11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | 0 | 0 |  | 0 | 0 | tmb trig pulse | trig_src_vec[10:9] |  |  |  |  |  | mxcfeb=7 | peak fence | cfeb_en[6:5] |  | perr_cfeb[6:5] |  |  |  |  | cfeb_badbits_found[6:5 |  |  |  |  |  |  | cfebs_read[6:5] |  |  |  |  |  |  | active_cfeb[6:5] |  |  |
| 41 | 0 |  | 0 | 0 | 0 | layer trig enabled | lyr_thresh_pretrig[2:0] |  |  |  |  |  |  | non-trig readout | triggered readout | non-trig match ro | non-trig clct ro |  |  |  | non-trig alct o | allow match ro |  |  |  | allow clct ro |  |  | allow alct ro |  |  |  |  | allow match |  | allow clct | allow alct |  |
| 42 | 0 |  | 0 | 0 | 0 | 6 |  |  |  |  |  |  | EOB End Header Block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly0[7:0] Triad bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly1[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly2[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly3[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly4[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Ly5[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly0[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly1[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly2[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly3[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly4[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  | CFEB 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Ly5[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 55-246 |  |  |  |  |  | --- |  |  |  |  |  |  | --- |  |  |  | --- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 247 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly0[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 248 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly1[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 249 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly2[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 250 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly3[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 251 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly4[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 252 |  |  |  |  |  | CFEB 4 |  |  |  |  |  |  | Tbin 6 |  |  |  | Ly5[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 253 |  |  |  |  |  | 6 |  |  |  |  |  |  | B0416 Begin RPC Raw Hits (if RPC readout enabled) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 254 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 0 |  |  |  | Pads[7:0] RPC0 Pads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 256 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 1 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 257 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 258 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 2 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 259 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 260 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 3 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 261 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 262 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 4 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 263 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 264 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 5 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 265 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 266 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | Tbin 6 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 267 |  |  |  |  |  | RPC 0 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 268 |  |  |  |  |  | RPC 1 |  |  |  |  |  |  | Tbin 0 |  |  |  | Pads[7:0] RPC1 Pads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 269 |  |  |  |  |  | RPC 1 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 270-279 |  |  |  |  |  | -- |  |  |  |  |  |  | -- |  |  |  | -- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 280 |  |  |  |  |  | RPC 1 |  |  |  |  |  |  | Tbin 6 |  |  |  | Pads[7:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 281 |  |  |  |  |  | RPC 1 |  |  |  |  |  |  | clct pretrigger | rpc_bxn[2:0] |  |  | Pads[15:8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 282 |  |  |  |  |  | 6 |  |  |  |  |  |  | E0416 End RPC Raw Hits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 283 |  |  |  |  |  | 6 |  |  |  |  |  |  | E0C16 End Cathode Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| opt |  |  |  |  |  | 2AAA16[14:0] (Optional to make word count multiple of 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| opt |  |  |  |  |  | 555516[14:0] (Optional to make word count multiple of 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 284 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | E0F16 [11:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 285 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | 1 TMB | CRC22[10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 286 | 0 |  | 0 | 0 | 1 | DDU Code 1012 |  |  |  |  |  |  | 1 TMB | CRC22[21:11] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 287 | 0 |  | 0 | 1 | 1 | DDU Code 1012 |  |  |  |  |  |  | 1 TMB | Word Count [10:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Write | 1 |  | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Sample TMB Raw Hits Dump

TMB internal pattern injector + RPC internal pattern injector

7-Time bins, full 5 CLCTs+ 2 RPCs raw hits readout  (Blocked CFEB DiStrips list turned off)

Adr=   0 Data=2DB0C

Adr=   1 Data=0DCC8

Adr=   2 Data=0D001

Adr=   3 Data=0D001

Adr=   4 Data=04045

Adr=   5 Data=0226A

Adr=   6 Data=0777F

Adr=   7 Data=0512C

Adr=   8 Data=00C47

Adr=   9 Data=00001

Adr=  10 Data=00000

Adr=  11 Data=00001

Adr=  12 Data=00000

Adr=  13 Data=00001

Adr=  14 Data=00000

Adr=  15 Data=00001

Adr=  16 Data=00000

Adr=  17 Data=002DD

Adr=  18 Data=00000

Adr=  19 Data=0023D

Adr=  20 Data=04204

Adr=  21 Data=01A46

Adr=  22 Data=07E01

Adr=  23 Data=003E1

Adr=  24 Data=00301

Adr=  25 Data=005AD

Adr=  26 Data=00000

Adr=  27 Data=0531C

Adr=  28 Data=010A7

Adr=  29 Data=01000

Adr=  30 Data=00001

Adr=  31 Data=07D0A

Adr=  32 Data=02605

Adr=  33 Data=00000

Adr=  34 Data=00000

Adr=  35 Data=07C01

Adr=  36 Data=008FB

Adr=  37 Data=01E25

Adr=  38 Data=007FF

Adr=  39 Data=00001

Adr=  40 Data=06001

Adr=  41 Data=02326

Adr=  42 Data=06E0B

Adr=  43 Data=00000

Adr=  44 Data=00000

Adr=  45 Data=00000

Adr=  46 Data=00000

Adr=  47 Data=00000

Adr=  48 Data=00000

Adr=  49 Data=00100

Adr=  50 Data=00100

Adr=  51 Data=00100

Adr=  52 Data=00100

Adr=  53 Data=00100

Adr=  54 Data=00100

Adr=  55 Data=00202

Adr=  56 Data=00202

Adr=  57 Data=00202

Adr=  58 Data=00202

Adr=  59 Data=00202

Adr=  60 Data=00202

Adr=  61 Data=00300

Adr=  62 Data=00302

Adr=  63 Data=00300

Adr=  64 Data=00302

Adr=  65 Data=00300

Adr=  66 Data=00302

Adr=  67 Data=00402

Adr=  68 Data=00400

Adr=  69 Data=00402

Adr=  70 Data=00400

Adr=  71 Data=00402

Adr=  72 Data=00400

Adr=  73 Data=00500

Adr=  74 Data=00500

Adr=  75 Data=00500

Adr=  76 Data=00500

Adr=  77 Data=00500

Adr=  78 Data=00500

Adr=  79 Data=00600

Adr=  80 Data=00600

Adr=  81 Data=00600

Adr=  82 Data=00600

Adr=  83 Data=00600

Adr=  84 Data=00600

Adr=  85 Data=01000

Adr=  86 Data=01000

Adr=  87 Data=01000

Adr=  88 Data=01000

Adr=  89 Data=01000

Adr=  90 Data=01000

Adr=  91 Data=01100

Adr=  92 Data=01100

Adr=  93 Data=01100

Adr=  94 Data=01100

Adr=  95 Data=01100

Adr=  96 Data=01100

Adr=  97 Data=01200

Adr=  98 Data=01200

Adr=  99 Data=01200

Adr= 100 Data=01200

Adr= 101 Data=01200

Adr= 102 Data=01200

Adr= 103 Data=01300

Adr= 104 Data=01300

Adr= 105 Data=01300

Adr= 106 Data=01300

Adr= 107 Data=01300

Adr= 108 Data=01300

Adr= 109 Data=01400

Adr= 110 Data=01400

Adr= 111 Data=01400

Adr= 112 Data=01400

Adr= 113 Data=01400

Adr= 114 Data=01400

Adr= 115 Data=01500

Adr= 116 Data=01500

Adr= 117 Data=01500

Adr= 118 Data=01500

Adr= 119 Data=01500

Adr= 120 Data=01500

Adr= 121 Data=01600

Adr= 122 Data=01600

Adr= 123 Data=01600

Adr= 124 Data=01600

Adr= 125 Data=01600

Adr= 126 Data=01600

Adr= 127 Data=02000

Adr= 128 Data=02000

Adr= 129 Data=02000

Adr= 130 Data=02000

Adr= 131 Data=02000

Adr= 132 Data=02000

Adr= 133 Data=02100

Adr= 134 Data=02100

Adr= 135 Data=02100

Adr= 136 Data=02100

Adr= 137 Data=02100

Adr= 138 Data=02100

Adr= 139 Data=02200

Adr= 140 Data=02200

Adr= 141 Data=02200

Adr= 142 Data=02200

Adr= 143 Data=02200

Adr= 144 Data=02200

Adr= 145 Data=02300

Adr= 146 Data=02300

Adr= 147 Data=02300

Adr= 148 Data=02300

Adr= 149 Data=02300

Adr= 150 Data=02300

Adr= 151 Data=02400

Adr= 152 Data=02400

Adr= 153 Data=02400

Adr= 154 Data=02400

Adr= 155 Data=02400

Adr= 156 Data=02400

Adr= 157 Data=02500

Adr= 158 Data=02500

Adr= 159 Data=02500

Adr= 160 Data=02500

Adr= 161 Data=02500

Adr= 162 Data=02500

Adr= 163 Data=02600

Adr= 164 Data=02600

Adr= 165 Data=02600

Adr= 166 Data=02600

Adr= 167 Data=02600

Adr= 168 Data=02600

Adr= 169 Data=03000

Adr= 170 Data=03000

Adr= 171 Data=03000

Adr= 172 Data=03000

Adr= 173 Data=03000

Adr= 174 Data=03000

Adr= 175 Data=03100

Adr= 176 Data=03100

Adr= 177 Data=03100

Adr= 178 Data=03100

Adr= 179 Data=03100

Adr= 180 Data=03100

Adr= 181 Data=03200

Adr= 182 Data=03200

Adr= 183 Data=03200

Adr= 184 Data=03200

Adr= 185 Data=03200

Adr= 186 Data=03200

Adr= 187 Data=03300

Adr= 188 Data=03300

Adr= 189 Data=03300

Adr= 190 Data=03300

Adr= 191 Data=03300

Adr= 192 Data=03300

Adr= 193 Data=03400

Adr= 194 Data=03400

Adr= 195 Data=03400

Adr= 196 Data=03400

Adr= 197 Data=03400

Adr= 198 Data=03400

Adr= 199 Data=03500

Adr= 200 Data=03500

Adr= 201 Data=03500

Adr= 202 Data=03500

Adr= 203 Data=03500

Adr= 204 Data=03500

Adr= 205 Data=03600

Adr= 206 Data=03600

Adr= 207 Data=03600

Adr= 208 Data=03600

Adr= 209 Data=03600

Adr= 210 Data=03600

Adr= 211 Data=04000

Adr= 212 Data=04000

Adr= 213 Data=04000

Adr= 214 Data=04000

Adr= 215 Data=04000

Adr= 216 Data=04000

Adr= 217 Data=04100

Adr= 218 Data=04100

Adr= 219 Data=04100

Adr= 220 Data=04100

Adr= 221 Data=04100

Adr= 222 Data=04100

Adr= 223 Data=04200

Adr= 224 Data=04200

Adr= 225 Data=04200

Adr= 226 Data=04200

Adr= 227 Data=04200

Adr= 228 Data=04200

Adr= 229 Data=04300

Adr= 230 Data=04300

Adr= 231 Data=04300

Adr= 232 Data=04300

Adr= 233 Data=04300

Adr= 234 Data=04300

Adr= 235 Data=04400

Adr= 236 Data=04400

Adr= 237 Data=04400

Adr= 238 Data=04400

Adr= 239 Data=04400

Adr= 240 Data=04400

Adr= 241 Data=04500

Adr= 242 Data=04500

Adr= 243 Data=04500

Adr= 244 Data=04500

Adr= 245 Data=04500

Adr= 246 Data=04500

Adr= 247 Data=04600

Adr= 248 Data=04600

Adr= 249 Data=04600

Adr= 250 Data=04600

Adr= 251 Data=04600

Adr= 252 Data=04600

Adr= 253 Data=06B04

Adr= 254 Data=00000

Adr= 255 Data=00000

Adr= 256 Data=00100

Adr= 257 Data=00000

Adr= 258 Data=00200

Adr= 259 Data=007AB

Adr= 260 Data=00301

Adr= 261 Data=006AB

Adr= 262 Data=00402

Adr= 263 Data=005AB

Adr= 264 Data=00503

Adr= 265 Data=004AB

Adr= 266 Data=00604

Adr= 267 Data=003AB

Adr= 268 Data=01000

Adr= 269 Data=01000

Adr= 270 Data=01100

Adr= 271 Data=01000

Adr= 272 Data=01200

Adr= 273 Data=017CD

Adr= 274 Data=01301

Adr= 275 Data=016CD

Adr= 276 Data=01402

Adr= 277 Data=015CD

Adr= 278 Data=01503

Adr= 279 Data=014CD

Adr= 280 Data=01604

Adr= 281 Data=013CD

Adr= 282 Data=06E04

Adr= 283 Data=06E0C

Adr= 284 Data=0DE0F

Adr= 285 Data=0D94F

Adr= 286 Data=0DDF2

Adr= 287 Data=1D920

# Configuration

### Shunt Settings

# Table 1: Shunts

| # | Shunt | Default | Function |
| --- | --- | --- | --- |
| 1 | SH501 [CLK SRC] | 1-2 [CCB] | 1-2 [CCB] CCB sources TMB s 40MHz Main clock 2-3 [XTAL] Onboard crystal sources TMBs 40MHz Main clock |
| 2 | SH502 [CFEB CLK] | 2-3 [DCC] | 1-2 [NRM] CFEB clocks sourced by 3D3444 64/36 duty-cycle 2-3 [DCC] CFEB clocks duty-cycle corrected |
| 3 | SH55 [GBL] | 1-2 [ENA] | 1-2 [ENA] Boot Register responds to TMB VME Global Address 2-3 [DIS] Boot Register ignores TMB VME Global Address |
| 4 | SH56 [GEO] | 1-2 [ENA] | 1-2 [ENA] Boot Register responds to VME Geographic Address 2-3 [DIS] Boot Register ignores VME Geographic Address |
| 5 | SH57 [ADM] | 1-2 [REQ] | 1-2 [REQ] Boot Register requires VME Address Mode 39h or 3Dh 2-3 [PAS] Boot Register accepts any VME Address Mode |
| 6 | SH97 [FPGA] | 1-2 [ENA] | 1-2 [ENA] Cleared Boot Register enables FPGA VME access 2-3 [DIS] Cleared Boot Register disables FPGA VME access |
| 7 | SH69-1 [REG IN] | 2-3 [REG] | 1-2 [BPL] Disconnect +3.3V from U69 1.5V regulator input 2-3 [REG] Connect +3.3V to U69 1.5V regulator input |
| 8 | SH69-2 [REG OUT] | 2-3 [REG] | 1-2 [BPL] Backplane sources +1.5Vtt 2-3 [REG] Onboard regulator U69 sources +1.5Vtt |
| 9 | SH95 [BOOT] | 1-2 [EN] | 1-2 [EN] CCB hard-reset clears Boot Register 2-3 [DIS] CCB hard-reset does not clear Boot Register |
| 10 | SH104 [VME] | 1-2 [EN] | 1-2 [EN] VME write to Boot Register can hard-reset TMB or ALCT 2-3 [DIS] Boot Register can not hard-reset TMB or ALCT |
| 11 | SH105 [CCB] | 1-2 [EN] | 1-2 [EN] CCB can hard-reset TMB or ALCT 2-3 [DIS] CCB can not hard-reset TMB or ALCT |
| 12 | SH106 [ALCT] | 1-2 [EN] | 1-2 [EN] TMB FPGA can hard-reset ALCT board if enabled by Boot 2-3 [DIS] TMB FPGA firmware can not hard-reset ALCT board |
| 13 | SH107 [SELF] | 2-3 [DIS] | 1-2 [EN] TMB FPGA firmware can hard-reset TMB 2-3 [DIS] TMB FPGA firmware can not hard-reset TMB |
| 14 | SH1081 [ALCT] | 1-2 [EN] | 1-2 [EN] TMB can send hard-reset to ALCT board 2-3 [DIS] TMB can not send hard-reset to ALCT board |
| 15 | SH1082 [TMB] | 1-2 [EN] | 1-2 [EN] TMB can send hard-reset to TMB 2-3 [DIS] TMB can not send hard-reset to TMB |
| 16 | SH1083 [RPC] | 1-2 [EN] | 1-2 [EN] TMB can send hard-reset to RAT/RPC board 2-3 [DIS] TMB can not send hard-reset to RAT/RPC board |
| 17 | SH1084 [PUP] | 1-2 [EN] | 1-2 [EN] Issue hard-reset to TMB on power-up 2-3 [DIS] Do not issue hard-reset to TMB on power-up |
| 18 | SH62 [VMEADR] | 1-2 [GEO] | 1-2 [GEO] Slot Address derived from VME backplane GEO pins 2-3 [LCL] Slot Address derived from SW2/SW1 hex switches |
| 19 | SH74 [JTAG SRC] | 2-3 [SW3] | 1-2 [XBL] X-blaster board sources JTAG chain address 2-3 [SW3] Onboard SW3 hex switch sources JTAG chain address |
| 20 | SH921 [RAT ADC] | 1-2 [ACORE] | 1-2 [ACORE] ADC Ch9 measures RAT core current 2-3 [+3.3VR] ADC Ch9 measures RAT main +3.3V supply |

### Switch Settings

# Table 2: Switch Settings

| Switch | Module | Default | Function |
| --- | --- | --- | --- |
| SW1 | VME | A | SW2/SW1 Form the VME slot address for Local Mode addressing and for the Boot Register Global Address. Normally, SH62 [VMEADR] will be set to 1-2 [GEO] and the VME Slot-Address is determined by the Slot-ID signals from the VME P1 backplane connector. SW2/SW1 should be set to 1A [26 decimal] to specify the Boot Register Global Address. SW2 is the most-significant hex digit. If SH62 [VMEADR] is set 2-3 [LCL], hex rotary switches SW2/SW1 determine the VME slot address for the module, and the P1 Geographic Address signals are ignored. |
| SW2 | VME | 1 |  |
| SW3 | JTAG | 4 | Selects the JTAG Chain Address for the Xblaster when SH74 [JTAG ADR SRC] is set to 2-3 [SW3]. If SH74 is set 1-2 [XBL], the JTAG Chain Address is determined by the Xblaster. 0000 0 ALCT SLOW USER 0001 1 ALCT SLOW PROM 0010 2 ALCT FPGA USER 0011 3 ALCT FPGA PROMs 01XX 4 TMB FPGA PROMs 10XX 8 TMB User PROMs 1100 C TMB FPGA Loop (for TMB self-test) 1101 D RAT FPGA PROM |

### Fuses

# Table 3: Fuses for TMB2005

| Fuse | Circuit | Amps | LED | Function | Littelfuse Part # |
| --- | --- | --- | --- | --- | --- |
| F1 | +5.0V | 5A | D_F1 Red | VME-bus I/O | 154005.DR |
| F2 | +3.3V | 10A | D_F2 Red | Main board logic, FPGA IOBs | 154010.DR |
| F3 | +1.5V | 5A | None | GTLP termination | 154005.DR |
| F4 | +3.3VXB | 1A | D_F4 Grn | Xblaster power | 154001.DR |

# Table 4A: Fuses for TMB2013

| Fuse | Circuit | Amps | LED | Function | Littelfuse Part # |
| --- | --- | --- | --- | --- | --- |
| F1 | +5.0V | 10A | D_F1 Red | VME-bus I/O | 0451010.MRL |
| F2 | +3.3V | 15A | D_F2 Red | Main board logic, FPGA IOBs | 0451015.MRL |
| F3 | +1.5V | 10A | None | GTLP termination | 0451010.MRL |
| F4 | +3.3VXB | 1A | D_F4 Grn | Xblaster power | 154001.DR |

# Signal Summary

### CCB

1 Input	LVDS    40MHz clock

46 Inputs 	GTLP at 40MHz

46 Outputs	GTLP at 40MHz

# Table 5: CCB Signal Summary

| Signal | Bits | Dir | Logic | Function |
| --- | --- | --- | --- | --- |
| Clock Bus |  |  |  |  |
| ccb_clock40 | 1 | In | LVDS |  |
| Total | 1 |  |  |  |
| Fast Control Bus |  |  |  |  |
| ccb_clock40_enable | 1 | In | GTLP |  |
| ccb_cmd[5..0] | 6 | In | GTLP |  |
| ccb_evcntres | 1 | In | GTLP |  |
| ccb_bcntres | 1 | In | GTLP |  |
| ccb_cmd_strobe | 1 | In | GTLP |  |
| ccb_bx0 | 1 | In | GTLP |  |
| ccb_l1accept | 1 | In | GTLP |  |
| ccb_data[7..0] | 8 | In | GTLP |  |
| ccb_data_strobe | 1 | In | GTLP |  |
| ccb_reserved[4..0] | 5 | In | GTLP |  |
| Total | 26 |  |  |  |
| TMB Reload Bus: ALCT+CLCT+TMB FPGA Reload |  |  |  |  |
| tmb_hard_reset | 1 | In | GTLP |  |
| tmb_cfg_done[8..0] | 9 | Out | GTLP |  |
| alct_hard_reset | 1 | In | GTLP |  |
| alct_cfg_done[8..0] | 9 | Out | GTLP |  |
| tmb_reserved[1..0] | 2 | In | GTLP |  |
| Total | 22 |  |  |  |
| DAQ Special Purpose Bus [Used by DMB and TMB] |  |  |  |  |
| dmb_cfeb_calibrate[2..0] | 3 | In | GTLP |  |
| dmb_l1a_release | 1 | (In) | GTLP |  |
| dmb_reserved_out[4..0] | 5 | In | GTLP |  |
| dmb_reserved_in[2..0] | 3 | (In) | GTLP |  |
| Total | 12 |  |  |  |
| Trigger Special Purpose Bus [Used by TMB only] |  |  |  |  |
| alct_adb_pulse_sync | 1 | In | GTLP |  |
| alct_adb_pulse_async | 1 | In | GTLP |  |
| clct_external_trigger | 1 | In | GTLP |  |
| alct_external_trigger | 1 | In | GTLP |  |
| clct_status[8..0] | 9 | Out | GTLP |  |
| alct_status[8..0] | 9 | Out | GTLP |  |
| tmb_l1a_request | 1 | Out | GTLP |  |
| tmb_l1a_release | 1 | Out | GTLP |  |
| tmb_reserved_in[4..0] | 5 | Out | GTLP |  |
| tmb_reserved_out[2..0] | 3 | In | GTLP |  |
| Total | 32 |  |  |  |

### ALCT

29 Inputs	LVDS Multiplexed at 80 MHz

20 Outputs	LVDS Multiplexed at 80 MHz

1 Output	LVDS 40MHz clock

# Table 6: ALCT Signal Summary

| Signal | Bits | Pins | Mux | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- | --- |
| first_valid | 1 | 0.5 | Yes | In | LVDS | Valid Pattern Flag, best muon |
| first_quality[1..0] | 2 | 1 | Yes | In | LVDS | Pattern Quality, best muon |
| first_amu | 1 | 0.5 | Yes | In | LVDS | Accelerator Muon Flag, best muon |
| first_key[6..0] | 7 | 3.5 | Yes | In | LVDS | Key Wire Group, best muon |
| second_valid | 1 | 0.5 | Yes | In | LVDS | Valid Pattern Flag, 2nd best muon |
| second_quality[1..0] | 2 | 1 | Yes | In | LVDS | Pattern Quality, 2nd best muon |
| second_amu | 1 | 0.5 | Yes | In | LVDS | Accelerator Muon Flag, 2nd best muon |
| second_key[6..0] | 7 | 3.5 | Yes | In | LVDS | Key Wire Group, 2nd best muon |
| bxn[4..0] | 5 | 2.5 | Yes | In | LVDS | Bunch Crossing Number |
| daq_data[13..0] | 14 | 7 | Yes | In | LVDS | DAQ data |
| /wr_fifo | 1 | 0.5 | Yes | In | LVDS | /Write_enable DAQ FIFO |
| lct_special | 1 | 0.5 | Yes | In | LVDS | LCT Special Word Flag |
| ddu_special | 1 | 0.5 | Yes | In | LVDS | DAQ Special Word Flag |
| first_frame | 1 | 0.5 | Yes | In | LVDS | First DAQ Frame |
| last_frame | 1 | 0.5 | Yes | In | LVDS | Last DAQ Frame |
| seq_status[1..0] | 2 | 1 | Yes | In | LVDS | Sequencer Status |
| seu_status[1..0] | 2 | 1 | Yes | In | LVDS | Radiation SEU Status |
| active_feb_flag | 1 | 0.5 | Yes | In | LVDS | Active FEB Flag (ALCT pre-triggered) |
| cfg_done | 1 | 0.5 | Yes | In | LVDS | FPGA configuration done |
| reserved_out[3..0] | 4 | 2 | Yes | In | LVDS | Future use |
| tdo | 1 | 1 | No | In | LVDS | JTAG tdo from ALCT |
| Total Inputs | 57 | 29 |  |  |  |  |

| Signal | Bits | Pins | Mux | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- | --- |
| ccb_brcst[7..0] | 8 | 4 | Yes | Out | LVDS | CCB broadcast command |
| brcst_str | 1 | 0.5 | Yes | Out | LVDS | ccb_brcst strobe |
| dout_str | 1 | 0.5 | Yes | Out | LVDS | ccb_dout strobe |
| subadr_str | 1 | 0.5 | Yes | Out | LVDS | ccb_subaddr strobe |
| bx0 | 1 | 0.5 | Yes | Out | LVDS | Bunch Crossing Zero |
| ext_inject | 1 | 0.5 | Yes | Out | LVDS | External Test Pattern Inject Command |
| ext_trig | 1 | 0.5 | Yes | Out | LVDS | External Trigger Command |
| level1_accept | 1 | 0.5 | Yes | Out | LVDS | Level 1 Accept |
| sync_adb_pulse | 1 | 0.5 | Yes | Out | LVDS | Synchronous ADB Test Pulse |
| seq_cmd[2..0] | 3 | 1.5 | Yes | Out | LVDS | Sequencer Command |
| reserved_in[4..0] | 5 | 2.5 | Yes | Out | LVDS | Future use |
| clock | 1 | 1 | No | Out | LVDS | 40MHz clock |
| clock_en | 1 | 1 | No | Out | LVDS | Clock enable |
| hard_reset | 1 | 1 | No | Out | LVDS | FPGA Reload Command |
| async_adb_pulse | 1 | 1 | No | Out | LVDS | Asynchronous ADB Test Pulse |
| jtag_select[1..0] | 2 | 2 | No | Out | LVDS | JTAG Chain Select |
| tck | 1 | 1 | No | Out | LVDS | JTAG tck to ALCT |
| tms | 1 | 1 | No | Out | LVDS | JTAG tms to ALCT |
| tdi | 1 | 1 | No | Out | LVDS | JTAG tdi to ALCT |
| Total Outputs | 33 | 21 |  |  |  |  |

### DMB

3 Inputs	LVTTL at 40 MHz

45 Outputs1	LVTTL at 40 MHz (see note below)

47 Outputs2	LVTTL at 40 MHz (see note below)

# Table 7: DMB Signal Summary

| Signal | Bits | Dir | Logic | Function |
| --- | --- | --- | --- | --- |
| tmb_data[14:0] | 15 | Out | LVTTL | TMB data[14:0] to DMB FIFO |
| alct_data[14:0] | 15 | Out | LVTTL | ALCT data [14:0] to DMB FIFO |
| tmb_ddu_special | 1 | Out | LVTTL | TMB DDU Special Word Flag |
| tmb_last_frame | 1 | Out | LVTTL | TMB Last FIFO frame |
| tmb_data_available | 1 | Out | LVTTL | TMB Data Available |
| /tmb_write_enable_fifo | 1 | Out | LVTTL | TMB FIFO /write_enable |
| tmb_active_feb_flag | 1 | Out | LVTTL | TMB Active Front-End-Board Flag |
| tmb_active_feb[4…0] 1 | 5 | Out | LVTTL | TMB Active FEB indicators[4...0] 1 |
| tmb_active_feb[6…0] 2 | 7 | Out | LVTTL | TMB Active FEB indicators[6...0] 2 |
| fifo_clock | 1 | Out | LVTTL | 40MHz FIFO storage clock [= tmb_clock] |
| alct_ddu_special | 1 | Out | LVTTL | ALCT DDU Special Word Flag |
| alct_last_frame | 1 | Out | LVTTL | ALCT Last FIFO frame |
| alct_first_frame(dav) | 1 | Out | LVTTL | ALCT First FIFO frame, data available |
| /alct_write_enable_fifo | 1 | Out | LVTTL | ALCT FIFO /write_enable |
| reserved_to_dmb | 5 | Out | LVTTL | Unassigned |
| Total Outputs | 57 |  |  |  |

| dmb_request_lct | 1 | In | LVTTL | DMB requests active_feb_flag from TMB |
| --- | --- | --- | --- | --- |
| dmb_ext_trig | 1 | In | LVTTL | DMB external trigger to TMB |
| dmb_fpga_pgm_done | 1 | In | LVTTL | DMB FPGA Program Done |
| reserved_from_dmb | 3 | In | LVTTL | Unassigned |
| Total Inputs | 6 |  |  |  |

### CFEB

120 Inputs	LVDS data multiplexed at 80 MHz

5 Outputs	LVDS 40MHz clock

# Table 8: CFEB Signal Summary

| Signal | Bits | Pins | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- |
| cfeb0_ly[5..0]_tr[7..0] | 48 | 24 | In | LVDS | CFEB0 6 layers x 8 triads, 80MHz |
| cfeb1_ly[5..0]_tr[7..0] | 48 | 24 | In | LVDS | CFEB1 6 layers x 8 triads, 80MHz |
| cfeb2_ly[5..0]_tr[7..0] | 48 | 24 | In | LVDS | CFEB2 6 layers x 8 triads, 80MHz |
| cfeb3_ly[5..0]_tr[7..0] | 48 | 24 | In | LVDS | CFEB3 6 layers x 8 triads, 80MHz |
| cfeb4_ly[5..0]_tr[7..0] | 48 | 24 | In | LVDS | CFEB4 6 layers x 8 triads, 80MHz |
| Total Inputs | 240 | 120 |  |  |  |

| Signal | Bits | Pins | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- |
| cfeb0_lct_clock | 1 | 1 | Out | LVDS | CFEB 1 40MHz clock |
| cfeb1_lct_clock | 1 | 1 | Out | LVDS | CFEB 2 40MHz clock |
| cfeb2_lct_clock | 1 | 1 | Out | LVDS | CFEB 3 40MHz clock |
| cfeb3_lct_clock | 1 | 1 | Out | LVDS | CFEB 4 40MHz clock |
| cfeb4_lct_clock | 1 | 1 | Out | LVDS | CFEB 5 40MHz clock |
| Total Outputs | 5 | 5 |  |  |  |

### MPC

1 Input 	GTLP at 80MHz

32 Outputs	GTLP at 80MHz

# Table 9: MPC Signal Summary

|  | Signal | Bits | Pins | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- | --- |
| First In Time | alct_first_key[6:0] | 7 | 3.5 | Out | GTLP | LCT 0 ALCT key wire-group |
|  | clct_first_pat[3:0] | 4 | 2 | Out | GTLP | LCT 0 CLCT pattern number |
|  | lct_first_quality[3:0] | 4 | 2 | Out | GTLP | LCT 0 Muon quality |
|  | first_vpf | 1 | 0.5 | Out | GTLP | LCT 0 Valid pattern flag |
|  | alct_second_key[6:0] | 7 | 3.5 | Out | GTLP | LCT 1 ALCT key wire-group |
|  | clct_second_pat[3:0] | 4 | 2 | Out | GTLP | LCT 1 CLCT pattern number |
|  | lct_second_quality[3:0] | 4 | 2 | Out | GTLP | LCT 1 Muon quality |
|  | second_vpf | 1 | 0.5 | Out | GTLP | LCT 1 Valid pattern flag |
|  |  | 0 | 0 |  |  |  |
| Second In Time | clct_first_key[7:0] | 8 | 4 | Out | GTLP | LCT 0 CLCT key ½-strip |
|  | clct_first_bend | 1 | 0.5 | Out | GTLP | LCT 0 CLCT bend direction |
|  | lct 0 sync_err | 1 | 0.5 | Out | GTLP | LCT 0 BXN does not match at BX0 |
|  | alct_first_bxn[0] | 1 | 0.5 | Out | GTLP | LCT 0 ALCT bunch crossing number |
|  | clct_first_bx0_local | 1 | 0.5 | Out | GTLP | LCT 0 local BXN from CLCT |
|  | csc_id[3:0] | 4 | 2 | Out | GTLP | CSC chamber ID |
|  | clct_second_key[7:0] | 8 | 4 | Out | GTLP | LCT 1 CLCT key ½-strip |
|  | clct_second_bend | 1 | 0.5 | Out | GTLP | LCT 1 CLCT bend direction |
|  | lct 1 sync_err | 1 | 0.5 | Out | GTLP | LCT 1 BXN does not match at BX0 |
|  | alct_second_bxn[0] | 1 | 0.5 | Out | GTLP | LCT 1 ALCT bunch crossing number |
|  | clct_second_bx0_local | 1 | 0.5 | Out | GTLP | LCT 1 local BXN from CLCT |
|  | csc_id[3:0] | 4 | 2 | Out | GTLP | CSC chamber ID |
|  | Total Output Signals | 37 | 18.5 |  |  | 2:1 Multiplexing at 80 MHz |

| Time | Signal | Bits | Pins | Dir | Logic | Function |
| --- | --- | --- | --- | --- | --- | --- |
| 1st | lct0_accept | 1 | 0.5 | In | GTLP | LCT 0 Accepted by MPC best 3 of 18 sort |
| 2nd | lct1_accept | 1 | 0.5 | In | GTLP | LCT 1 Accepted by MPC best 3 of 18 sort |
|  | Total Input Signals | 2 | 1 |  |  | 2:1 Multiplexing at 80 MHz |

### RPC

80 Inputs	40MHz LVTTL [from receivers on transition module]

0 Outputs

# Table 10: RPC Signal Summary

| Signal | Bits | Dir | Logic | Function |
| --- | --- | --- | --- | --- |
| rpc0_seg[15..0] | 16 | In | LVTTL | RPC Segment[15..0] |
| rpc0_bxn[2..0] | 3 | In | LVTTL | Bunch Crossing Number [2..0] |
| rpc0_clock | 1 | In | LVTTL | Clock from RPC Link Board |
| rpc1_seg[15..0] | 16 | In | LVTTL | RPC Segment[15..0] |
| rpc1_bxn[2..0] | 3 | In | LVTTL | Bunch Crossing Number [2..0] |
| rpc1_clock | 1 | In | LVTTL | Clock from RPC Link Board |
| Total data bits | 40 |  |  |  |

### VME

# 24 Inputs TTL Address

16 BiDir  TTL Data

# Table 11: VME Signal Summary

| Signal | Bits | Dir | Logic | Function |
| --- | --- | --- | --- | --- |
| address | 24 | In | TTL | VME Address[23..0] |
| data | 16 | BiDir | TTL | VME Data[15..0] |
| control in |  | In | TTL | VME Control Inputs |
| control out |  | Out | TTL | VME Control Outputs |

### JTAG

5 Inputs	LVDS

1 Outputs	LVDS

# Table 12: JTAG Signal Summary

| Signal | Bits | Dir | Logic | Function |
| --- | --- | --- | --- | --- |
| tck | 1 | In | LVDS | JTAG TCK |
| tms | 1 | In | LVDS | JTAG TMS |
| tdi | 1 | In | LVDS | JTAG TDI |
| chain_select | 2 | In | LVDS | Chain Select Address |
| tdo | 1 | Out | LVDS | JTAG TDO |

### LEDs & Testpoints

# Table 13: TMB Front Panel LEDs

| LED Label | Color | Function |
| --- | --- | --- |
| LCT | Blue | ALCT vpf and CLCT vpf match within the 75ns ALCT window (see note below) |
| ALCT | Green | ALCT active FEB flag (may not always be the same as ALCT valid pattern flag) |
| CLCT | Green | CLCT active FEB flag Or any external trigger except ALCT |
| L1A | Green | Level 1 Accept arrived in L1A window |
| INVP | Yellow | Invalid CLCT pattern after drift delay Pattern dropped below threshold, probably triggered on noise |
| NMAT | Yellow | ALCT vpf or CLCT vpf arrived but did not match in ALCT window |
| NL1A | Red | No Level 1 Accept arrived in L1A window after TMB triggered Constant flash rate = buffers full |
| VME | Green | ON = TMB FPGA loaded successfully from PROM Flashes OFF when module addressed by VME |

# Table 12A: Mez-2013 SMT LEDs

| LED Label | Color | Function |
| --- | --- | --- |
| D1 | Blue | When lit, indicates at least one fiber link had an error since last reset |
| D2 | Green | When lit, indicates TMB clock0 MMCM has locked since the last reset |
| D3 | Yellow | When lit, indicates TMB clock0 MMCM is not currently locked |
| D4 | Red | When lit, indicates TMB clock0 MMCM lost lock at least once since the last reset |
| D5 | Green | When lit, indicates the QPLL has locked since the last reset |
| D6 | Yellow | When lit, indicates the QPLL is not currently locked |
| D7 | Red | When lit, indicates the QPLL lost lock at least once since the last reset |
| D8 | Green | When lit, indicates at least one fiber link has a stable input |
| D0 | Yellow | When lit, indicated the FPGA is Not Programmed (i.e. DONE is False) |

# Table 12B: Mez-2013 Testpoints

| Testpoint Label | Function |
| --- | --- |
| 9 | TRUE indicates at least one fiber link had over 100 errors since the last reset |
| 8 | TRUE indicates all fiber links have a stable input |
| 7:1 | TRUE signas indicate that links 7:1 have a stable input, respectively |

### TMB Total I/O Count

# Table 14: TMB Total I/O Count

| Bits | Pins | Dir | Logic | Connect To | Function |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | In | LVDS | CCB | Clock Bus |
| 26 | 26 | In | GTLP | CCB | Fast Control Bus |
| 4 | 4 | In | GTLP | CCB | TMB Reload Bus |
| 12 | 12 | In | GTLP | CCB + 9 DMB + 9 TMB | DAQ Special Purpose Bus |
| 7 | 7 | In | GTLP | CCB + 9 TMB | Trigger Special Purpose Bus |
| 240 | 120 | In | LVDS | 5 CFEBs | CFEB Comparators |
| 57 | 29 | In | LVDS | ALCT | ALCT Module |
| 6 | 6 | In | LVTTL | 1 DMB | DMB commands |
| 2 | 1 | In | GTLP | MPC | MPC winner |
| 80 | 40 | In | LVTTL | RPC | RPC Inputs |
| 24 | 24 | In | TTL | VME | VME Address |
| 16 | 16 | BiDir | TTL | VME | VME Data |
| 5 | 5 | In | LVTTL | JTAG | JTAG |
| 480 | 292 |  |  |  | Totals |

| Bits | Pins | Dir | Logic | Connect To | Function |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Out | LVDS | CCB | Clock Bus |
| 0 | 0 | Out | GTLP | CCB | Fast Control Bus |
| 18 | 18 | Out | GTLP | CCB | TMB Reload Bus |
| 0 | 0 | Out | GTLP | CCB + 9 DMB + 9 TMB | DAQ Special Purpose Bus |
| 25 | 25 | Out | GTLP | CCB + 9 TMB | Trigger Special Purpose Bus |
| 5 | 5 | Out | LVDS | 5 CFEBs | CFEB Comparators |
| 33 | 21 | Out | LVDS | ALCT | ALCT Module |
| 50 | 50 | Out | LVTTL | 1 DMB | DMB data |
| 64 | 32 | Out | GTLP | MPC | MPC winner |
| 0 | 0 | Out | LVTTL | RPC | RPC Inputs |
| 0 | 0 | Out | TTL | VME | VME data is BiDir |
| 1 | 1 | Out | LVTTL | JTAG | JTAG |
| 196 | 152 |  |  |  | Totals |

# FPGA I/O Estimate: 392 in – 7 clock + 152 out = 537

# Connectors

### TMB Connector Summary

# Table 15: TMB2005 Connector Summary

| ID | Pins | Type | Function |
| --- | --- | --- | --- |
| J0 | 50 | SCSI-II | CFEB0 Inputs + Clock Out |
| J1 | 50 | SCSI-II | CFEB1 Inputs + Clock Out |
| J2 | 50 | SCSI-II | CFEB2 Inputs + Clock Out |
| J3 | 50 | SCSI-II | CFEB3 Inputs + Clock Out |
| J4 | 50 | SCSI-II | CFEB4 Inputs + Clock Out |
| J5 | 50 | SCSI-II | ALCT Cable 1 Inputs |
| J6 | 50 | SCSI-II | ALCT Cable 2 I/O |
| J7 | 10 | Header | Xilinx LVDS X-Blaster I/O |
| P1 | 160 | VME64x | VME J1/P1 Bus I/O |
| P2A | 125 | Z-Pack 25x5 | CCB + DMB I/O |
| P2B | 55 | Z-Pack 11x5 | DMB I/O |
| P3A | 55 | Z-Pack 11x5 | MPC I/O |
| P3B | 125 | Z-Pack 25x5 | RPC Inputs + ALCT alternate I/O |
| MTP Rx | - | aqua MTP with reduced flange & SC footprint | 10 Gbps panel-mount adapter to receive up to 12 fiber links carrying DCFEB comparator data to Mez-2013 boards |

### J0-J4 CFEB0-CFEB4 Connectors

# Function:		Receives 80MHz data from CFEBs. Transmits 40MHz clock.

Connector Type:	PCB:		AMP 787190-5

Cable:		AMP 749111-4

Shell:		AMP 749889-3 [with latches]

# Table 16: J0-J4 CFEB0/4-to-TMB Connectors

(This table uses Layer numbers Ly0-to-Ly5, Triad numbers Tr0-to-Tr7)

| Pair | Pin |  | Dir | Logic | Multiplexed Signals |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1+ | 2- | In | LVDS | Ly0Tr0 | Ly3Tr0 |
| 2 | 3+ | 4- | In | LVDS | Ly0Tr2 | Ly3Tr2 |
| 3 | 5+ | 6- | In | LVDS | Ly5Tr0 | Ly4Tr0 |
| 4 | 7+ | 8- | In | LVDS | Ly5Tr2 | Ly4Tr2 |
| 5 | 9+ | 10- | In | LVDS | Ly1Tr0 | Ly2Tr0 |
| 6 | 11+ | 12- | In | LVDS | Ly1Tr2 | Ly2Tr2 |
| 7 | 13+ | 14- | In | LVDS | Ly0Tr4 | Ly3Tr4 |
| 8 | 15+ | 16- | In | LVDS | Ly0Tr6 | Ly3Tr6 |
| 9 | 17+ | 18- | In | LVDS | Ly5Tr4 | Ly4Tr4 |
| 10 | 19+ | 20- | In | LVDS | Ly5Tr6 | Ly4Tr6 |
| 11 | 21+ | 22- | In | LVDS | Ly1Tr4 | Ly2Tr4 |
| 12 | 23+ | 24- | In | LVDS | Ly1Tr6 | Ly2Tr6 |
| 13 | 25+ | 26- | Out | LVDS | LCT_Clock |  |
| 14 | 27+ | 28- | In | LVDS | Ly1Tr7 | Ly2Tr7 |
| 15 | 29+ | 30- | In | LVDS | Ly1Tr5 | Ly2Tr5 |
| 16 | 31+ | 32- | In | LVDS | Ly5Tr7 | Ly4Tr7 |
| 17 | 33+ | 34- | In | LVDS | Ly5Tr5 | Ly4Tr5 |
| 18 | 35+ | 36- | In | LVDS | Ly0Tr7 | Ly3Tr7 |
| 19 | 37+ | 38- | In | LVDS | Ly0Tr5 | Ly3Tr5 |
| 20 | 39+ | 40- | In | LVDS | Ly1Tr3 | Ly2Tr3 |
| 21 | 41+ | 42- | In | LVDS | Ly1Tr1 | Ly2Tr1 |
| 22 | 43+ | 44- | In | LVDS | Ly5Tr3 | Ly4Tr3 |
| 23 | 45+ | 46- | In | LVDS | Ly5Tr1 | Ly4Tr1 |
| 24 | 47+ | 48- | In | LVDS | Ly0Tr3 | Ly3Tr3 |
| 25 | 49+ | 50- | In | LVDS | Ly0Tr1 | Ly3Tr1 |

### J5 ALCT Cable1 Connector (Receiver)

# Function:		Receives 80MHz data from ALCT.

Connector Type:	PCB:		AMP 787190-5

Cable:		AMP 749111-4

Shell:		AMP 749889-3 [with latches]

# Table 17: J5 ALCT Cable1 Connector [J10  on ALCT board]

Modified 4/12/01 to match ALCT2001 PCB. Stinking bad signal inversion =

| Pair | Inverted | Pin |  | Dir | Logic | Multiplexed Signals |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | + | - |  |  | First in Time | Second in Time |
| 1 |  | 1+ | 2- | In | LVDS | first_valid | second_valid |
| 2 |  | 49+ | 50- | In | LVDS | first_amu | second_amu |
| 3 |  | 3+ | 4- | In | LVDS | first_quality0 | second_quality0 |
| 4 |  | 47+ | 48- | In | LVDS | first_quality1 | second_quality1 |
| 5 |  | 5+ | 6- | In | LVDS | first_key0 | second_key0 |
| 6 |  | 45+ | 46- | In | LVDS | first_key1 | second_key1 |
| 7 |  | 7+ | 8- | In | LVDS | first_key2 | second_key2 |
| 8 |  | 43+ | 44- | In | LVDS | first_key3 | second_key3 |
| 9 |  | 9+ | 10- | In | LVDS | first_key4 | second_key4 |
| 10 |  | 41+ | 42- | In | LVDS | first_key5 | second_key5 |
| 11 |  | 11+ | 12- | In | LVDS | first_key6 | second_key6 |
| 12 |  | 39+ | 40- | In | LVDS | bxn0 | bxn3 |
| 13 |  | 13+ | 14- | In | LVDS | bxn1 | bxn4 |
| 14 |  | 37+ | 38- | In | LVDS | bxn2 | /wr_fifo |
| 15 |  | 15+ | 16- | In | LVDS | daq_data0 | daq_data7 |
| 16 |  | 35+ | 36- | In | LVDS | daq_data1 | daq_data8 |
| 17 |  | 17+ | 18- | In | LVDS | daq_data2 | daq_data9 |
| 18 |  | 33+ | 34- | In | LVDS | daq_data3 | daq_data10 |
| 19 |  | 19+ | 20- | In | LVDS | daq_data4 | daq_data11 |
| 20 |  | 31+ | 32- | In | LVDS | daq_data5 | daq_data12 |
| 21 |  | 21+ | 22- | In | LVDS | daq_data6 | daq_data13 |
| 22 |  | 29+ | 30- | In | LVDS | lct_special | first_frame |
| 23 |  | 23+ | 24- | In | LVDS | parity_out0 seq_status0 | parity_out2 seu_status0 |
| 24 |  | 27+ | 28- | In | LVDS | parity_out1 seq_status1 | parity_out3 seu_status1 |
| 25 |  | 25+ | 26- | In | LVDS | ddu_special | last_frame |

### J6 ALCT Cable2 Connector (Transmitter)

# Function:		Sends/Receives 80MHz data to/from ALCT.

Connector Type:	PCB:		AMP 787190-5

Cable:		AMP 749111-4

Shell:		AMP 749889-3 [with latches]

# Table 18: J6 ALCT Cable2 Connector [J11 on ALCT board]

Modified 4/12/01 to match ALCT2001 PCB. Stinking bad signal inversion

| Pair | Inverted | Pin |  | Dir | Logic | Multiplexed Signals |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | + | - |  |  | First in Time | Second in Time |
| 1 |  | 1+ | 2- | Out | LVDS | tdi |  |
| 2 |  | 49+ | 50- | Out | LVDS | tms |  |
| 3 |  | 3+ | 4- | Out | LVDS | tck |  |
| 4 |  | 47+ | 48- | Out | LVDS | jtag_select0 |  |
| 5 |  | 5+ | 6- | Out | LVDS | jtag_select1 |  |
| 6 |  | 45+ | 46- | Out | LVDS | ccb_brcst0 | ccb_brcst4 |
| 7 |  | 7+ | 8- | Out | LVDS | ccb_brcst1 | ccb_brcst5 |
| 8 |  | 43+ | 44- | Out | LVDS | ccb_brcst2 | ccb_brcst6 |
| 9 |  | 9+ | 10- | Out | LVDS | ccb_brcst3 | ccb_brcst7 |
| 10 |  | 41+ | 42- | Out | LVDS | brcst_str1 | subaddr_str |
| 11 |  | 11+ | 12- | Out | LVDS | dout_str | bx0 |
| 12 |  | 39+ | 40- | Out | LVDS | ext_inject | ext_trig |
| 13 |  | 13+ | 14- | Out | LVDS | level1_accept | sync_adb_pulse |
| 14 |  | 37+ | 38- | Out | LVDS | seq_cmd0 | seq_cmd2 |
| 15 |  | 15+ | 16- | Out | LVDS | seq_cmd1 | seq_cmd3 reserved_in4 |
| 16 |  | 35+ | 36- | Out | LVDS | parity_in0 reserved_in0 | parity_in2 reserved_in2 |
| 17 |  | 17+ | 18- | Out | LVDS | parity_in1 reserved_in1 | parity_in3 reserved_in3 |
| 18 |  | 33+ | 34- | Out | LVDS | async_adb_pulse |  |
| 19 |  | 19+ | 20- | Out | LVDS | /hard_reset |  |
| 20 |  | 31+ | 32- | Out | LVDS | clock_en |  |
| 21 |  | 21+ | 22- | Out | LVDS | clock |  |
| 22 |  | 29+ | 30- | In | LVDS | tdo |  |
| 23 |  | 23+ | 24- | In | LVDS | parity_out4 reserved_out0 | parity_out6 reserved_out2 |
| 24 |  | 27+ | 28- | In | LVDS | parity_out5 reserved_out1 | alct_bx0 reserved_out3 |
| 25 |  | 25+ | 26- | In | LVDS | active_feb_flag | cfg_done |

### J1-J6 SCSI-II 50-Pin Connector Pin Convention

# Figure 5: 50-Pin PCB Connector (Female)

(Looking into PCB Connector) 
------------------------------------- 
\ 25                                          1/
\ 26                                     50/ 
 ---------------------------------

# Figure 6: 50 Pin Cable Connector (Male)

(Looking into Cable Connector) 
------------------------------------- 
\ 1                                           25/ 
\ 50                                      26/ 
 ---------------------------------

# Figure 7: 50 Pin PCB Connector Pin Convention

(Looking At Top of PCB)

| 1 |  |  |  | 50 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 |  |  |  | 49 |  |
| 3 |  |  |  | 48 |  |  |  |
|  |  | 4 |  |  |  | 47 |  |
| 5 |  |  |  | 46 |  |  |  |
| Cable Side |  | 6 |  |  |  | 45 |  |
| 7 |  |  |  | 44 |  |  |  |
|  |  | 8 |  |  |  | 43 |  |
| 9 |  |  |  | 42 |  |  |  |
|  |  | 10 |  |  |  | 41 |  |
| 11 |  |  |  | 40 |  |  |  |
|  |  | 12 |  |  |  | 39 |  |
| 13 |  |  |  | 38 |  |  |  |
|  |  | 14 |  |  |  | 37 |  |
| 15 |  |  |  | 36 |  |  |  |
|  |  | 16 |  |  |  | 35 |  |
| 17 |  |  |  | 34 |  |  |  |
|  |  | 18 |  |  |  | 33 |  |
| 19 |  |  |  | 32 |  |  |  |
|  |  | 20 |  |  |  | 31 |  |
| 21 |  |  |  | 30 |  |  |  |
|  |  | 22 |  |  |  | 29 |  |
| 23 |  |  |  | 28 |  |  |  |
|  |  | 24 |  |  |  | 27 |  |
| 25 |  |  |  | 26 |  |  |  |

### J7 Xilinx LVDS Xilinx X-Blaster Connector

Function:		Connects TMB2005 to LVDS x-Blaster for programming FPGAs and PROMs.

The LVDS signals and voltage sources on this connector are not directly	compatible with the standard Xilinx programming cable.

JTAG chain select signals SEL[3:0] are TTL .

Connector Type:	PCB:		3M 3316-5002		16-pin right angle center key

Cable:		3M 3452-6600		16-pin center bump

# Table 19: J8 Xilinx LVDS X-Blaster Connector

| +TCK | In | 1 |  | 2 | In | -TCK |
| --- | --- | --- | --- | --- | --- | --- |
| +TDO | Out | 3 |  | 4 | Out | -TDO |
| +TMS | In | 5 |  | 6 | In | -TMS |
| +3.3V | Out | 7 |  | 8 | - | GND |
| +TDI | In | 9 |  | 10 | In | -TDI |
| +3.3V | In | 11 |  | 12 | In | JTAG_EN(TTL) |
| SEL0 (TTL) | Out | 13 |  | 14 | In | SEL1 (TTL) |
| SEL2 (TTL) | In | 15 |  | 16 | In | SEL3 (TTL) |

### P1 Backplane VME64x J1/P1 Connector

# Function:		VME interface.

Connector Type:	PCB:		Harting 02-02-160-2101 Male Right-Angle

Backplane:	Harting 02-01-160-2201 Female

Address bits:			24

Data bits:			16

Geographic Address bits:	5

# Table 20: P1 VME64x Connector

| Pin | Row z | Row a | Row b | Row c | Row d |
| --- | --- | --- | --- | --- | --- |
| 1 | MPR | D00 | BBSY* | D08 | VPC |
| 2 | GND | D01 | BCLR* | D09 | GND |
| 3 | MCLK | D02 | ACFAIL* | D10 | +V1 |
| 4 | GND | D03 | BG0IN* | D11 | +V2 |
| 5 | MSD | D04 | BG0OUT* | D12 | RsvU |
| 6 | GND | D05 | BG1IN* | D13 | -V1 |
| 7 | MMD | D06 | BG1OUT* | D14 | -V2 |
| 8 | GND | D07 | BG2IN* | D15 | RsvU |
| 9 | MCTL | GND | BG2OUT* | GND | GAP* |
| 10 | GND | SYSCLK | BG3IN* | SYSFAIL* | GA0* |
| 11 | RESP* | GND | BG3OUT* | BERR* | GA1* |
| 12 | GND | DS1* | BR0* | SYSRESET* | +3.3V |
| 13 | RsvBus1 | DS0* | BR1* | LWORD* | GA2* |
| 14 | GND | WRITE* | BR2* | AM5 | +3.3V |
| 15 | RsvBus2 | GND | BR3* | A23 | GA3* |
| 16 | GND | DTACK* | AM0 | A22 | +3.3V |
| 17 | RsvBus3 | GND | AM1 | A21 | GA4* |
| 18 | GND | AS* | AM2 | A20 | +3.3V |
| 19 | RsvBus4 | GND | AM3 | A19 | RsvBus11 |
| 20 | GND | IACK* | GND | A18 | +3.3V |
| 21 | RsvBus5 | IACKIN* | SERCLK | A17 | RsvBus12 |
| 22 | GND | IACKOUT* | SERDAT | A16 | +3.3V |
| 23 | RsvBus6 | AM4 | GND | A15 | RsvBus13 |
| 24 | GND | A07 | IRQ7* | A14 | +3.3V |
| 25 | RsvBus7 | A06 | IRQ6* | A13 | RsvBus14 |
| 26 | GND | A05 | IRQ5* | A12 | +3.3V |
| 27 | RsvBus8 | A04 | IRQ4* | A11 | LI/I* |
| 28 | GND | A03 | IRQ3* | A10 | +3.3V |
| 29 | RsvBus9 | A02 | IRQ2* | A09 | LI/O* |
| 30 | GND | A01 | IRQ1* | A08 | +3.3V |
| 31 | RsvBus10 | -12V | +5VSTDBY | +12V | GND |
| 32 | GND | +5V | +5V | +5V | VPC |

### P2A Backplane CCB+DMB Connector

# Function:		Sends and receives data to/from CCB, and carries some DMB signals.

Connector Type:	PCB:		AMP Z-Pack 125 (25 rows of 5 pins) female	AMP 100145-1

Backplane:	AMP Z-Pack 125 (25 rows of 5 pins) male	AMP ?

# Table 21: P2A Backplane CCB+DMB Connector

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| A1 | In | LVDS | ccb_clock40+ |
| A2 | In | GTLP | ccb_clock40_enable |
| A3 | In | GTLP | ccb_cmd0 |
| A4 | In | GTLP | ccb_cmb4 |
| A5 | In | GTLP | ccb_cmd_strobe |
| A6 | In | GTLP | ccb_data0 |
| A7 | In | GTLP | ccb_data4 |
| A8 | In | GTLP | ccb_reserved0 |
| A9 | In | GTLP | tmb_hard_reset |
| A10 | In | GTLP | alct_adb_pulse_sync |
| A11 | Out | GTLP | clct_status0 |
| A12 | Out | GTLP | clct_status4 |
| A13 | Out | GTLP | clct_status6 |
| A14 | Out | GTLP | alct_status3 |
| A15 | Out | GTLP | alct_status7 |
| A16 | Out | GTLP | tmb_reserved_in0 |
| A17 | Out | GTLP | tmb_reserved_in4 |
| A18 |  |  |  |
| A19 | In | GTLP | dmb_cfeb_calibrate0 |
| A20 | In | GTLP | dmb_reserved_out0 |
| A21 | In | GTLP | dmb_reserved_out4 |
| A22 |  |  |  |
| A23 | Out | LVTTL | tmb_data0 |
| A24 | Out | LVTTL | tmb_data4 |
| A25 | Out | LVTTL | tmb_data8 |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| B1 | In | LVDS | ccb_clock40- |
| B2 | In | GTLP | ccb_reserved4 |
| B3 | In | GTLP | ccb_cmd1 |
| B4 | In | GTLP | ccb_cmb5 |
| B5 | In | GTLP | ccb_bx0 |
| B6 | In | GTLP | ccb_data1 |
| B7 | In | GTLP | ccb_data5 |
| B8 | In | GTLP | ccb_reserved1 |
| B9 | In | GTLP | alct_hard_reset |
| B10 | In | GTLP | alct_adb_pulse_async |
| B11 | Out | GTLP | clct_status1 |
| B12 | Out | GTLP | clct_status5 |
| B13 | Out | GTLP | alct_status0 |
| B14 | Out | GTLP | alct_status4 |
| B15 | Out | GTLP | alct_status8 |
| B16 | Out | GTLP | tmb_reserved_in1 |
| B17 | In | GTLP | tmb_reserved_out0 |
| B18 |  |  |  |
| B19 | In | GTLP | dmb_cfeb_calibrate1 |
| B20 | In | GTLP | dmb_reserved_out1 |
| B21 | (In)1 | GTLP | dmb_reserved_in0 |
| B22 |  |  |  |
| B23 | Out | LVTTL | tmb_data1 |
| B24 | Out | LVTTL | tmb_data5 |
| B25 | Out | LVTTL | tmb_data9 |

# Pins C1 through C25 are connected to Backplane Ground

Notes:

- TMB can monitor signals to/from DMB, but can not assert them.

### P2A Backplane CCB+DMB Connector Continued

# Table 20: P2A Backplane CCB Connector Continued

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| D1 | Out | GTLP | tmb_config_done |
| D2 |  |  |  |
| D3 | In | GTLP | ccb_cmd2 |
| D4 | In | GTLP | ccb_evcntres |
| D5 | In | GTLP | ccb_l1accept |
| D6 | In | GTLP | ccb_data2 |
| D7 | In | GTLP | ccb_data6 |
| D8 | In | GTLP | ccb_reserved2 |
| D9 | In | GTLP | tmb_reserved0 |
| D10 | In | GTLP | clct_external_trigger |
| D11 | Out | GTLP | clct_status2 |
| D12 | Out | GTLP | clct_status6 |
| D13 | Out | GTLP | clct_status1 |
| D14 | Out | GTLP | alct_status5 |
| D15 | Out | GTLP | tmb_l1a_request |
| D16 | Out | GTLP | tmb_reserved_in2 |
| D17 | In | GTLP | tmb_reserved_out1 |
| D18 |  |  |  |
| D19 | In | GTLP | dmb_cfeb_calibrate2 |
| D20 | In | GTLP | dmb_reserved_out2 |
| D21 | (In)1 | GTLP | dmb_reserved_in1 |
| D22 |  |  |  |
| D23 | Out | LVTTL | tmb_data2 |
| D24 | Out | LVTTL | tmb_data7 |
| D25 | Out | LVTTL | tmb_data11 |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| E1 | Out | GTLP | alct_config_done |
| E2 |  |  |  |
| E3 | In | GTLP | ccb_cmd3 |
| E4 | In | GTLP | ccb_bcntres |
| E5 | In | GTLP | ccb_data_strobe |
| E6 | In | GTLP | ccb_data3 |
| E7 | In | GTLP | ccb_data7 |
| E8 | In | GTLP | ccb_reserved3 |
| E9 | In | GTLP | tmb_reserved1 |
| E10 | In | GTLP | alct_external_trigger |
| E11 | Out | GTLP | clct_status3 |
| E12 | Out | GTLP | clct_status7 |
| E13 | Out | GTLP | alct_status2 |
| E14 | Out | GTLP | alct_status6 |
| E15 | Out | GTLP | tmb_l1a_release |
| E16 | Out | GTLP | tmb_reserved_in3 |
| E17 | In | GTLP | tmb_reserved_out2 |
| E18 |  |  |  |
| E19 | (In)1 | GTLP | dmb_l1a_release |
| E20 | In | GTLP | dmb_reserved_out3 |
| E21 | (In)1 | GTLP | dmb_reserved_in2 |
| E22 |  |  |  |
| E23 | Out | LVTTL | tmb_data3 |
| E24 | Out | LVTTL | tmb_data7 |
| E25 | Out | LVTTL | tmb_data11 |

Notes:

1) TMB can monitor signals to/from DMB, but can not assert them.

### P2B Backplane DMB Connector

# Function:		Sends and receives data to/from DMB.

Connector Type:	PCB:		AMP Z-Pack 55 (11 rows of 5 pins) female	AMP 100161-1

Backplane:	AMP Z-Pack 55 (11 rows of 5 pins) male	AMP ?

# Table 22: P2B Backplane DMB Connector

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| A1 | Out | LVTTL | tmb_data12 |
| A2 | Out | LVTTL | alct_data1 |
| A3 | Out | LVTTL | alct_data5 |
| A4 | Out | LVTTL | alct_data9 |
| A5 | Out | LVTTL | alct_data13 |
| A6 | Out | LVTTL | tmb_data_available |
| A7 | Out | LVTTL | tmb_active_feb1 |
| A8 | Out | LVTTL | fifo_clock |
| A9 | Out | LVTTL | alct_last_frame |
| A10 | In | LVTTL | res_from_dmb2 |
| A11 | Out | LVTTL | tmb_active_feb6 (v6) |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| D1 | Out | LVTTL | tmb_data14 |
| D2 | Out | LVTTL | alct_data3 |
| D3 | Out | LVTTL | alct_data7 |
| D4 | Out | LVTTL | alct_data11 |
| D5 | Out | LVTTL | tmb_ddu_special |
| D6 | Out | LVTTL | tmb_active_feb_flag |
| D7 | Out | LVTTL | tmb_active_feb3 |
| D8 | Out | LVTTL | /alct_write_enable_fifo |
| D9 | In | LVTTL | res_from_dmb1 |
| D10 | Out | LVTTL | res_to_dmb5 |
| D11 | Out | LVTTL | res_to_dmb4 |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| B1 | Out | LVTTL | tmb_data13 |
| B2 | Out | LVTTL | alct_data2 |
| B3 | Out | LVTTL | alct_data6 |
| B4 | Out | LVTTL | alct_data10 |
| B5 | Out | LVTTL | alct_data14 |
| B6 | Out | LVTTL | /tmb_write_enable_fifo |
| B7 | Out | LVTTL | tmb_active_feb2 |
| B8 | In | LVTTL | dmb_request_lct |
| B9 | In | LVTTL | dmb_ext_trig |
| B10 | In | LVTTL | res_from_dmb3 |
| B11 | Out | LVTTL | res_to_dmb3 |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| E1 | Out | LVTTL | alct_data0 |
| E2 | Out | LVTTL | alct_data4 |
| E3 | Out | LVTTL | alct_data8 |
| E4 | Out | LVTTL | alct_data12 |
| E5 | Out | LVTTL | tmb_last_frame |
| E6 | Out | LVTTL | tmb_active_feb0 |
| E7 | Out | LVTTL | tmb_active_feb4 |
| E8 | Out | LVTTL | alct_ddu_special |
| E9 | Out | LVTTL | alct_data_available |
| E10 | Out | LVTTL | tmb_active_feb5 (v6) |
| E11 | In | LVTTL | dmb_fpga_pgm_done |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| C1 | In | Pwr | Gnd |
| C2 | In | Pwr | VTT (+1.5V) |
| C3 | In | Pwr | Gnd |
| C4 | In | Pwr | VTT |
| C5 | In | Pwr | Gnd |
| C6 | In | Pwr | VTT |
| C7 | In | Pwr | Gnd |
| C8 | In | Pwr | VTT |
| C9 | In | Pwr | Gnd |
| C10 | In | Pwr | VTT |
| C11 | In | Pwr | Gnd |

### P3A Backplane MPC Connector

# Function:		Sends and receives data to/from MPC.

Connector Type:	PCB:		AMP Z-Pack 55 (11 rows of 5 pins) female	AMP 100161-1

Backplane:	AMP Z-Pack 55 (11 rows of 5 pins) male	AMP ?

# Table 23: P3A Backplane MPC Connector

See ADR_MPCx_FRAMEx on p49 for signal assignments

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| A1 | Out | GTLP | /mpc_out0 |
| A2 | Out | GTLP | /mpc_out4 |
| A3 | Out | GTLP | /mpc_out8 |
| A4 | Out | GTLP | /mpc_out12 |
| A5 | Out | GTLP | /mpc_out16 |
| A6 | Out | GTLP | /mpc_out20 |
| A7 | Out | GTLP | /mpc_out24 |
| A8 | Out | GTLP | /mpc_out28 |
| A9 | In | GTLP | lct_winner |
| A10 |  |  |  |
| A11 |  |  |  |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| D1 | Out | GTLP | /mpc_out2 |
| D2 | Out | GTLP | /mpc_out6 |
| D3 | Out | GTLP | /mpc_out10 |
| D4 | Out | GTLP | /mpc_out14 |
| D5 | Out | GTLP | /mpc_out18 |
| D6 | Out | GTLP | /mpc_out22 |
| D7 | Out | GTLP | /mpc_out26 |
| D8 | Out | GTLP | /mpc_out30 |
| D9 |  |  |  |
| D10 |  |  |  |
| D11 |  |  |  |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| B1 | Out | GTLP | /mpc_out1 |
| B2 | Out | GTLP | /mpc_out5 |
| B3 | Out | GTLP | /mpc_out9 |
| B4 | Out | GTLP | /mpc_out13 |
| B5 | Out | GTLP | /mpc_out17 |
| B6 | Out | GTLP | /mpc_out21 |
| B7 | Out | GTLP | /mpc_out25 |
| B8 | Out | GTLP | /mpc_out29 |
| B9 |  |  |  |
| B10 |  |  |  |
| B11 |  |  |  |

| Pin | Dir | Logic | Signal |
| --- | --- | --- | --- |
| E1 | Out | GTLP | /mpc_out3 |
| E2 | Out | GTLP | /mpc_out7 |
| E3 | Out | GTLP | /mpc_out11 |
| E4 | Out | GTLP | /mpc_out15 |
| E5 | Out | GTLP | /mpc_out19 |
| E6 | Out | GTLP | /mpc_out23 |
| E7 | Out | GTLP | /mpc_out27 |
| E8 | Out | GTLP | /mpc_out31 |
| E9 |  |  |  |
| E10 |  |  |  |
| E11 |  |  |  |

# Pins C1 through C11 are connected to Backplane Ground

### P3B Backplane RPC+ALCT Connector

# Function:		Sends and receives data to/from ALCT, and receives from RPC.

Connector Type:	PCB:		AMP Z-Pack 125 (25 rows of 5 pins) female	AMP 100145-1

Backplane:	AMP Z-Pack 125 (25 rows of 5 pins) male	AMP ?

# Table 24: P3B Backplane RPC+ALCT Connector

| Pin | Signal |  | Pin | Signal |  | Pin | Signal |  | Pin | Signal |  | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | rpc_rx31 |  | B1 | rpc_rx30 |  | C1 | rpc_rx29 |  | D1 | rpc_rx28 |  | E1 | rpc_rx27 |
| A2 | rpc_rx26 |  | B2 | rpc_rx25 |  | C2 | rpc_rx24 |  | D2 | rpc_rx23 |  | E2 | rpc_rx22 |
| A3 | rpc_rx21 |  | B3 | rpc_rx20 |  | C3 | rpc_rx19 |  | D3 | rpc_rx18 |  | E3 | rpc_rx17 |
| A4 | rpc_rx16 |  | B4 | rpc_rx15 |  | C4 | rpc_rx14 |  | D4 | rpc_rx13 |  | E4 | rpc_rx12 |
| A5 | rpc_rx11 |  | B5 | rpc_rx10 |  | C5 | rpc_rx9 |  | D5 | rpc_rx8 |  | E5 | rpc_rx7 |
| A6 | rpc_rx6 |  | B6 | rpc_rx5 |  | C6 | rpc_rx4 |  | D6 | rpc_rx3 |  | E6 | rpc_rx2 |
| A7 | rpc_rx1 |  | B7 | rpc_rx0 |  | C7 | rpc_clock |  | D7 | smb_clk |  | E7 | tck |
| A8 | tms |  | B8 | tdi |  | C8 | rpc_loop |  | D8 | posneg |  | E8 | sync |
| A9 | tdo |  | B9 | smbrx |  | C9 | rpc_in37 |  | D9 | rpc_in36 |  | E9 | rpc_in35 |
| A10 | rpc_in34 |  | B10 | rpc_in33 |  | C10 | rpc_in32 |  | D10 |  |  | E10 | +3.3V |
| A11 | +3.3V |  | B11 | +3.3V |  | C11 | GND |  | D11 | GND |  | E11 | GND |
| A12 | alct_rx31 |  | B12 | alct_rx30 |  | C12 | alct_rx29 |  | D12 | alct_rx28 |  | E12 | alct_rx27 |
| A13 | alct_rx26 |  | B13 | alct_rx25 |  | C13 | alct_rx24 |  | D13 | alct_rx23 |  | E13 | alct_rx22 |
| A14 | alct_rx21 |  | B14 | alct_rx20 |  | C14 | alct_rx19 |  | D14 | alct_rx18 |  | E14 | alct_rx17 |
| A15 | alct_rx16 |  | B15 | alct_rx15 |  | C15 | alct_rx14 |  | D15 | alct_rx13 |  | E15 | alct_rx12 |
| A16 | alct_rx11 |  | B16 | alct_rx10 |  | C16 | alct_rx9 |  | D16 | alct_rx8 |  | E16 | alct_rx7 |
| A17 | alct_rx6 |  | B17 | alct_rx5 |  | C17 | alct_rx4 |  | D17 | alct_rx3 |  | E17 | alct_rx2 |
| A18 | alct_rx1 |  | B18 | alct_rx0 |  | C18 | smbtx |  | D18 | hrst_rpc |  | E18 | free_tx0 |
| A19 | alct_oe |  | B19 | alct_clock |  | C19 | alct_clk_en |  | D19 | txoe |  | E19 | alct_loop |
| A20 | alct_tx23 |  | B20 | alct_tx22 |  | C20 | alct_tx21 |  | D20 | alct_tx20 |  | E20 | alct_tx19 |
| A21 | alct_tx18 |  | B21 | alct_tx17 |  | C21 | alct_tx16 |  | D21 | alct_tx15 |  | E21 | alct_tx14 |
| A22 | alct_tx13 |  | B22 | alct_tx12 |  | C22 | alct_tx11 |  | D22 | alct_tx10 |  | E22 | alct_tx9 |
| A23 | alct_tx8 |  | B23 | alct_tx7 |  | C23 | alct_tx6 |  | D23 | alct_tx5 |  | E23 | alct_tx4 |
| A24 | alct_tx3 |  | B24 | alct_tx2 |  | C24 | alct_tx1 |  | D24 | alct_tx0 |  | E24 | +1.8V |
| A25 | +1.8V |  | B25 | +1.8V |  | C25 | GND |  | D25 | GND |  | E25 | GND |

### Backplane Pin Diagram

### Front Panel Connector Locations

--------------------------

# Front Panel   |

| Multipurpose connector goes here

| ----

||. . | CFEB0

|| . .|

||. . |

|| . .|

||. . |

----- || . .| <- Pin 1 here (10.510)

Pin      ^   | ----

one      |   |

|   | ----

|   ||. . |

to   2.130” || . .|

|   ||. . |

Pin      |   || . .|

one     \/   ||. . |

----- || . .| <- Pin 1 here (8.380)

Pin      ^   | ----

one      |   |

|   | ----

|   ||. . |

to   2.130” || . .|

|   ||. . |

Pin      |   || . .|

one     \/   ||. . |

----- || . .| <- Pin 1 here (6.250)

Pin      ^   | ----

one      |   |

|   | ----

|   ||. . |

to   2.130” || . .|

|   ||. . |

Pin      |   || . .|

one     \/   ||. . |

----- || . .| <- Pin 1 here (4.120)

Pin      ^   | ----

one      |   |

|   | ----

|   ||. . |

to   2.130” || . .|

|   ||. . |

Pin      |   || . .|

one     \/   ||. . |

----- || . .| <- Pin 1 here (moved up from previous position 4/11/2001)

Pin      ^   | ----

one      |   |

to   1.990” |  Optical output here

bottom    |   |

edge    \/   |

----- ----------------------------------------

### CCB Front Panel

CCB Input connector P10

| Pin (+) | Pin (-) | Signal |
| --- | --- | --- |
| 1 | 2 | external_clock40 |
| 3 | 4 | external_clock40_enable |
| 5 | 6 | external_l1accept |
| 7 | 8 | dmb_cfeb_calibrate[0] |
| 9 | 10 | dmb_cfeb_calibrate[1] |
| 11 | 12 | dmb_cfeb_calibrate[2] |
| 13 | 14 | alct_adb_pulse_sync |
| 15 | 16 | alct_adb_pulse_async |
| 17 | 18 | clct_external_trigger |
| 19 | 20 | alct_external_trigger |
| 21 | 22 | tmb_l1a_request |
| 23 | 24 | ccb_fp_reserved_in[0] |
| 25 | 26 | ccb_fp_reserved_in[1] |
| 27 | 28 |  |
| 29 | 30 |  |
| 31 | 32 |  |
| 33 | 34 |  |

CCB Output connector P11

| Pin (+) | Pin (-) | Signal | Test Point | TMB Assignment | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | clct_status[0] | 391-1 | pretrig | Sequencer pre-triggered |
| 3 | 4 | clct_status[1] | 391-2 | seq_busy | Sequencer busy |
| 5 | 6 | clct_status[2] | 391-3 | invpat | Invalid pattern after drift delay |
| 7 | 8 | clct_status[3] | 391-4 | daqmb | Dump to DMB in progress |
| 9 | 10 | clct_status[4] | 391-5 | l1a_window | L1A window |
| 11 | 12 | clct_status[5] | 391-6 | l1a | L1A (should be in L1A window) |
| 13 | 14 | clct_status[6] | 391-7 | tmb | CLCT sent for TMB match |
| 15 | 16 | clct_status[7] | 391-8 | tmb_flush | TMB found no match or rejected trigger |
| 17 | 18 | clct_status[8] | 391-9 | nol1a_flush | No L1A, Sequencer flushing event |
| 19 | 20 | ccb_clock40 |  |  |  |
| 21 | 22 | ccb_bx0 |  |  |  |
| 23 | 24 | ccb_l1accept |  |  |  |
| 25 | 26 | ccb_cmdstr |  |  |  |
| 27 | 28 | ccb_fp_reserved_out[0] |  |  |  |
| 29 | 30 |  |  |  |  |
| 31 | 32 |  |  |  |  |
| 33 | 34 |  |  |  |  |

CCB Output connector P12

| Pin (+) | Pin (-) | Signal | Test Point | TMB Assignment | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | alct_status[0] | 392-1 | alct_active_feb | ALCT fast active AFEB |
| 3 | 4 | alct_status[1] | 392-2 | first_valid | First muon valid pattern flag |
| 5 | 6 | alct_status[2] | 392-3 | second_valid | Second muon valid pattern flag |
| 7 | 8 | alct_status[3] | 392-4 | first_amu | First accelerator muon flag |
| 9 | 10 | alct_status[4] | 392-5 | second_amu | Second accelerator muon flag |
| 11 | 12 | alct_status[5] | 392-6 | /wr_fifo(alct) | ALCT /write enable raw-hit FIFO |
| 13 | 14 | alct_status[6] | 392-7 | alct_vpf | ALCT 1st Valid Pattern (TMB pipe) |
| 15 | 16 | alct_status[7] | 392-8 | clct_vpf | CLCT 1st Valid Pattern (TMB pipe) |
| 17 | 18 | alct_status[8] | 392-9 | scint_veto | Scintillator Veto (clears via VME) |
| 19 | 20 |  |  |  |  |
| 21 | 22 |  |  |  |  |
| 23 | 24 |  |  |  |  |
| 25 | 26 |  |  |  |  |
| 27 | 28 |  |  |  |  |
| 29 | 30 |  |  |  |  |
| 31 | 32 |  |  |  |  |
| 33 | 34 |  |  |  |  |

# Documentation Revision History

| Version | Date | Action |
| --- | --- | --- |
| # 1.00 | # 07/12/2003 | # Initial, copied from TMB2001 |
| 1.01 | 10/29/2003 | Replaced PHOS4 registers with DDD, update shunts table, rat signals updated |
| 1.02 | 03/15/2004 | Update 3D3444 registers, add RAT/RPC registers |
| 1.03 | 03/16/2004 | Move RAT register addy to match bdtest.v |
| 1.04 | 04/15/2004 | Copy from TMB2003 |
| 2.01 | 05/19/2004 | Add RPC Readout |
| 2.01 | 06/09/2004 | New registers B6-CC, 4 new header words, new raw hits format, started logic docs |
| 2.02 | 06/10/2004 | Add mpc_tx_delay, modify header 21,22 |
| 2.03 | 06/18/2004 | Add register CE for new scope trigger source, add mpc data format |
| 2.04 | 07/23/2004 | Add nph_pattern to header word 21 |
| 2.05 | 08/03/2004 | Add counter registers |
| 2.06 | 10/01/2004 | Add scp bit to header |
| 2.07 | 10/04/2004 | Add mpc_bx0 to reg ADR_TMB_TRIG |
| 2.08 | 10/07/2004 | Typos fixed |
| 2.09 | 12/15/2004 | Typos fixed |
| 3.00 | 06/02/2005 | Ported from TMB2004 v2.09 dated 12/15/04 |
| 3.00 | 06/09/2005 | Add rpc_hard_reset to boot reg, update ADR_LOOPBK for RAT signals |
| 3.01 | 06/28/2005 | alct_rxoe, alct_txoe in 0E are now readonly to foil errant software |
| 3.02 | 08/01/2005 | Change default alct_status_en, tmb_status_en=0 for multi-crate use |
| 3.03 | 08/08/2005 | Change c_status_oe to ccb_status_oe |
| 3.04 | 02/27/2006 | Add configuration section |
| 3.05 | 05/24/2006 | Add jtag state machine registers, add alct scope channels |
| 3.06 | 07/28/2006 | Add vme state machine registers, update raw hits header, add prom info |
| 3.07 | 08/08/2006 | Add layer-or trigger mode |
| 3.08 | 09/05/2006 | Add rat delay registers |
| 3.09 | 09/15/2006 | Add CLCT processing steps |
| 3.10 | 10/16/2006 | Add ISE Version register, triad persistence default now 6 for 6 clocks |
| 3.11 | 04/09/2007 | Reduce RPCs from 4 to 2, add mpc_oe, affects 86,B6,BA,BE,C4,CA,CC,header22 |
| 3.12 | 04/11/2007 | Remove duplicate header22 description, update header tables |
| 3.13 | 04/27/2007 | Add ignore ttc_start/stop to Adr 2C, expand fmm_state in Adr 9C |
| 3.14 | 05/16/2007 | New pattern finder registers |
| 4.01 | 05/24/0207 | New clct pattern finder algorithm |
| 4.02 | 06/21/2007 | New pattern ID numbers, new layer triggermods to Adrs 10,D4, and A0,A6,A8,AA,CA,CC,header16 |
| 4.03 | 07/05/2007 | Key layer shifted from ly3 to ly2, adjcfeb_dist expanded to 6 bits, activefeb bugfix |
| 4.04 | 07/10/2007 | Firmware update, no change to doc |
| 4.05 | 07/24/2007 | Update Adr 78,88,8C descriptions to match current CLCT0/1 4-bit pattern IDs |
| 4.06 | 07/26/2007 | Fix text for adr 68, 6C, A6 |
| 4.07 | 08/06/2007 | Increase injector read-write adr to 10 bins in Adr 44, Adr BE |
| 4.08 | 08/30/2007 | New header/trailer format |
| 4.09 | 09/04/2007 | Mods to header format, expand VME counters to 30 bits |
| 4.10 | 09/10/2007 | New fields in header 24, new VME counters |
| 4.11 | 09/14/2007 | Header bug fixes, new no-alct counter |
| 4.12 | 09//21/2007 | Push hsds bit into pat[3] in documentation |
| 4.13 | 12/17/2007 | Switch to big-buffer logic |
| 4.14 | 01/28/2008 | Replace entire L1A logic, replace lct_ quality |
| 4.15 | 02/05/2008 | Add raw hits RAM parity errors to header27 |
| 4.16 | 04/29/2008 | Major update for new flow-through triggering |
| 4.17 | 05/01/2008 | All scope channels reassigned, header frame bug fixes |
| 4.18 | 05/12/2008 | Add clct + alct bx0 heartbeat to mpc_tx[11], mpc_tx[27] |
| 4.19 | 05/23/2008 | Adr B0[15:14] Add clock_lock_lost and sync_er |
| 4.20 | 06/03/2008 | Add counter clears on ttc_resync, add aclt debug to scope, new active_feb_flag |
| 4.21 | 07/09/2008 | Replace jtag state machine, rename thresh signals, add pid_thresh_postdrift |
| 4.22 | 07/15/2008 | non-trigger readout mods |
| 4.23 | 08/12/2008 | Add bx0_match, add me1a/b separation ram mux, add alct data tx delay |
| 4.24 | 08/28/2008 | Programmable stagger removed, csc firmware types created, new counters |
| 4.25 | 09/12/2008 | Add tmb logic signals to header40,41 |
| 4.26 | 09/16/2008 | Update tables for header words 36(rpc),40,41. Updated sample raw hits dump |
| 4.27 | 10/01/2008 | Mod header08,36,40,41, replaced scope module and vme register, jtag sel |
| 4.28 | 11/18/2008 | Add reg FE, L1A and parity logic mods, add sync err and parity err counters |
| 4.29 | 11/19/2008 | Remove stagger_csc from Adr F4[1], staggering info is in Adr CC |
| 4.30 | 12/10/2008 | Add l1a_lookback mode, l1a_lookback reg + sequencer debug reg; add parity ram |
| 4.31 | 02/05/2009 | Add alct-loopback logic in alct.v module |
| 4.32 | 03/16/2009 | Add ecc to alct data, add ecc counters, add cfeb counters |
| 4.33 | 04/15/2009 | Updates for alct 80MHz output stages |
| 4.34 | 05/15/2009 | Miniscope added, RPC readout mods, bx0 test mode |
| 4.35 | 05/29/2009 | Add alct muonic timing |
| 4.36 | 06/22/2009 | More alct muonic mods |
| 4.37 | 06/29/2009 | Replace alct_txd, alct_rxd DDD delays with digital phase shifters |
| 4.38 | 07/13/2009 | Add cfeb muonic timing, 5 digital phase shifters, 2 vme delay registers |
| 4.39 | 08/14/2009 | CFEB muonic version, removed cfeb and alct_rx posnegs, still has alct_tx posneg |
| 4.40 | 08/25/2009 | ALCT+CFEB muonic with posneg sync stages, ISE 8.2sp3 |
| 4.41 | 09/02/2009 | Correct adr 16 |
| 4.42 | 09/08/2009 | Add digital phase shifter autostart |
| 4.43 | 09/21/2009 | Add sync register 0x120, limit bxn offsets to be < lhc_cycle |
| 4.44 | 10/15/2009 | Add alct0==alct1 error counter, change D0[5] default to 1, enables alct err counter |
| 4.45 | 11/23/2009 | Text corrections |
| 4.46 | 12/09/2009 | Add alct and cfeb muonic figures |
| 4.47 | 12/16/2009 | Add cfeb bad bit detection and registers |
| 4.48 | 01/14/2010 | Add cfeb bad bit list to header30, mod bad bit ctrl reg |
| 4.49 | 02/10/2010 | Add event clear for aff, clct, mpc VME diagnostic registers |
| 4.50 | 03/12/2010 | Add blocked bits readout, clct-only mode bug fix |
| 4.51 | 04/09/1200 | Fix header30 typo, add firmware log for 3/19/2010 version |
| 4.52 | 05/14/2010 | Mod Adr 0x2A and Adr 0x120, add firmware log for 5/14/10 version |
| 4.53 | 07/02/2010 | Mod Adr 0x32, 0x42, 0x9E, 0x100, and hdr38[14] |
| 4.54 | 07/08/2010 | Move injector ram msbs to lookback reg, revert injector wen,ren |
| 4.55 | 07/15/2010 | Add miniscope section to header chapter |
| 4.56 | 09/03/2010 | Convert to MS Word 2010 |
| 4.57 | 08/17/2012 | Add startup delay for ALCT Spartan-6 mezzanine |
| 4.58 | 09/16/2012 | Add Virtex-6 GTX optical receivers |
| 4.59 | 09/23/2012 | Add Virtex-6 Sysmon |
| 4.60 | 10/15/2012 | Change pid_thresh descriptions in adr 0xF4 |
| 4.61 | 03/07/2013 | Convert doc to docx, mod header + add registers for 7 DCFEB Virtex-6 |
| 4.62 | 2013/10/24 | Change order of keys in LCT quality module, documentation updated accordingly |
| 5.01 | 2013/12/19 | Created assignments for Mez-2013 TPs & SMT LEDs: qpll & mmcm lock monitoring. Also minor corrections for VME GTX register table (Adr 14C-158) |
| 5.02 | 2014/01/06 | Changes to Reset for CFEB badbits, fiber link monitor logic & GTX VME registers 14A-158 |
| 5.03 | 2014/01/08 | Add optical links to Fig. 1 and made some minor format changes for some tables |

# Firmware Change Log

Version 01/06/2014

Improved control & monitoring for Virtex-6 SNAP12 GTX fiber optic receivers

First applied in firmware version 12302013

- TTC_Resync now included in reset logic for CFEB badbits

cfeb_badbits_reset[4:0]  = (cfeb_badbits_ctrl_wr[4:0] | {5{ttc_resync}});

cfeb_badbits_reset[6:5]  = (cfeb_v6_badbits_ctrl_wr[1:0] | {2{ttc_resync}});

- GTX VME registers 14A-158 changed significantly to reflect improved fiber link monitor & control logic

Removed functions that were not useful (previously accessed in bits 10, 7:4, 2)

Reassigned some other functions to different bits (previous bit 9  bit 7, bit 3  bit 2)

Added new functions to bits 6:3 (gtx_link_bad, gtx_link_had_err, gtx_link_good, gtx_rx_sync_done)

Changed the readout response for bits 2:0 in registers 14C – 158 when master GTX control is asserted

When the master GTX control bits are set in register 14A (bits 2:0) they are now accurately reflected in the respective readout bits for the individual GTX links (bitwise OR in each GTX)

The error count now counts link sync failures when the PRBS function is not enabled

An occassional loss of sync for a link is expected due to SEUs, but large numbers of them or repeated occurrences may indicate a problem in the system

Increased the error count size to 8 bits and assigned this to bits 15:8

For registers 14C – 158 the maximum error count is hex E0 (n.b.  a count larger than 1 is severe)

Register 14A is the sum of the other 7 counters, with maximum allowed value of hex FE

Version 12/19/2013

Adds diagnostic features for Mez-2013

Applied in firmware versions 12162013 – 12172013

- Assign Mez-2013 SMT LEDs to indicate qpll & mmcm lock conditions and status changes

See Table 12A

- Assign additional diagnostic signals to the Mez-2013 test points and SMT LEDs

See Tables 12A and 12B

- Minor corrections to the table for VME GTX registers (Adr 14C-158)

Removed references to “all”

Version 03/08/2013

Adds support for 7 DCFEBs with Virtex-6

1) Modified ADR_SEQCLCTM=0xB0  Sequencer CLCT msbs Register, Readonly

Added seq_clctmsb_rd[9:8] = clctf_vme[6:5],  bits were formerly unused

Now   seq_clctmsb_rd[9:3]  = clctf_vme[6:0]   Active cfeb list at TMB match

2) ADR_PARITY=0xFA Parity errors

Changed sub-address assignments to accommodate DCFEB[6:5]

3) Modified ADR_CNT_CTRL=0xD0  Status Counter Control

Inserted 2 new counters after cnt[18], shifts subsequent counter addresses by 2

cnt_en[19]	<= cfeb_hit_at_pretrig[5]	CLCT pretrigger is on CFEB5

cnt_en[20]	<= cfeb_hit_at_pretrig[6]	CLCT pretrigger is on CFEB6

4) Modified  ADR_DELAY1_INT=0x11E DDR Interstage delays for DCFEB[6:5]

Added:

cfeb5_rxd_int_delay[3:0] = delay1_int_wr[7:4];

cfeb6_rxd_int_delay[3:0] = delay1_int_wr[11:8];

5) Added ADR_V6_CFEB_BADBITS_CTRL=0x15C CFEB  Bad Bits Control/Status

For DCFEB[6:5] badbits, extends 5 bit fields from ADR_CFEB_BADBITS_CTRL

6) Added 6 new bad bit VME registers

ADR_V6_CFEB5_BADBITS_LY01	= 0x15E   CFEB5 Bad Bit Array

ADR_V6_CFEB5_BADBITS_LY23	= 0x160   CFEB5 Bad Bit Array

ADR_V6_CFEB5_BADBITS_LY45	= 0x162   CFEB5 Bad Bit Array

ADR_V6_CFEB6_BADBITS_LY01	= 0x164   CFEB6 Bad Bit Array

ADR_V6_CFEB6_BADBITS_LY23	= 0x166   CFEB6 Bad Bit Array

ADR_V6_CFEB6_BADBITS_LY45	= 0x168   CFEB6 Bad Bit Array

7) Added 2 new Digital Phase Shifter VME registers

ADR_V6_PHASER7=0x16A   DCM Phase Shifter Register: CFEB5 rxd

ADR_V6_PHASER8=0x16C   DCM Phase Shifter Register: CFEB6 rxd

8)  Added 6 New hot channel mask registers

ADR_V6_HCM501 = 0x16E	CFEB5 Ly0,Ly1 Hot Channel Mask

ADR_V6_HCM523 = 0x170	CFEB5 Ly2,Ly3 Hot Channel Mask

ADR_V6_HCM545 = 0x172	CFEB5 Ly4,Ly5 Hot Channel Mask

ADR_V6_HCM601 = 0x174	CFEB6 Ly0,Ly1 Hot Channel Mask

ADR_V6_HCM623	= 0x176 CFEB6 Ly2,Ly3 Hot Channel Mask

ADR_V6_HCM645	= 0x178 CFEB6 Ly4,Ly5 Hot Channel Mask

9) Modified ADR_MOD_CFG 0x28 TMB Module Configuration Register

Replaced led_flash_rate from  mod_cfg_wr[11:10] with cfeb_exists[6:5]

Now: mod_cfg_rd[11:5] =  cfeb_exists[6:0] = CFEBs instantiated in this firmware

10) Added new VME register ADR_V6_EXTEND = 0x17A:  DCFEB 7-bit extensions

Extends 5-bit cfeb fields in Adr 0x42 and 0x68  to 7 bits

mask_all[6:5]          =[1:0] Extend 0x42[4:0]   = mask_all[4:0]1=Enable, 0=Turn off all CFEB inputs

inj_febsel[6:5]        =[3:2] Extend 0x42[9:5]   = inj_febsel[4:0]1=Select CFEBn for RAM read/write

injector_mask_cfeb[6:5]=[5:4] Extend 0x42[14:10] = injector_mask_cfeb[4:0] 1=Enable CFEB(n) for injector

cfeb_en_vme[6:5]       =[7:6] Extend 0x68[14:10] = cfeb_en_vme[4:0] 1=Enable CFEBs for triggering

11) Modified ADR_LOOPBK 0x0E	Loop-Back Control Register 2 remove 2 spare DMB signals

Reassigned dmb_tx_reserved to remove the 2 new active feb bits from the unused list

Updated bits [10:5] to match current firmware

12) TMB-to-DMB Active CFEB List backplane signals extended 5 bits to 7

Assigned 2 new bits to the next available spare DMB signals.

dmb_tx[45:44] <= active_feb_list[6:5]

TMB schematic signals:

Signal res_to_dmb1 is now active_feb_list[5]   on backplane pin E10

Signal res_to_dmb2 is now active_feb_list[6]   on backplane pin A11

13) Modified header40_[11:0] for 7-bit extensions to various 5-bit header fields

Header40_[11:0] used to contain the peak RAM fence counter for raw hits storage debugging.

The peak count has been removed, but can still be read out with VME, and its dynamic

status is indicated  by header40_[10].

The flag bit in header40_[11] indicates when bits [10:0] are valid for7-dcfeb firmware versions.

header40_[1:0] = active_feb_mux[6:5];	Extend Hdr23[4:0]    Active CFEB list sent to DMB

header40_[3:2] = r_cfebs_read[6:5];		Extend Hdr23[9:5]     CFEBs read out for this event

header40_[5:4] = perr_cfeb_ff[6:5];		Extend Hdr27[12:8]   CFEB RAM parity error, latched

header40_[7:6] = cfeb_badbits_found[6:5];	Extend Hdr30[11:7]   CFEB[n] has at least 1 bad bit

header40_[9:8] = cfeb_en[6:5];		Extend Hdr35[14:10] CFEBs enabled for triggering

header40_[10]  = buf_fence_cnt_is_peak;	Current fence is peak number of fences in RAM

header40_[11]  = (MXCFEB==7);		TMB has 7 DCFEBs so hdr40_[10:0] are active

14) Modified raw hits readout

Readout can now include raw hits from 0 to 7 CFEBs, as

indicated by cfebs_read[6:0].  The 7-bit CFFEB list cfebs_read[6:0] is constructed

from cfebs_read[4:0] in header23_[9:5] and cfebs_read[6:5] in header40_[3:2].

In 5-CFEB firmware versions, header40[11] is zero (unless the is a fault in the readout logic).

A zero in header40_[11] indicates a 5-CFEB TMB, and the bits in header40_[10:0] should

not be used to extend 5-bit fields (cfebs_read for instance).

15) Blocked bits  readout

Readout can now include blocked hits from 0 to 7 CFEBs, as

indicated by cfebs_read[6:0].

Version 09/23/2012

- Add VME register 0x15A:  ADR_VIRTEX6_SYSMON

Version 09/16/2012

Adds support for Virtex-6 SNAP12 GTX fiber optic receivers

- Add VME register 0x148:  ADR_VIRTEX6_SNAP12_QPLL

Virtex-6 mezzanine QPLL reset and status

Virtex-6 mezzanine SNAP12 receiver serial interface

- Add VME register 0x14A: ADR_VIRTEX6_GTX_RX_ALL

GTX control and status common to all 7 receivers

3) Add 7 VME registers 0x14C-0x158:  ADR_VIRTEX6_GTX_RX0 - ADR_VIRTEX6_GTX_RX6

GTX control and status for individual receivers

4) Add event counters 79-85:

GTX receiver counters for fibers 0-6

Clears on gtx_rx_reset_err_cnt

5) Add mezzanine test points for GTX receiver [0]

`ifdef VIRTEX6

assign meztp20 = gtx_rx_start[0];

assign meztp21 = gtx_rx_valid[0];

assign meztp22 = gtx_rx_match[0];

assign meztp23 = gtx_rx_fc[0];

assign meztp24 = alct_wait_cfg;

assign meztp25 = lock_tmb_clock0;

assign meztp26 = 0;

assign meztp27 = sump;

`else

assign meztp20 = alct_startup_msec;

assign meztp21 = alct_wait_dll;

assign meztp22 = alct_startup_done;

assign meztp23 = alct_wait_vme;

assign meztp24 = alct_wait_cfg;

assign meztp25 = lock_tmb_clock0;

assign meztp26 = 0;

assign meztp27 = sump;

`endif

Version 08/17/2012

- Delay JTAG PROM data stream to ALCT by 116msec to allow Spartan-6 mezzanine to finish configuration.

Spartan-6 takes 212msec to configure and TMB takes 100msec.

TMB sends JTAG data to ALCT 100+116=216msec, after a

simultaneous hard reset.

The 4msec pad allows ALCTs DLL/PLL time to lock.

2) Add VME register Adr 0x144 ALCT Spartan-6 startup delay

3) Add VME register Adr 0x146 ALCT Spartan-6 startup state machine status

4) Change Adr 0xD4[2]=jsm_sel, was Write-only, is now Read/Write

Version 07/07/2010

1) Injector RAM mods:

Move injector RAM data msbs [17:16] from 0x44 to 0x100

Revert 0x44 wen and ren to independent RAM enables

Version 07/04/2010

1) Changed default Adr 0x100[15] l1a_win_pri_en = 1

Enables window prioritizing mode to limit TMB to 1 readout per L1A

Version 07/01/2010

1) Bug fix in alct*clct matching for a rare case when 2 CLCTs are exactly clct_window bx apart in time.

Caused 1st CLCT to be replaced by 2nd CLCT, sending 2 identical events to L1A pipeline.

2) New algorithm for L1A matching prevents multiple events from reading out for 1 L1A.

Original algorithm allowed multiple events to read out if they were within the L1A window.

A system downstream of TMB apparently fails to tolerate multiple readouts per L1A.

Adr 0x100[15] l1a_win_pri_en = 0  enables original multiple event readouts per L1A mode

l1a_win_pri_en = 1  enables a prioritizing mode that limits TMB to 1 readout per L1A

Current default l1a_win_pri_en = 0  to allow checking alct*clct matching logic.

Next firmware release will set the default to 1.

3) New counter 58 is inserted after counter 57, all subsequent counters shift up 1channel number.

Counter 58 counts events lost from readout queue due to L1A window prioritizing that limits TMB to

1 event readout per L1A.

4) A buffer stalled-at-least-once bit has been added to Adr 0x9E[7] and Header38[14].

Indicates there was a least one buffer stall since the last resync.

5) Adr 0x44 rebuilt to expand CFEB pattern injectors to also assert L1A and ALCTs at an arbitrary time.

The 3 individual injector RAM select bits ren and wen have been replaced by 2-bit RAM addresses.

Injector RAM data width expanded from 16 to 18 bits to provide storage for 2 ALCTs and L1A.

6) Adr 0x32[3] alct_inj_ram_en = 1 enables ALCT pattern injector RAM

Adr 0x32[4] l1a_inj_ram_en = 1 enables L1A pattern injector RAM

7) Miniscope is now turned on by default: Adr 0x10C[0] mini_read_enable = 1.

Miniscope word count automatically inserted in readout stream by default Adr 0x10C[2]=1

New channel assignments, see Adr 0x10C section.

Version 05/14/2010

1) Add bx0 emulator enable to Adr 0x2A[15].  Power-up default is 0.

Generates bx0 that is ORed with ttc_bx0. For use in systems lacking a CCB.

2) Add clock_lock_lost_err_en to Adr 0x120[5]  (shifts other bits left by 1).

Add clock_lock_lost_err_ff  to Adr 0x120[14].

Add force_sync_err to Adr 0x120[15].

3) Redesign vme.v to use initial blocks to specify power-up state for VME registers instead of load pulse

Add explicit integer widths to VME address decoder case statement.

Replace constants passed as signals from top level module with defparam mechanism.

4) Remove clock_lock_lost from OR with bx0_sync_err in sequencer.v

Add clock_lock_lost term to sync_err_ff in sync_err_ctrl.v

Remove bx0_sync_err from sync_err_ff,  add it as an independent OR with sync_err signal.

Fixes sync_err fails-to-clear bug:

Sync_err_ff clears on ttc_resync but bx0_sync_err takes n bx to clear.

TMB was synchronizing correctly on ttc_resync, but incorrectly latched sync_err=1.

5) Modify bx0 sync error counter[61] to count only when TMB is in trigger-run state.

Was counting sync errors before ttc_resync arrived, now only counts errors that occur after a resync.

A non-zero value in this counter indicates incorrect bxns in TMB readouts and LCTs.

6) Modify FMM state machine fmm_trig_stop signal to power up as a 1.

Was powering up as 0, then set to 1 after the 1st clock cycle.

Entered trigger-run state for 1bx but was ignored since pre-trigger logic is held off 5bx after power up

7) Fix tmb.v kill_clct logic for type C|D for 1 clct + 2 alct case where clct is on ME1A

Version 03/19/2010

1) Mod cfeb.v module busy hs delimiters for me1a me1b cscs to separate cfeb4 from cfebs0-3.

Prevents pattern finder from discarding 2nd CLCT at the me1a|me1b boundary.

Version 03/07/2010

1) Added requested mod that blocks bad cfeb distrips from both trigger path and data path.

2) Add bcb_read_enable to Adr72[15] p45  to include blocked CFEB DiStrip list in the

DMB readout stream:

Blocked bits include:

CFEB DiStrip bits turned off in the Hot Channel Mask,

CFEB DiStrip bits turned off by automatic bad-bits detection

Entire CFEBs turned off via mask_all

Set Adr72[[15]=1 to enable blocked bits readout.

Power up default is 0, which is backwards compatible with older versions of the readout stream.

3) Add bcb_read_enable to Header29[13]

4) Document CFEB blocked DiStrip readout format, p102

5) Bug fix for CLCT-only trigger mode

The 2nd CLCT in previous firmware versions had bxn=0 in the LCT sent to MPC when using the clct-only mode, and there was also no ALCT coincidence.

Version 02/10/2010

1) Add event_clear_vme to AdrAC[15] to clear aff, clct, and mpc VME read-back registers

2) Add active_feb list reversal for TypeB CSCs

Version 01/14/2010 CFEB bad di-strip bit detection

1) Header30[11:7] = cfeb_badbits_found[4:0]	Bad distrip bits detected in cfebn[n]

Header30[12] = 0

2) New VME regisers Adr 0x122 to 0x142

Adr 122	ADR_CFEB_BADBITS_CTRL, CFEB Bad Bits Control/Status

[04:00]	RW  cfeb_badbits_reset[4:0]  Reset bad cfeb bits FFs for cfeb[n]

[09:05]	RW  cfeb_badbits_block[4:0]  Block bad cfeb bits in cfeb[n]

[14:10]	R   cfeb_badbits_found[4:0]  CFEB[n] has at least 1 bad bit

[15]	R   cfeb_badbits_blocked     At least one CFEB has a bad bit that was blocked

Adr 124	ADR_CFEB_BADBITS_TIMER	CFEB Bad Bits Check Interval

Sets number of bx a bit must be continuously high before being marked as bad.

Adr 126	ADR_CFEB0_BADBITS_LY01	CFEB0 Ly0,Ly1 Bad Bits List

Adr 128	ADR_CFEB0_BADBITS_LY23	CFEB0 Ly2,Ly3 Bad Bits List

Adr 12A	ADR_CFEB0_BADBITS_LY45	CFEB0 Ly4,Ly5 Bad Bits List

Adr 12C	ADR_CFEB1_BADBITS_LY01	CFEB1 Ly0,Ly1 Bad Bits List

Adr 12E	ADR_CFEB1_BADBITS_LY23	CFEB1 Ly2,Ly3 Bad Bits List

Adr 130	ADR_CFEB1_BADBITS_LY45	CFEB1 Ly4,Ly5 Bad Bits List

Adr 132	ADR_CFEB2_BADBITS_LY01	CFEB2 Ly0,Ly1 Bad Bits List

Adr 134	ADR_CFEB2_BADBITS_LY23	CFEB2 Ly2,Ly3 Bad Bits List

Adr 136	ADR_CFEB2_BADBITS_LY45	CFEB2 Ly4,Ly5 Bad Bits List

Adr 138	ADR_CFEB3_BADBITS_LY01	CFEB3 Ly0,Ly1 Bad Bits List

Adr 13A	ADR_CFEB3_BADBITS_LY23	CFEB3 Ly2,Ly3 Bad Bits List

Adr 13C	ADR_CFEB3_BADBITS_LY45	CFEB3 Ly4,Ly5 Bad Bits List

Adr 13E	ADR_CFEB4_BADBITS_LY01	CFEB4 Ly0,Ly1 Bad Bits List

Adr 140	ADR_CFEB4_BADBITS_LY23	CFEB4 Ly2,Ly3 Bad Bits List

Adr 142	ADR_CFEB4_BADBITS_LY45	CFEB4 Ly4,Ly5 Bad Bits List

Usage Notes:

1) Dead channel detection:

Detects CFEB channels that never fire

Set Adr122[09:5]=0x00     to turn off badbit blocking

Set Adr124[15:0]=0x0001 to set high-time threshold to 1 bx

Read dead channel list from Adrs126-142

2) Noisy channel detection:

Detects CFEB channels that have after-pulsing or frequent firing,

for instance, 3 consecutive triad starts)

Set Adr122[09:5]=0x00     to turn off badbit blocking

Set Adr124[15:0]=0x0007 to set high-time threshold to 7 bx

Read noisy channel list from Adrs126-142

3) Bad bit detection or blocking

Detects CFEB channels that are always high or high for an unreasonable length of time

Set Adr122[09:5]=0x1F       to turn on badbit blocking (or set 0x00 for just monitoring)

Set Adr124[15:0]=0x0DEC to set high-time threshold to 3564 bx or something similar

Read bad channel list from Adrs126-142

Version 10/15/09 ALCT duplicate alct detection + Header r-type

1) Header05[10:9] r_type always == 1 on previous versions.

It should equal fifo_mode unless the event buffer is full.

2) Added ALCT structure error counter cnt[75] to count events where alct0==alct1.

3) AdrD0[5] Changed default to cnt_alct_debug=1 to enable ALCT data structure error counters.

N.B. The ALCT structure error counters are only 8 bits, and could reach full scale quickly.

So,  AdrD0[02] cnt_stop_on_ovf  should be 0, when setting cnt_alct_debug=1, otherwise

all event counters will stop counting if there are excessive ALCT errors.

Version 09/21/09 Synchronization Error Control Register + bxn offset limit

Add limits for bxn_offset_pretrig and bxn_offset_l1a

bxn_offset > lhc_cycle is converted to lhc_cycle-1

Add sync_err_ctrl register Adr 0x120

Version 09/08/09 Digital Phase Shifter Autostart

Add vsm_phaser_auto to AdrDA[11].

Default = 1, starts digital phase shifters after VME user PROM is read

Version 08/25/09 PosNeg sync FFs for ALCT and CFEBs

Same as 8/14/09 version but has ALCT and CFEB posneg sync stages enabled.

Switched to ISE 8.2sp3 because ISE 10.1sp3 could not complete PAR.

Documentation updates to conform firmware to c++ demo code:

Phaser register signal names now absorb hcycle and qcycle bits into 1 8-bit phase delay:

Adr10E: 	alct_rxd_delay[7:0]		Delays latching data received from ALCT in 0.1ns steps

Adr110:	alct_txd_delay[7:0]		Delays data transmitted to ALCT in 0.1ns steps

Adr112-11A:	cfeb[n]_rxd_delay[7:0]	Delays latching data received from CFEB[n] in 0.1ns steps

Modify interstage delay signals to make it clear they are integer bx delays:

Adr38:		alct_txd_int_delay[3:0]	Delay data transmitted to ALCT by integer bx

Adr11C-11E:	cfeb[n]_rxd_int_delay[3:0]	Delay data received from CFEB[n] by integer bx

Version 08/14/09 Digital Phase shifters for CFEBs

Has both ALCT and CFEB muonic timing.

Disabled cfeb posnegs and alct_rxd_posneg else compile fails. Alct_txd_posneg is OK.

Notes on ALCT and CFEB timing adjustments:

ALCT:

1) Select a Time of Flight delay:

Using DDD 2ns steps, ranging from 0 to 12, spanning 0 to 24ns

Based on distance from IP to “some point” on the CSC

Also compensate for tmb-to-alct cable propagation delay differences between CSCs

Write alct_tof_delay to DDD chip in Adr16[3:0]

2) Tune alct_rxd_delay to the good-data window center

Using Digital Phase Shifter 0.1ns steps, ranging from 0 to 255, spanning 0 to 25ns

Put ALCT into loopback mode to send a test pattern to TMB

Scan alct_rxd_delay 0-255   using Phaser0 Adr10E[15:8]

Scan alct_rxd_posneg 0-1    using Phaser0 Adr10E[15:8] (disabled in 8/14/09 firmware)

3) Tune alct_txd_delay to the good-data window center

Using Digital Phase Shifter 0.1ns steps, ranging from 0 to 255, spanning 0 to 25ns

Put ALCT into loopback mode to send a test pattern to TMB

Scan alct_txd_delay 0-255   using Phaser1 Adr110[15:8]

Scan alct_rxd_posneg 0-1    using Phaser1 Adr110[15:8] (not disabled in 8/14/09 firmware)

CFEBs:

1) Select a Time of Flight delay:

Using DDD 2ns steps, ranging from 0 to 12, spanning 0 to 24ns

Based on distance from IP to “some point” on the CSC

Also compensate for tmb-to-cfeb cable propagation delay differences between CSCs

Write cfeb_tof_delay to DDD chip in Adr18[11:8]

2) Tune cfeb[n] clock delays for simultaneous arrival at all 5 cfebs

Using DDD 2ns steps, ranging from 0 to 12, spanning 0 to 24ns

Delays might be set according to known cable propagation delays

Delays might be determined empirically by setting high comparator thresholds to make

the analog signal time-over-threshold less than 25ns, then scanning DDD delay vs

6-hit efficiency.

Write cfeb[n] clock delays to DDD channels in Adr18[15:12] and Adr1A[15:0]

3) Tune cfeb_rxd_delay for cfeb[n] to the good-data window center

Using Digital Phase Shifter 0.1ns steps, ranging from 0 to 255, spanning 0 to 25ns

Generate CFEB test pulses or use muon tracks

Scan cfeb_rxd_delay 0-255  using Phaser2-6  Adr112-Adr11A bits[15:8]

Scan cfeb_rxd_posneg 0-1   using Phaser2-6  Adr112-Adr11A

4) Tune cfeb inter-stage integer delay for cfeb[n]

Set a delay 0-15bx so that triad bits from all 5 CFEBs arrive at TMB on the same bxn

Might be done by pulsing all 5 CFEBs simultaneously, then checking the CFEB

raw hits readout to see that triad start bits all appear in the same bxn.

Set cfeb[n] inter-stage delays in Adr11C-Adr11E

Version 07/13/09 Digital Phase shifters for CFEBs

Added 5 digital phase shifters for cfeb rxd delays: Adr112-Adr11A

Add 2 VME registers: Adr11C-Adr11E for CFEB interstage delays

Version 06/29/09 Digital Phase shifters for ALCT

Two digital phase shifters replace DDD delays for alct_txd_delay and alct_rxd_delay

Add VME registers Adr10E and Adr110 for digital phase shifters

Adr14[13,11,10] reverted to old format, removed phase shifter DCM locks

Adr30[15:13] removed posnegs, they now reside in Adr10E and Adr110, cfeb posneg is gone for now

Version 06/22/09 Muonic Timing for ALCT

Added muonic timing to float ALCT board in clock-space independently

of good-data rxd|txd windows.

Changes to DLL lock register:

Adr14[10]   lock_alct_rxd	[these get undone in 6/29/09 version]

Adr14[11]   lock alct_txd

Adr 14[13]  lock_cfeb_rxd

Changes to DDD delay and posneg registers:

Adr16[3:0]   alct_tof_delay        Shift entire ALCT in clock-space to compensate for muon time of flight

Adr16[7:4]   alct_txd_delay       Latches TMB-to-ALCT data in middle of transmit data window

Adr16[11:8]  dmb_tx_delay       Change default to 6

Adr18[3:0]   alct_rxd_delay        Latches ALCT-to-TMB data in middle of receive  data window

Adr18[7:4]   cfeb_rxd_delay       Latches CFEB-to-TMB data in middle of receive  data window

Adr18[11:8] cfeb_tof_delay        Shift all CFEBs in clock-space to compensate for muon time of flight.

Adr30[8]   alct_clock_en_use_ccb     moved from [11]

Adr30[9]   alct_clock_en_use_vme    moved from [12]

Adr30[10]  alct_muonic             1=ALCT muonic version instantiated, readonly

Adr30[11]  cfeb_muonic            1=CFEB muonic version instantiated, readonly

Adr30[12]  unassigned

[These changes get undone in 6/29/09 version:]

Adr30[13]  cfeb_rxd_posneg     Sets receive  data posneg clock polarity   (new signal)

Adr30[14]  alct_txd_posneg      Sets transmit data posneg clock polarity, (was alct_posneg)

Adr30[15]  alct_rxd_posneg      Sets receive  data posneg clock polarity   (new signal)

Changed 8bx constant delay in alct random number pipeline to be VME programmable

Added 2bx to compensate for muonic sync stages.

Default delay is now 8+2-1=9bx

Good spots for reference TMB+ALCT384 occur at pipedepth 4 when alct_sync_rxdata_pre=9

Adr104[15:12] = alct_sync_rxdata_pre[3:0], default=9

Changed Adr 0E[15:11] to connect dmb_tx_reserved[4:0]  to dmb_tx[48:44]

Adr 0E[15:11] =dmb_tx_reserved[4:0], just set to 0 for now

Version 06/05/09

Re-structure dmb_tx[48:0] flip-flops to force IOB instantiation

No other changes

Version 05/15/09

Added miniscope to monitor clct pretrigger processing and alct*clct matching.

Added miniscope VME register Adr 0x10C.

Rebuilt parity register Adr 0xFA to accept miniscope RAM parity.

Restructured DMB image RAM from 5 BRAMs down to 4 BRAMs to free up 1 RAM

Reduced ALCT raw hits storage RAM from 2048bx down to 1024bx to free up 1 RAM

Replaced Virtex-E era RPC de-mux and pipeline stages to minimize latency.

Added clct pre-trigger signal to RPC readout to DMB in a former always-zero bit.

Added data=address test mode to RPC storage RAM

Added 8bx constant delay in alct random number pipeline. Good spots at depth 12 before are now at 4.

Removed legacy alct signals from Adr 0x38,0x30 and Hdr 30[12:7] that are now used for ECC parity.

Added alct_ecc_err_blank to Adr 038[2] to blank alct data that has uncorrected ecc errors.

Added counter[6] to count alct data blanked due to uncorrected ecc errors

Adr 0x30: 	Removed Adr30[11:8] alct_reserved_out[3:0], as these bits now carry ecc parity.

Adr 0x38: 	Adr38 has been reorganized to make room for the new alct_ecc_err_blank signal

Moved alct_ecc_en to Adr38[1]		n.b. this affects loop-back test software

Added alct_ecc_err_blank to Adr38[2] 	blanks alcts with uncorrected ecc errors.

Moved alct_sync_ecc_err to Adr38[4:3]	n.b. this affects loop-back test software

Removed Adr38[2:1] seq_status[1:0]

Removed Adr38[4:3] seu_status[1:0]

Removed Adr38[8:5] reserved_out[3:0]

alct_stat_rd[0]     = alct_cfg_done;		// R	ALCT FPGA loaded

alct_stat_rd[1]     = alct_ecc_en;		// RW	Enable ALCT ECC decoder, else do no ECC correction

alct_stat_rd[2]     = alct_ecc_err_blank;	// RW	Blank alcts with uncorrected ecc errors

alct_stat_rd[4:3]   = alct_sync_ecc_err[1:0];// R	ALCT sync mode ecc error syndrome

alct_stat_rd[11:5]  = alct_stat_wr[11:5];	// RW	Free

alct_stat_rd[15:12] = alct_txd_delay[3:0];	// RW	ALCT data transmit delay, integer bx

Adr 0xFA: 	AdrFA has been reorganized to make room for the new miniscope RAM parity

perr_adr[] expanded from 3 to 4 bits.

Adr 0x10C:	New Miniscope control register.

Adr 0xBC[14] Added rpc_tbins_test for RPC RAM data=address test mode

Adr 0xCA[9]	Moved bx0_match to [10]

Added bx0_vpf_test to [9]

Hdr27[13] now ORs miniscope RAM parity errors with RPC RAM parity errors

Hdr19[14] vme_exists replaced by mini_read_enable

RPC readout format: unused tbin bit[11] was always 0, now has clct-pretrigger flag

Counter[06]:	Inserted scnt[06]="ALCT: trigger path ECC>=2-bit error, ALCT discarded"

Shifts subsequent counters up by 1 address.

TMB readout format changed to include miniscope data and markers when mini_read_enable=1.

By default, inserts B07 marker, 22 scope words, then E07 marker after RPC data.

ALCT legacy cable signals that are now ecc parity updated in

Table 25: J5 ALCT Cable1 Connector [J10  on ALCT board]

Table 17: J6 ALCT Cable2 Connector [J11 on ALCT board]

Versions 04/07/2009 - 04/14/09

Added TMB-to-ALCT sync-stage and inter-stage in alct.v module.

Improves alct_rx_clock windows, and allows a ½-cycle shift

in alct_rx_clock at inter-stage, but adds 2bx to output signals.

Add Adr 30[14] alct_posneg

Modified UCF to constrain ALCT inter-stage flip-flops to FPGA slice locations near ALCT IOBs

Compiled 1 version with ALCT 80 MHz IOBs set to Slew=Fast | Drive=12

Another version has         ALCT 80 MHz IOBs set to LVDCI_33 (Digitally Controlled Impedance, 50)

Added TMB-to-ALCT 80MHz diagram to this doc

Version 03/16/2009

New Features:

Error Correcting Code to ALCT-to-TMB trigger data path, using reserved rx signals

Error Correcting Code to TMB-to-ALCT TTC command path, using reserved tx signals

Separate bxn counter and offset for L1A

New event counters + re-numbered ALCT-counter-group 0-11 [see counter register adr 0x000]

(see AdrD0 p62 for details)

3 for ECC rx data

3 for ECC tx data

5 for individual CFEB pre-triggers

1 for alct_bx0

Register Changes:

Adr 016  change delay_ch0[3:0] alct_tx_clock default to 11 for use with reference ALCT

Adr 076  rename from adr_seq_offset to adr_seq_offset0

Adr 076  rename bxn_offset[11:0] to bxn_offset_pretrig[11:0]

Adr 0D0  increase cnt_select[6:0] by 1 bit to address more event counters

Adr 0D0  move perr_reset from AdrD0[15] to AdrFA[6]

Adr 10A  new register adr_seq_offset1

Adr 10A  add bxn_offset_pretrig[11:0], which is a separate bxn offset for the new L1A bxn counter

Adr038[10:9] new signal:  alct_sync_ecc_err[1:0]  is ALCT sync-mode ECC error code, readonly

Adr038[11]    new signal:  alct_ecc_en is ALCT ECC trigger data correction enable with default=1

N.B. setting alct_ecc_en =0 stops ALCT trigger data correction, but does not affect ECC counters

Header Changes:

Header30[6:5]  now contains alct_ecc_err[1:0]

ALCT Cable Signal Changes for ECC

| Old | New | Comment |
| --- | --- | --- |
| reserved_in[0] | parity_in[0] | ECC parity [5:0] for TMB-to-ALCT |
| reserved_in[1] | parity_in[1] | “” |
| reserved_in[2] | parity_in[2] | “” |
| reserved_in[3] | parity_in[3] | “” |
| seq_cmd[0] | seq_cmd[0] | Activates ALCT sync mode |
| seq_cmd[1] | parity_in[4] seq_cmd[1] | Parity sent unless in sync mode |
| seq_cmd[2] | seq_cmd[2] | Activates ALCT sync mode |
| seq_cmd[3] | seq_cmd[3] ¦ parity_in[5] | Parity sent unless in sync mode |
| seq_status[0] | parity_out[0] | ECC parity [6:0] for ALCT-to-TMB |
| seq_status[1] | parity_out[1] | “” |
| seu_status[0] | parity_out[2] | “” |
| seu_status[1] | parity_out[3] | “” |
| reserved_out[0] | parity_out[4] | “” |
| reserved_out[1] | parity_out[5] | “” |
| reserved_out[02] | parity_out[6] | “” |

Version 02/05/2009

Added ALCT-TMB sync mode loop-back test logic to alct.v module.

Mod Adr F2: Change compiler ID field to accommodate extra digit for ISE 10.1I

Add Adr104, Adr106, Adr108 for ALCT sync-mode data

Version 01/13/2009

Rename ALCT cable 2 pair 15 from reserved_in4 to seq_cmd3

Adr 30: alct_cfg:  rename Adr30[7] to seq_cmd3, replaces reserved_in4

Version 12/10/2008

Add L1A-only readout mode with full header and raw hits

Add L1A lookback offset in new VME register	Adr 0x100

Add sequencer debug signals to new VME register	Adr 0x102

Replaced parity errors Adr 0x0FA with sub-adr multiplexing

Add 35-bit RAM parity error array to Adr 0x0FA

Version 11/18/2008

Replaced L1A data storage logic:

Moved L1A data from header RAM to fence queue RAM

Allows L1A-only TMB readout mode to have valid data in short-header (i.e. bxn at L1A arrival)

To enable L1A-only TMB readout

[1] fifo_mode=3		sets short header

[2] l1a_allow_notmb=1	allows readout when tmb didn’t trigger for that L1A

[3] turn off TMB pre-triggers (set mask_all=0 or halt pre-trigger machine)

[4] send L1A via TTC....all ~500 TMBs should send short header to DDU

Inverted raw hits RAM parity.

Now parity bit=1 if RAM data[7:0]=8’b00000000

Modified CFEB and RPC raw hits RAMB16s to be read-first instead of write-first.

Guarantees parity data on port B is valid before writing new data to port A.

AdrFE[2] BXN latched at last L1A

New Event Counters

Inserted 2 new counters after counter[47]

counter[48]= Sync error, bxn!=offset at bx0 arrival or bx0 did not arrive at bxn==offset

counter[49]= Raw hits RAM parity error, possible radiation SEU

Note on enabling internal scope readout to DMB/DDU

Adr98   =  0x108B

Adr9A   = 0x0000

AdrCE = 0x0000

Version 09/30/2008

AdrD4[2] add jsm_sel to select old/new alct user prom format.

This is a write-only bit, it reads back the value of vsm_jtag_auto

New Internal Scope Logic:

Allows scope channel data to be inserted in DMB readout stream

All scope channel signal assignments have bee replaced

Adr98 Replaced with new internal scope signals

Adr9A Replaced with new internal scope signals

To enable scope data in DMB readout, set

scp_ch_trig_en =1

scp_runstop=1

scp_force_trig=0

scp_auto=1   (also appears in Header19[13]

scp_tbins=4  (may be 0 to 7, number of scope tbins = 64*(scp_tbins+1), thus spanning 64 to 512)

scp_nowrite=0

Rename Event Counters to better describe their functions

counter[22]= TMB matching discarded an ALCT pair (all alcts in the pair were discarded)

counter[23]= TMB matching discarded a CLCT pair (all clcts in the pair were discarded)

New Event Counters

Inserted 2 new counters after counter[23]

counter[24]= TMB matching discarded CLCT0 from ME1A

counter[25]= TMB matching discarded CLCT1 from ME1A

Shifts all subsequent counter addresses up by 2

Header Updates:

header08_[12]		r_tmb_clct0_discard;		TMB discarded clct0 from ME1A

header08_[13]		r_tmb_clct1_discard;		TMB discarded clct1 from ME1A

Version 09/12/2008

Added tmb_trig_pulse to header40[14]

Added tmb_trig_keep to header41[9]

Added tmb_non_trig_keep to header41[10]

Version 09/05/2008

Add blocking of LCTs to MPC for ME1A

Version 08/28/2008

Logic modifications:

- Firmware compile type codes introduced, replaces programmable stagger and reversal

- Added hs reversal for ME1A and ME1B and full hs reversal for non-ME1A/B CSCs

- Added blocking for ME1A to MPC, but is not yet functional in this release

- Added 2 state machines to detect and count TTC lock loss signals from CCB

Adr 2E[9]: Add ccb_ttcrx_ready	TTC ready signal from CCB

Adr 2E[10]: Add ccb_qpll_locked	Lock signal from CCB

Adr 2E changed register symbolic name from ADR_CCB_STAT to ADR_CCB_STAT0

Adr FA[15:0] Add new register ADR_PARITY contains parity SEU error status

Adr FC[15:0] Add new register ADR_CCB_STAT1 contains TTC lock status from lock state machines

Rename event counter [5] “Pre-trigger was on any cfeb”

Inserted 2 new event counters after counter[5], shifts all other counter addresses up 2

[6] Pre-trigger was on ME1A cfeb4 only

[7] Pre-trigger was on ME1B cfebs0-3 only

Add 2 new event counters after counter[57],

[58] CCB:   TTCrx lock lost

[59] CCB:  qPLL lock lost

Add new register AdrFC for CCB lock detection

AdrFC[00]	ccb_ttcrx_lock_never	TTCrx lock never achieved

AdrFC [01]	ccb_ttcrx_lost_ever	TTCrx lock was lost at least once

AdrFC [02]	ccb_qpll_lock_never	QPLL lock never achieved

AdrFC [03]	ccb_qpll_lost_ever	QPLL lock was lost at least once

Extended trigger source vector in VME and Header:

Adr7C[9] me1a_only_pretrig

Adr7C[10] me1b_only_pretrig

Hdr40[12]=r_trig_source_vec[9]

Hdr40[13]=r_trig_source_vec[10]

Introduced Firmware Compile Type Codes

A=Normal      CSC

B=Reversed CSC

C=Normal  ME1B,  Reversed ME1A

D=Reversed ME1B, Normal   ME1A

Extended ½-strip reversal and ME1A/B signals to Adr CC

Adr CC[05] csc_me1ab	1= CSC is ME1A or ME1B. 0=normal CSC

Adr CC[06] stagger_hs_csc	1=Staggered Adr CSC, 0=non-staggered

Adr CC[07] reverse_hs_csc	1=Reversed staggered CSC, non-me1

Adr CC[08] reverse_hs_me1a	1=reversed me1a hstrips

Adr CC[09] reverse_hs_me1b	1=reversed me1b hstrips

Adr CC[15:12] csc_type[3:0]	Firmware compile type A, B,C or D

Added ½-strip reversal signals to Header

Header does not contain csc_type explicitly, but csc_type can be inferred from reversal signals

in Hdr39[14:12] and Hdr20[14]

Hdr20[14]	stagger_hs_csc	CSC Staggering ON

Hdr39[12]	reverse_hs_csc	1=Reverse staggered CSC, non-me1

Hdr39[13]	reverse_hs_me1a	1=ME1A hstrip order reversed

Hdr39[14]	reverse_hs_me1b	1=ME1B hstrip order reversed

Added ME1A LCT blocking to MPC [not yet functional in this release]

Adr CC[03] mpc_me1a_block	Block ME1A LCTs from MPC, still queue for readout

Adr CC[04] cnt_non_me1ab_en	Allow clct pretrig counters count non me1ab events

End of 8/28/2008 mods

Version 08/12/2008

Adr38[15:12] add alct_txd_delay[3:0] to delay alct tx data, delay=0 by default has same timing as previous firmware versions.

Adr CA[9]: add bx0_match

Hdr30[14]: add bx0_match

Version 08/04/2008

Adr F6[6]: add clct_sep_ram_sel_ab to select A or B separation RAM data readback

Version 08/01/2008

Adr F4[1]: stagger_csc is now read-only, set at firmware compile time

Header36: r_nrpcs_read in header now gated with rpc_read_enable.

now indicates 0 rpcs when rpc readout is disabled

Version 07/15/2008

Added ability to readout non-triggering events

Header41[0] = VME settings for tmb_allow_alct, for trigger and readout

Header41[1] = VME settings for tmb_allow_clct, for trigger and readout

Header41[2] = VME settings for tmb_allow_match, for trigger and readout

Header41[3] = VME settings for tmb_allow_alct_ro, for non-triggering readout

Header41[4] = VME settings for tmb_allow_clct_ro, for non-triggering readout

Header41[5] = VME settings for tmb_allow_match_ro, for non-triggering readout

Header41[6] = alct-only non-triggering event

Header41[7] = clct-only non-triggering event

Header41[8] = alct*clct match non-triggering event

Header41[9] = This event is a non-triggering readout

New Event counter at subadr[19] counts non-triggering events queued for readout

Shifts all subsequent counter addresses up 1

New VME register adrCC:

AdrCC[0] = tmb_allow_alct_ro	allow alct-only non-triggering event readout

AdrCC[1] = tmb_allow_clct_ro	allow clct-only non-triggering event readout

AdrCC[2] = tmb_allow_match_ro	allow alct*clct-match non-triggering event readout

Version 07/09/2008

Replaced entire ALCT UserPROM JTAG State Machine

New compressed data format: see JTAG PROM-1 section p97

Adr 70: Move dmb_thresh[2:0] from adr F4[8:6] to adr70[9:7]

Adr 70: Rename dmb_thresh to dmb_thresh_pretrig

Adr 70: Rename hit_thresh to hit_thresh_pretrig

Adr 70: Rename nph_thresh to hit_thresh_postdrift

Adr F0: Rename lyr_thresh[2:0] to lyr_thresh_pretrig[2:0]

Adr F4: Remove dmb_thresh from F4[8:6]

Adr F4: Add new signal pid_thresh_postdrift[3:0] to F4[9:6]

Adr F4: Move adjcfeb_dist from F4[14:9] to F4[15:10]

Adr F4: Rename pid_thresh to pid_thresh_pretrig

Add adr D8[12] jsm_tckcnt_ok		JTAG PROM TCKs sent matches TCKs in trailer frame

Add adr D8[13] jsm_end_ok			JTAG PROM FF end marker found where expected

Add adr D8[14] jsm_header_ok 		JTAG PROM BA begin marker found where expected

Add adr D8[15] jsm_chain_ok 		JTAG PROM Chain Block marker found where expected

Add adr DE[14:13] jtag_sm_vec[1:0]	JSM JTAG signal state machine vector

Add adr E0[11:8] jsm_prom_sm_vec[3:0]	JSM PROM state machine vector

Add adr E0[14:12] jsm_format_sm_vec[3:0]	JSM Data format state machine vector

Add adr EA[15] jsm_tckcnt_ok		JSM tckcnt added to board status

Header20: 	remove header20_[12:10]	lyr_thresh[2:0]

Header20	remove header20_[13]	layer_trig_en

Header20	add       header20_[13:10]	pid_thresh_postdrift[3:0]

Header41: 	add header41_[13:11]		lyr_thresh_pretrig[2:0]

Header41	add header41_[14]		layer_trig_en

New Counters:

Inserted 2 counters after counter at SubAdr[8]

SubAdr[9]CLCT:  CLCT0 passed hit thresh but failed pid thresh after drift

SubAdr[10]CLCT:  CLCT0 passed hit thresh but failed pid thresh after drift

Shifts all other counter addresses up by 2,  i.e. old counter at SubAdr[9] moved to [11]

Change CLCT Processing Algorithm at bx11 to also require pid  pid_thresh_postdrift

Version 06/03/2008

Modifies global_reset and ttc_resync behavior

Adds ability to send active feb flag to DMB at  tmb alct*clct matching, retains ability to send at pre-trig

Overlays ALCT rx data with normal scope channels to aid alct debugging

(1) Remove Adr A8[12] alct_raw_sync, wasn’t being used

(2) Add temporary alct structure error counters [48]-[52]

(3) Add Adr D0[6] cnt_clear_on_resync clears VME counters [0]-[40] on ttc_resync, default=0

(4) Add Adr D0[7] hdr_clear_on_resync clears header counters [41]-[47] on ttc_resync, default=1

(5) Update MPC frame format doc (reflects changes to 5/12/08 firmware)

(6) Added a startup state to the readout state machine to wait 1bx for buf_q_empty to update after a reset

(prevents machine from resuming a readout that was in progress at the time of a ttc_resync)

(7) Add perr_reset (one-shot) to Adr D0[15], removed ttc_resync perr reset logic

(8) Block ttc_resync from clearing resync event counter, requires vme-clear

(9) Adr2A[3] change ccb_status_oe default from 1 to 0, turns off backplane drivers to ccb

Added write-only bits to parallel non-decoded ccb commands:

Adr2A [12]	vme_evcntres	Event counter   reset  || ccb_evcntres

Adr2A[13]	vme_bcntres	Bunch crossing reset || ccb_bcntres

Adr2A14]	vme_bx0	Bx0 signal                 || ccb_bx0

(10) Adr28[12] now contains global_reset_en=1 to enable resets on DLL lock-lost

(11) Header08[14] now contains clock_lock_lost

(12) AdrAC[14]=active_feb_src,  0=pretrig, 1=at tmb matching

(13) AdrB0[7:3]=clctf[4:0] active cfeb list at tmb matching

(14) Header23[4:0] active feb list is stored either at pretrig time or tmb match, depending active_feb_src

(15) Header23[14] now contains active_feb_src bit

(16) Header29[14] now contains hs_layer_trig (moved from header23[14])

(17) AdrCE[15]=scp_ch_overlay, 0=normal scope channels, 1=use debugging channel overlay

Current overlay assignments:

scp_ch[71:0] = normal

scp_ch[128:72] = scp_alct_rx[55:0]

assign	scp_alct_rx[0]		= alct_active_feb_flag;

assign	scp_alct_rx[1]		= alct_first_valid;

assign	scp_alct_rx[2]		= alct_first_amu;

assign	scp_alct_rx[4:3]		= alct_first_quality[1:0];

assign	scp_alct_rx[11:5]		= alct_first_key[6:0];

assign	scp_alct_rx[12]		= alct_second_valid;

assign	scp_alct_rx[13]		= alct_second_amu;

assign	scp_alct_rx[15:14]	= alct_second_quality[1:0];

assign	scp_alct_rx[22:16]	= alct_second_key[6:0];

assign	scp_alct_rx[27:23]	= alct_bxn[4:0];

assign	scp_alct_rx[28]		= #alct_wr_fifo;

assign	scp_alct_rx[29]		= alct_first_frame;

assign	scp_alct_rx[43:30]	= alct_daq_data[13:0];

assign	scp_alct_rx[44]		= alct_lct_special;

assign	scp_alct_rx[45]		= alct_ddu_special;

assign	scp_alct_rx[46]		= alct_last_frame;

assign	scp_alct_rx[48:47]	= alct_seq_status[1:0];

assign	scp_alct_rx[50:49]	= alct_seu_status[1:0];

assign	scp_alct_rx[54:51]	= alct_reserved_out[3:0];

assign	scp_alct_rx[55]		= alct_cfg_done;

Version 05/23/2008

(1) Adr B0[15:14] Add clock_lock_lost and sync_er

Version 05/12/2008

(1) mpc alct_bx0 and clct_bx0 signals now bypass mpc_tx_delay

(2) Note: Adr86[13] default=0 uses ttc_bx0, set it to 1 to use local bxn counter instead

Adr86[14] default=0 enables bx0 to mpc continuously, 1 blanks mpc frames unless triggering

(3) Adr CA[8] default changed to 1 to enable using reserved[3] signal from alct as alct_bx0

(4) Adr90[15:14] now contains bx0 injector one-shots

Version 05/01/2008

(1) All scope channels replaced, see Scope Channel Assignments p52

(2) Header11 CLCT counter was behind by 1 event, fixed.

(3) Header22 Trigger source for alct*clct matching is set even if match mode is off, fixed.

(4) Header28/29/30 ALCT data was latched n-bx early, leaving empty frames, fixed.

Version 04/29/2008

(1) ALCT signal reserved_out[3] (alct-to-tmb) is now alct_bx0

(2) Adr 68[02] renamed match_pat_trig_en to alct_match_trig_en

(3) Adr AC[5] hdr_wr_continuous should be set to 0 unless using l1a_allow_notmb=1 mode

(4) Adr AC[14] removed allow_pretrig_noflush bit

(5) Adr AE[11:0] sequencer state shortened from [14:0]

(6) Adr B2[7:4] Renamed clct_width to clct_window

(7) Adr BA[15:8] Now contains rpc bxn differences moved from Adr C4

(8) Adr C4 renamed to ADR_RPC_TBINS

Adr C4 added rpc tbins, tbins before pre-trigger, and rpc_decouple [=0 to copy cfeb tbins]

(9) Adr CA[] new name ADR_BX0_DELAY, all new signals for bx0_delay and bx0 source

(10) Adr D0[8] now contains counter lower-half / upper-half mux bit cnt_adr_lsb

Adr D0[14:9] now contains counter sub-address cnt_adr[5:0]

Adr D0  Event counters replaced with new names and new sub-address channel numbers

(11) Adr F0[15:8] = clct_throttle[7:0], default=0

(12) Adr F4[0]=clct_blanking=1 (new default), prevented from setting to 0unless l1a_allow_notmb=1 or

tmb_alct_only=1

(13) Header21[14:11] Renamed clct_width to clct_window

(14) Header36 replaced to display rpc_tbins and rpc_pretrig, affects event-size calculation software,

rpc_exists[1:0] deleted.

(15) Header41 now contains the enabled TMB matching modes

Version 02/5/2008

Added parity checking to cfeb and rpc raw hits RAMs for SEU detection

Added parity error bits to header27

Add drift_delay to header29

Add alct pretrig window position to header28

Version 01/24/2008

Replaced entire L1A logic.

Replaced 8-buffer system with 2048 buffers.

Replaced  readout stack with event queue.

Replaced lct_quality.

Several VME addresses have changed:

(1) Adr AC[4] is now wr_buf_autoclr (formerly clct_turbo).

Adr AC[5]   1=allow continuous header buffer writing for invalid triggers

Previously an unused bit. Default remains 0 until new trigger logic is ready.

(2) New buffer status signals now occupy Adrs 9E,A0,A2,A4,A6.

ADR_BUF_STAT0=9E

ADR_BUF_STAT1=A0

ADR_BUF_STAT2=A2

ADR_BUF_STAT3=A4

ADR_BUF_STAT4=A6

Old Adr A2 (alctfifo1) moved to A8  (old A8 was empty)

Old Adr A4 (alctfifo2) moved to AA (old AA was empty)

Old Adr A6[5:0] adjcfeb_dist[5:0] moved to F4[14:9]

(3) Adr 72[13] is now fifo_no_raw_hits   [1=do not wait to store raw hits.  A no_daq mode.]

(4) Adr 74[15:13] is now l1a_internal_dly[2:0]  (mostly for use by the simulator)

(5) Adr AE[14:11] signal names changed for buffer status

(6) Adrs B0, 78, 7A contain new CLCT internal format

(7) Header04[13] is now buf_q_ovf_err, formerly stack_ovf_latch

Header05[14] is now buf_stalled, formerly buf_full

Header37,38,39, 40 changed for buffer status [some assignments are probably temporary]

Header25,26,27 contain new CLCT internal format

(8) Scope channels replaced

(9) Counters 1A, 1C, 1E changed, now count debug signal presence instead of time-outs

Version 10/11/2007

(1) CLCT raw hits CRC now stops at the frame before the DE0F marker due to DDUs failure to include the marker.

(2) ALCT raw hits CRC check stops at the frame before the DE0D marker, same reason.

Version 9/14/2007

(1) Header bug fixes in readout counter and lct-duplication flags

(2) New no-alct VME counter, channel 32/33, shifts subsequent channel numbers up 2

(3) ALCT DDR transmitter constraints minimize routing delays between alct_rx_clock and main clock.

Version 9/10/2007

(1) New raw hits readout header+trailer format replaces all previous header field assignments.

(2) New VME counters, channels 32 to 4C.

Version 7/10/2007

(1) Increase tmb.v-sequencer.v handshake time-out from 8bx to 15bx to prevent late alct*clct matches

from being counted as discarded events when using higher than normal clct_width values.

(2) Modify pre-trigger state machine to wait for alct*clct matching (or clct-only) before re-arming for the next pre-trigger event to prevent writing current event to wrong header buffer when using higher than normal clct_width values.

(3) Modify pre-trigger state machine to wait for active_feb signal to return to 0 before the next pre-trigger event to avoid re-triggering on same event when using longer than normal triad_persistence.

Version 7/05/2007

(1) Pattern-finder key layer shifted from ly3 back to ly2.

Ly3 pattern templates  were flipped top-to-bottom to shift key layer to ly2, and flipped left-to-right to preserve bend direction.

(2) Adr A6 adjcfeb_dist[5:0] is now 6-bits instead of 5 to allow dist=32 to span a full cfeb, default value remains 5hs.

(3) Adr F4 dmb_thresh[3:0] default is now 4 to reduce spurious active feb signals to DMB

(4) CLCT Processing Algorithm description updated for current patterns

Version 6/21/2007

(1) Adr D4 bit 11 now contains wr_usr_jtag_dis.

When wr_usr_jtag_dis=1, write access to register adr_usr_jtag is blocked.

This allows parallel writes to jtag chains for selected alcts.

Adr 10 bit 14 indicates the state of wr_usr_jtag_dis.

(2) Adrs A0, A8, AA, CA, CC are now obsolete.

Their FFs have been removed, and they read back 0000h.

(3) Adr A6 now contains adjcfeb[4:0] with a default value of 5 hstrips.

This replaces the function of mask registers A6, A8, and AA.

If  there is n hstrip key on hs 0,1,2,3,4  on CFEBn, with hits  hit_thresh_pretrig, then CFEBn-1 will be marked in the active_feb list for DMB readout.

If  there is a hstrip key on hs 31,30,29,28,27  on CFEBn, with hits  hit_thresh_pretrig, then CFEBn+1 will be marked in the active_feb list for DMB readout.

(4) Adr 6C, layer_trig_dly has been removed.

(5) Header05[12] now contains trigger source vector bit [8] (layer trigger)

Header05[13] hsds bit removed.

(6) Header16[14:11] now contains pid_thresh[3:0] instead of ds_thresh[2:0].

(7) References to hs_thresh[2:0] have been changed to hit_thresh_pretrig[2:0].

References to ds_thresh[2:0] have been removed.

(8) Adr 70, ds_thresh[2:0] has been removed.

(9) Pattern ID numbers have been shifted from 0-to-8 to 2-to-10.

Pattern ID=1 now indicates a layer-trigger event.

Pattern ID=0 now indicates no pattern matches found.

(10) Adr F4, clct_blanking=1 is the new default value

For TMB 2005 operation with up to 5 CFEBs.

For TMB 2013 operation with 7 DCFEBs.

NB: All external triggers (including scintillator and ALCT active FEB) create a dummy CLCT to force the TMB to read out raw hits.  ALCT triggers can produce an LCT match if the ALCT active FEB is followed by an ALCT valid pattern flag.

Reserved cable input signals connect to ALCT FPGA user input pins

Copied from CFEB design: http://www.physics.ohio-state.edu/~gujh/works/cmpdata.html